# GPIO

MICRO-ARCHITECTURE SPECIFICATION – V1.3

QUY NHON SEMICONDUCTORS – QNSC

# Table of Contents

# Table of Tables

Table 0-1. Revision history

Table 4-1. Upstream IP used

Table 5-1. GPIO wrapper interface

Table 6-1. GPIO register map

Table 8-1. Planned GPIO instances

Table 9-1. Functions this block does not provide

Table 10-1. GPIO wrapper tie-offs

Table 11-1. Requirements on other owners

Table A-1. Acronyms

Table B-1. First review

# Revision history

Table 0-1. Revision history

| Version | Date | Author | Reviewer | Description of change |
| --- | --- | --- | --- | --- |
| V1.3 | 2026-09-28 | Bui Hieu | – | Added the GPIO wrapper contract and verification requirements |

# 1. Overview

GPIO gives software control of eight bidirectional GPIO functions and detects input edges on those functions. QSOC uses one reusable m_qnsc_wrap_gpio wrapper around the PULP apb_gpio IP and plans three top-level instances, providing 24 GPIO functions.

The wrapper renames ports, fixes the IP configuration and ties off the unused DFT input. It does not implement the IO MUX, pad cells, APB routing, access protection or interrupt aggregation.

# 2. Features

- Eight GPIO functions per wrapper instance.

- Independent output value and active-high output enable for each GPIO.

- Two-stage input synchronisation in the peripheral clock domain.

- Per-pin falling-edge, rising-edge or both-edge interrupt selection.

- Sticky per-pin interrupt status with read-to-clear behaviour.

- Four pad-configuration bits per GPIO.

- Zero-wait-state 32-bit APB interface.

- One interrupt pulse output per wrapper instance.

# 3. Block diagram

The implemented signal flow is:

P_BUS -> APB register and decode logic -> PADOUT and PADDIR -> IO MUX -> pad
pad -> IO MUX -> two-stage synchroniser -> PADIN and edge detector
edge detector -> interrupt qualification -> o_int_gpio and INTSTATUS

The wrapper and all sequential logic in apb_gpio use i_clk_peri and asynchronous active-low i_rst_n_peri. The current repository does not yet contain the three top-level GPIO instantiations.

# 4. IP used

Table 4-1. Upstream IP used

| From | Module | Commit | Licence |
| --- | --- | --- | --- |
| pulp-platform/apb_gpio | apb_gpio | f82caeb7f7d89427f05e9af5ed31e0675efe0d83 | SolderPad Hardware License 0.51 |

# 5. Interface

Table 5-1. GPIO wrapper interface

| Signal | Dir | Width | Description |
| --- | --- | --- | --- |
| i_clk_peri | in | 1 | Peripheral-domain clock connected to IP port HCLK |
| i_rst_n_peri | in | 1 | Asynchronous active-low reset connected to HRESETn |
| i_bus_apb_paddr | in | 12 | Local APB offset; the IP decodes only bits [6:2] |
| i_bus_apb_psel | in | 1 | APB slave select |
| i_bus_apb_penable | in | 1 | APB access-phase indicator |
| i_bus_apb_pwrite | in | 1 | APB write indicator |
| i_bus_apb_pwdata | in | 32 | APB write data |
| o_bus_apb_prdata | out | 32 | APB read data from the IP |
| o_bus_apb_pready | out | 1 | APB completion; the selected IP revision drives constant 1 |
| o_bus_apb_pslverr | out | 1 | APB error; the selected IP revision drives constant 0 |
| i_pad_gpio | in | 8 | Input values from the IO MUX and pad input receivers |
| o_pad_gpio | out | 8 | Output values toward the IO MUX and pads |
| o_pad_gpio_oe | out | 8 | Active-high output enable; bit n controls GPIO n |
| o_pad_gpio_cfg | out | 32 | Flattened pad configuration; bits [4*n +: 4] configure GPIO n |
| o_int_gpio | out | 1 | OR of qualified edge events in this instance |

The upstream gpio_in_sync output is intentionally left open. Software reads the sampled input through PADIN; no QSOC block consumes a second copy.

# 6. Register map

Only GPIO fields 0 through 7 are implemented. Unimplemented upper read bits are zero. Writes to upper bits have no effect.

Table 6-1. GPIO register map

| Offset | Register | Field | Bits | Access | Reset | Description |
| --- | --- | --- | --- | --- | --- | --- |
| 0x00 | PADDIR | DIR | 7:0 | RW | 0x00 | 1 enables output drive; 0 releases the GPIO output |
| 0x04 | GPIOEN | EN | 7:0 | RW | 0x00 | Enables input sampling clock and per-pin interrupt qualification |
| 0x08 | PADIN | IN | 7:0 | RO | 0x00 | Previous synchronised input sample used by edge detection |
| 0x0C | PADOUT | OUT | 7:0 | RW | 0x00 | Output value |
| 0x10 | PADOUTSET | SET | 7:0 | WO | – | Writing 1 sets the corresponding PADOUT bit |
| 0x14 | PADOUTCLR | CLR | 7:0 | WO | – | Writing 1 clears the corresponding PADOUT bit |
| 0x18 | INTEN | EN | 7:0 | RW | 0x00 | Per-pin interrupt enable |
| 0x1C | INTTYPE | TYPE | 15:0 | RW | 0x0000 | Two bits per pin: 00 falling, 01 rising, 10 both edges, 11 disabled |
| 0x24 | INTSTATUS | STATUS | 7:0 | RC | 0x00 | Sticky event status; reading clears all implemented status bits |
| 0x28 | PADCFG | CFG | 31:0 | RW | 0x0000_0000 | Four configuration bits per GPIO |

# 7. Functional behaviour

## 7.1 APB access

An access completes when PSEL and PENABLE are asserted. The selected IP revision drives PREADY=1 and PSLVERR=0, so the wrapper adds no wait state or error response.

The IP decodes PADDR[6:2]. Address bits [11:7] and [1:0] do not participate in register selection, so offsets that differ only in those bits alias. This aliasing is accepted by the QSOC APB integration rule.

The IP has no PSTRB or PPROT port. The wrapper does not emulate either signal. Firmware shall use aligned 32-bit writes; a sub-word write is unsupported and may replace the complete addressed register value. Access protection, if required, is implemented in the interconnect.

## 7.2 Output path

PADOUT[n] drives o_pad_gpio[n]. PADDIR[n] drives active-high o_pad_gpio_oe[n]. PADOUTSET and PADOUTCLR modify PADOUT without a software read-modify-write operation.

The IO MUX selects whether the GPIO function reaches a physical pad. When the GPIO function is selected, the pad drives o_pad_gpio[n] only while o_pad_gpio_oe[n] is 1.

## 7.3 Input path

i_pad_gpio[n] passes through two synchroniser flip-flops. A third register keeps the previous synchronised sample for edge detection and supplies the PADIN read value.

Input sampling clocks are enabled in groups of four GPIOs. GPIOEN[3:0] collectively enables sampling for GPIO 0 through 3, and GPIOEN[7:4] collectively enables sampling for GPIO 4 through 7. Interrupt qualification still requires the corresponding per-pin GPIOEN[n] bit.

## 7.4 Interrupt path

For pin n, an interrupt event requires GPIOEN[n]=1, INTEN[n]=1 and an edge selected by INTTYPE[n]. Qualified events are OR-reduced to the one-cycle o_int_gpio pulse. INTSTATUS[n] records the pin that produced the event.

Reading INTSTATUS clears all eight implemented status bits. If a qualified event and an INTSTATUS read occur in the same cycle, event set has priority over read-clear.

## 7.5 Reset

Asserting i_rst_n_peri=0 clears direction, output value, input samples, GPIO enable, interrupt enable, interrupt type, interrupt status and pad configuration. The output value and output enable are therefore both low after reset.

# 8. Instances

Table 8-1. Planned GPIO instances

| Instance | APB port | Base address | GPIO functions | Interrupt output |
| --- | --- | --- | --- | --- |
| GPIO0 | APB_M3 | 0x8000_C000 | 8 | 1 |
| GPIO1 | APB_M4 | 0x8001_0000 | 8 | 1 |
| GPIO2 | APB_M5 | 0x8001_4000 | 8 | 1 |

The three instances use the same wrapper and fixed IP configuration. The top-level integration is planned but is not implemented in the current RTL.

# 9. What is not provided here and who provides it

Table 9-1. Functions this block does not provide

| Function | Where it lives |
| --- | --- |
| APB address routing and base-address subtraction | P_BUS in design/bus |
| GPIO function selection and signal sharing | design/iomux |
| Physical input, output and output-enable cells | IOPAD integration |
| Aggregation of three GPIO interrupt outputs | design/intmap |
| APB access protection | P_BUS or another integration-level policy block |

# 10. Tie-offs

Table 10-1. GPIO wrapper tie-offs

| Port | Tied to | Why |
| --- | --- | --- |
| dft_cg_enable_i | 1'b0 | QSOC v1 has no system DFT control source |
| gpio_in_sync | open | Software reads the sampled value through PADIN; no second consumer exists |

# 11. Requirements on others and open items

Table 11-1. Requirements on other owners

| Item | Owner | What it blocks |
| --- | --- | --- |
| Instantiate three wrappers on APB_M3, APB_M4 and APB_M5 | TOP owner | Chip-level GPIO access |
| Connect i_pad_gpio, o_pad_gpio, o_pad_gpio_oe and o_pad_gpio_cfg | IO MUX and IOPAD owners | Physical GPIO operation |
| Route the three o_int_gpio outputs to the agreed interrupt line | INTMAP and TOP owners | CPU GPIO interrupt delivery |
| Use aligned 32-bit GPIO writes | Firmware owner | Correct operation without PSTRB |
| Revisit dft_cg_enable_i when a DFT strategy is defined | DFT and TOP owners | Scan/test clock operation |

# 12. Verification

The self-checking GPIO regression shall verify:

- reset values and disabled outputs;

- APB full-word read and write of direction, output and pad configuration;

- PADOUTSET and PADOUTCLR behaviour;

- documented address aliasing;

- synchronised input visibility in PADIN;

- rising, falling and both-edge interrupt selection;

- INTSTATUS read-to-clear behaviour and set priority;

- eight GPIO functions, 32 pad-configuration bits, one interrupt output, and constant PREADY=1 and PSLVERR=0 responses.

# Appendix A. Acronyms

Table A-1. Acronyms

| Acronym | Description |
| --- | --- |
| APB | Advanced Peripheral Bus |
| DFT | Design for Test |
| GPIO | General-Purpose Input Output |
| IP | Intellectual Property block |
| IO MUX | Input Output Multiplexer |
| RC | Read to Clear |

# Appendix B. First review

Table B-1. First review

| Item | Reviewer | Response |
| --- | --- | --- |
| Add PSTRB and PPROT in the wrapper | Mentor feedback | Not added; document the upstream limitation and leave the ports unconnected at top level |
| Number of GPIO banks | Team decision | Three instances of eight GPIO functions, 24 functions total |
| Wrapper generation method | Mentor feedback | Emacs verilog-mode AUTO flow is used for source and generated RTL |