# DMA

MICRO-ARCHITECTURE SPECIFICATION – V3.0

QUY NHON SEMICONDUCTORS – QNSC

# Table of Contents

# Table of Tables

Table 3-1. Sub-blocks

Table 4-1. Upstream IP used

Table 5-1. DMA interface

Table 5-2. Configuration, fixed at the instances in m_qnsc_wrap_dma

Table 6-1. Memory map, regions behind APB_M13

Table 6-2. Register map, idma_reg32_2d

Table 6-3. CONF fields

Table 7-1. Examples

Table 9-1. Functions this block does not provide

Table 10-1. Tie-offs inside m_qnsc_wrap_dma

Table 11-1. Requirements on other owners

Table 11-2. HAS text this specification departs from

Table A-1. Acronyms

Table B-1. First review

# Table of Figures

Figure 3-1. DMA in QSOC

Figure 6-1. CONF and STATUS fields

Figure 7-1. One DMA job, firmware view

Figure 7-2. A 1D job of four words, memory to memory

# Revision history

The reasoning behind each change, the V1.0–V2.0 history, the V2.0 text and the research report it was based on are in QNSC_DMA_DECISIONS.md.

| Version | Date | Author | Reviewer | Description of change |
| --- | --- | --- | --- | --- |
| V3.0 | 2026-09-28 | Nghia VT (lead), for Ong Bao Vinh | – | Upstream iDMA used unmodified (frontend reg, 32-bit, 2D, APB), per the mentor: open-source 32-bit DMA, IP not redesigned. In-house frontend, descriptor engine and peripheral channels of V2.0 removed; APB_M13; idle interrupt |

# 1. Overview

The DMA copies blocks of data between any two addresses of QSOC without the CPU executing a load and a store per word. Firmware writes a source, a destination and a length, launches the job, and continues or sleeps until the DMA interrupt.

The fact that shapes it: it is pulp-platform/iDMA used as released. Frontend, midend and backend are upstream; this block is a wrapper that joins them and connects them to APB_M13 and AXI_S2.

Every transfer is started by software. There is no hardware request from peripherals: a peripheral is reached as an ordinary address through S_BUS, AXI2APB and P_BUS.

Block directory design/dma, wrapper m_qnsc_wrap_dma, owner Ong Bao Vinh.

# 2. Features

- Memory-to-memory, memory-to-peripheral and peripheral-to-memory copies at any byte address and length – 7.1.

- 2D jobs: REPS repetitions with independent source and destination strides; a stride of 0 reaches a FIFO data register – 7.3.

- AXI4 bursts on AXI_S2, 32-bit data – 7.2.

- Configuration on APB_M13, upstream register file – 6.

- One level interrupt while the DMA is idle – 7.4.

Use in QSOC: block copies between memories and buffers, and SPI host and SPI Device data at high rates. UART and I2C are served by the CPU: at 113 636 baud a 16-byte FIFO needs one interrupt every 1.4 ms.

# 3. Block diagram

Figure 3-1. DMA in QSOC

Table 3-1. Sub-blocks

| Block | Module | Source | Function |
| --- | --- | --- | --- |
| Frontend | idma_reg32_2d | upstream, generated (7.6) | APB register file; builds a 2D job; launches it on a read of NEXT_ID |
| Transfer IDs | idma_transfer_id_gen | upstream | Issues the job ID returned by NEXT_ID; counts completed jobs into DONE_ID |
| Midend | idma_nd_midend | upstream | Splits a 2D job into REPS 1D transfers |
| Backend | idma_backend_rw_axi | upstream, generated (7.6) | Executes a 1D transfer as AXI4 read and write bursts |
| Port join | in m_qnsc_wrap_dma | wires | Read port AR/R and write port AW/W/B onto one AXI port, AXI_S2 |
| Idle | in m_qnsc_wrap_dma | one NOR | o_int_dma = no unit busy |

The block is in the peri clock cluster, SCRC CLK_EN[10].

# 4. IP used

Table 4-1. Upstream IP used

| From | Module | Commit | Licence |
| --- | --- | --- | --- |
| pulp-platform/iDMA | idma_reg.sv.tpl, idma_reg.rdl (frontend), idma_nd_midend, idma_transfer_id_gen, backend templates, idma_pkg | 2e0b0fe5 | SHL-0.51 |

Facts this specification relies on, read at that commit:

- Backend protocols: AXI4, AXI4-Lite, AXI4-Stream, OBI, TileLink, Init (src/db). No APB, which QSOC does not need: the configuration port is the frontend’s.

- The frontend template has an APB4 configuration port (--cpuif apb4-flat, the idma.mk default) and a 32-bit variant. It has no interrupt and no hardware request input.

- A read of NEXT_ID launches the job and returns the next transfer ID; one launch is held until the midend accepts it (idma_reg.sv.tpl, launch_pending_q).

- The frontend drives burst = INCR for source and destination (idma_reg.sv.tpl).

- idma_rt_midend launches on a time counter, not on a peripheral event; it is not used.

# 5. Interface

Table 5-1. DMA interface

| Signal | Dir | Width | Description |
| --- | --- | --- | --- |
| i_clk_peri, i_rst_n_peri | in | 1 | SCRC o_clk_dma, o_rst_n_dma |
| i_bus_apb_psel, i_bus_apb_penable, i_bus_apb_pwrite | in | 1 | P_BUS APB_M13 |
| i_bus_apb_paddr | in | 12 | Offset in the window (contract meta.apb_paddr_width) |
| i_bus_apb_pwdata, i_bus_apb_pstrb, i_bus_apb_pprot | in | 32, 4, 3 | APB4 |
| o_bus_apb_prdata, o_bus_apb_pready, o_bus_apb_pslverr | out | 32, 1, 1 | APB4 response |
| o_bus_axi_ar_*, i_bus_axi_r_* | – | AXI4 | Read channels to S_BUS AXI_S2, from the backend read port |
| o_bus_axi_aw_*, o_bus_axi_w_*, i_bus_axi_b_* | – | AXI4 | Write channels to AXI_S2, from the backend write port |
| o_int_dma | out | 1 | 1 while the DMA is idle – 7.4. To INTMAP i_int_dma, line 0 |

Table 5-2. Configuration, fixed at the instances in m_qnsc_wrap_dma

| Parameter | Value | Why |
| --- | --- | --- |
| Frontend | reg32_2d, apb4-flat, 1 register port, 1 stream | 32-bit system, 2D for the stride-0 case; one job stream |
| DataWidth, AddrWidth | 32, 32 | QSOC bus widths |
| AxiIdWidth | 5 | AXI_S2 ID width assumed by S_BUS (QNSC_BUS_MAS 11) |
| TFLenWidth | 32 | LENGTH is a 32-bit register |
| NumAxInFlight, BufferDepth | 2, 2 | Backend template defaults |
| ErrorCap | NO_ERROR_HANDLING | No error-handler port – 7.5 |
| HardwareLegalizer, RejectZeroTransfers | 1, 1 | Bursts split at 4 KiB and to legal lengths; a zero-length job is dropped |
| EnableCompute | 0 | No on-the-fly compute; COMPUTE_CFG has no effect |

# 6. Register map

Table 6-1. Memory map, regions behind APB_M13

| Base | Size | Region | Port | Kind | Note |
| --- | --- | --- | --- | --- | --- |
| 0x80034000 | 16 KiB | dma_cfg | APB_M13 | peripheral | Register interface of the DMA |

The frontend decodes offset bits 7:0, so the map repeats every 256 bytes in the window. All registers are 32-bit; an offset with no register reads 0.

Table 6-2. Register map, idma_reg32_2d

| Offset | Register | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 0x00 | CONF | RW | 0 | Job options, Figure 6-1 |
| 0x04 | STATUS | RO | 0 | busy: bit 8 midend, bits 7:0 the eight backend units of idma_busy_t; bit 9 reads 0 |
| 0x08–0x40 | STATUS of streams 1–15 | RO | 0 | Not present; read 0 |
| 0x44 | NEXT_ID | RO, read has effect | 0 | Read launches the job and returns its ID |
| 0x48–0x80 | NEXT_ID of streams 1–15 | RO | 0 | Not present; read 0, launch nothing |
| 0x84 | DONE_ID | RO | 0 | ID of the last completed job |
| 0x88–0xC0 | DONE_ID of streams 1–15 | RO | 0 | Not present |
| 0xD0 | DST_ADDR | RW | 0 | Destination byte address |
| 0xD4 | SRC_ADDR | RW | 0 | Source byte address |
| 0xD8 | LENGTH | RW | 0 | Bytes per 1D transfer |
| 0xDC | DST_STRIDE | RW | 0 | Destination address step between repetitions |
| 0xE0 | SRC_STRIDE | RW | 0 | Source address step between repetitions |
| 0xE4 | REPS | RW | 0 | Number of repetitions (2D) |
| 0xE8 | COMPUTE_CFG | RW | 0 | No effect (EnableCompute = 0) |

Figure 6-1. CONF and STATUS fields

Table 6-3. CONF fields

| Bits | Field | QSOC value | Meaning |
| --- | --- | --- | --- |
| 0 | decouple_aw | 0 | Write address may be issued before read data; 0 = coupled |
| 1 | decouple_rw | 0 | Read and write decoupled; 0 = coupled |
| 2, 3 | src_reduce_len, dst_reduce_len | 0 | 1 limits the burst to 2^max_llen beats |
| 6:4, 9:7 | src_max_llen, dst_max_llen | 0 | Log2 of the maximum burst length when reduced |
| 11:10 | enable_nd | 0 or 1 | 1 = 2D job (REPS, strides used) |
| 14:12, 17:15 | src_protocol, dst_protocol | 0 | 0 = AXI, the only backend protocol |
| 31:18 | – | 0 | Reserved |

# 7. Functional behaviour

## 7.1 Job

Figure 7-1. One DMA job, firmware view

- A 1D job copies LENGTH bytes from SRC_ADDR to DST_ADDR, both incrementing. Addresses and length need no alignment; the backend handles the byte offsets.

- A 2D job (enable_nd = 1) copies REPS blocks of LENGTH bytes; block k starts at SRC_ADDR + k x SRC_STRIDE and DST_ADDR + k x DST_STRIDE.

- Reading NEXT_ID samples the registers and launches the job; the registers may be rewritten at once.

- The frontend holds one launch until the midend accepts it; a second read of NEXT_ID before that is lost. Firmware runs one job at a time: it launches the next job after DONE_ID has reached the previous ID.

- DONE_ID is the ID of the last job completed. A job with ID i is complete when DONE_ID >= i, modulo 2^32.

## 7.2 Bus traffic

Figure 7-2. A 1D job of four words, memory to memory

- Reads and writes use INCR bursts on AXI_S2, split at 4 KiB boundaries.

- With the ISRAM/DSRAM controller, a read burst returns one beat every 2 cycles and a write burst takes one beat per cycle (QNSC_RAM_MAS 7.1): about 2 cycles per word, against about 10 for a CPU copy loop (estimate, not simulated).

- A peripheral address goes through AXI_M3, AXI2APB and P_BUS: one APB access per beat.

- The DMA and Ibex share S_BUS; the crossbar arbitrates between them.

## 7.3 Peripheral FIFO: stride 0

A FIFO data register is reached with a 2D job whose peripheral-side stride is 0:

Table 7-1. Examples

| Job | SRC_ADDR | DST_ADDR | LENGTH | REPS | SRC_STRIDE | DST_STRIDE |
| --- | --- | --- | --- | --- | --- | --- |
| 16 bytes to UART0 THR | buffer | 0x8002_0000 | 1 | 16 | 1 | 0 |
| 16 bytes from UART0 RBR | 0x8002_0000 | buffer | 1 | 16 | 0 | 1 |
| 4 KiB memory copy (1D) | source | destination | 4096 | – | – | – |

Firmware starts a FIFO job only when the FIFO has room or data for all REPS entries, for example from the peripheral’s FIFO interrupt. Nothing paces the DMA by the peripheral.

## 7.4 Interrupt

- o_int_dma = 1 while neither the midend nor any backend unit is busy (STATUS = 0). It is a level, 1 out of reset.

- Firmware enables mie bit 16 after launching, and the handler compares DONE_ID with the launched ID: equal or later, the job is done and the handler clears mie[16]; earlier, the line was sampled before the job became busy and the handler returns.

- Line 0 is the highest-priority fast line (QNSC_Interrupt_Map_MAS 7.2).

## 7.5 Errors

- LENGTH = 0 is not a job: firmware does not launch one.

- With ErrorCap = NO_ERROR_HANDLING, an AXI error response on AXI_S2 (decode error, SLVERR from the ROM or a blocked peripheral) is not reported: the job completes and DONE_ID advances.

## 7.6 Generated files

The frontend and the backend are generated once, at the pinned commit, and committed with the command that made them in util/gen/idma/:

- idma_reg32_2d_reg_top.sv, idma_reg32_2d_reg_pkg.sv: PeakRDL regblock on idma_reg.rdl, SysAddrWidth=32 NumDims=2 Log2NumDims=1, --cpuif apb4-flat.

- idma_reg32_2d_top.sv: gen_idma.py on idma_reg.sv.tpl.

- idma_backend_rw_axi.sv, idma_legalizer_rw_axi.sv, idma_transport_layer_rw_axi.sv: gen_idma.py on the backend templates, --ids rw_axi.

None is edited by hand. design/dma/rtl/ holds the wrapper only.

## 7.7 Clock and reset

- Clock o_clk_dma, gateable by CLK_EN[10]; reset o_rst_n_dma, released with the other domains (QNSC_SCRC_MAS 7.5).

- The SCRC APB guard covers APB_M13 only. Before gating the DMA, firmware waits until STATUS = 0, so no burst is cut on S_BUS (QNSC_SCRC_MAS 7.10).

# 8. Instances

One m_qnsc_wrap_dma in design/top.

# 9. What is not provided here, and who provides it

Table 9-1. Functions this block does not provide

| Function | Where it lives |
| --- | --- |
| Hardware request from a peripheral | not provided; firmware starts every job from the peripheral’s interrupt (7.3) |
| Descriptor chains, circular buffers | not provided; firmware launches the next job |
| Reporting an AXI error | not provided (7.5) |
| Memory fill without a source | not provided; copy from a zero buffer |
| Routing AXI_S2 to memories and AXI_M3 | S_BUS |

# 10. Tie-offs

Table 10-1. Tie-offs inside m_qnsc_wrap_dma

| Port | Tied to | Why |
| --- | --- | --- |
| Frontend midend_busy_i | midend busy_o | Reported in STATUS[9] |
| Backend error-handler request | none (NO_ERROR_HANDLING has no port) | 7.5 |
| AXI user on every channel | 0 | Not used on S_BUS |
| AXI ar_prot, aw_prot | as driven by the backend | Not checked by any QSOC slave |

# 11. Requirements on others, and open items

Table 11-1. Requirements on other owners

| Item | Owner | What it blocks |
| --- | --- | --- |
| AXI_S2 ID width 5; AXI_S2 reaches AXI_M0–AXI_M3 | bus owner | 7.2 |
| UART wrapper drops o_dma_tx_req, o_dma_rx_req and the patch 0001-add-dma-request-lines | UART owner | a vendor patch that nothing uses |
| Contract: close tbd “DMA peripheral channel for each requester”; interrupt line 0 note: idle level | lead | contract matching this block |

Table 11-2. HAS text this specification departs from

| QSOC_HAS_Report_EN_v4_final Table 10-1/10-2 says | This specification | Why |
| --- | --- | --- |
| Three modes: register job, descriptor chain, eight peripheral channels | upstream frontend reg only | Mentor: open-source DMA, IP not redesigned; QSOC peripherals are slow enough for CPU-paced jobs |
| Configuration port APB_M14 at 0x8003_8000 | APB_M13 at 0x8003_4000 | Contract since GPIO3 was dropped |
| DMA_ISR write-1-to-clear flags, IRQ_MODE | idle level from STATUS | The upstream frontend has no interrupt; an idle level needs no register |
| axi_mux 3:1 onto AXI_S2 | wires | Without a descriptor fetch port only the read and write ports remain, on disjoint channels |

Open items, DMA owner:

- Generate the files of 7.6 and commit them with the commands.

- Run the upstream rw_axi backend jobs and a wrapper test in simulation.

Accepted limits:

- No AXI error is visible to software (7.5).

- A FIFO job is paced by firmware, not by the peripheral (7.3).

- One job at a time (7.1).

# 12. Verification

- DMA_001 Register reset values, access and the 256-byte alias of section 6.

- DMA_002 A 1D job copies LENGTH bytes for unaligned addresses and lengths 1, 3, 4, 5, 4096 and across a 4 KiB boundary.

- DMA_003 A 2D job with DST_STRIDE = 0 writes one address REPS times, in order.

- DMA_004 Two jobs, each launched after DONE_ID reached the previous ID, complete in order and DONE_ID reaches each ID.

- DMA_005 A read of NEXT_ID while a launch is still held changes nothing (documents the limit of 7.1).

- DMA_006 o_int_dma is 1 exactly while STATUS = 0.

- DMA_007 A job to UART0 at 0x8002_0000 through S_BUS and P_BUS (SoC test).

- DMA_008 Ibex and the DMA both reach ISRAM during a job; both complete (SoC test).

- DMA_009 The generated files match a regeneration from the pinned commit (CI).

# Appendix A. Acronyms

Table A-1. Acronyms

| Acronym | Description |
| --- | --- |
| DMA | Direct Memory Access |
| iDMA | pulp-platform’s modular DMA engine |
| INCR | AXI incrementing burst |
| ND | N-dimensional (here 2D) transfer |

# Appendix B. First review

Table B-1. First review

| Item | Reviewer | Response |
| --- | --- | --- |
| Use an open-source 32-bit DMA; do not redesign the IP | Quan (mentor) | Upstream iDMA unmodified, 3, 4 |
| iDMA has no hardware request from peripherals | Vinh (research) | Correct; QSOC does not need it, 1, 9 |
| Is an APB backend needed to reach the peripherals? | Vinh | No: peripherals are addresses behind S_BUS, 1 |
| What is the DMA for if the CPU starts every job? | Vinh | Block copies and fast SPI without a CPU load/store per word; one interrupt per job, 2, 7.2 |
| APB_M14 against contract APB_M13 | lead, V3.0 | 6 |