Table of Contents

# QNSC_Interrupt_Map_MAS

Block intmap · Owner Nghia Van Trong · Version 2.0 · 2026-09-23

Twenty-seven interrupt sources reach the CPU as twelve wires. INTMAP is the combinational OR tree that does the reduction.

Why the specification says what it says — the two designs that were dropped, the evidence read out of Ibex, the questions that closed — is in QNSC_Interrupt_Map_DECISIONS.md. This file is the contract.

## 1. Scope

INTMAP groups the interrupt sources of fourteen blocks onto the CPU’s fast interrupt lines, one line per peripheral, and passes the watchdog bark straight through to the non-maskable input.

It does not: hold state, decode an address, appear on any bus, prioritise (the core does that), or latch a pulse. It has no clock and no reset.

## 2. Block diagram

INTMAP — 27 sources, one OR per peripheral, 12 wires to the CPU

One combinational layer. No sub-blocks, no clock domain, no reset domain.

## 3. IP used

| From | Module | Commit |
| --- | --- | --- |
| — | — | — |

Designed in house. The block instantiates nothing; it is assign statements.

## 4. Interface

Every port. Naming follows QNSC_RTL_Design_Naming_Rule V1.0 section 3.6, i_int_<source> / o_int_<source>.

| Signal | Dir | Width | Description |
| --- | --- | --- | --- |
| i_int_dma | in | 1 | DMA completion. Level, held by DMA_ISR |
| i_int_spi_dev | in | 8 | spi_device intr_*_o, in declaration order |
| i_int_spi_host | in | 2 | spi_host error, event |
| i_int_i2c | in | 1 | I²C |
| i_int_uart_0 | in | 1 | UART0. Driven by the IP’s INT port |
| i_int_uart_1 | in | 1 | UART1 |
| i_int_timer_1 | in | 2 | TIMER1 irq_lo_o, irq_hi_o |
| i_int_pwm | in | 4 | PWM events_o |
| i_int_wdt_wkup | in | 1 | aon_timer wake-up expiry |
| i_int_gpio | in | 4 | one per GPIO instance |
| i_int_timer_0 | in | 1 | TIMER0 irq_lo_o only — 64-bit mode |
| i_int_wdt_bark | in | 1 | aon_timer nmi_wdog_timer_bark_o |
| o_int_fast | out | 11 | to Ibex irq_fast_i[10:0] |
| o_int_nm | out | 1 | to Ibex irq_nm_i |

27 inputs, 12 outputs, and no other ports. No i_clk_*, no i_rst_n_*.

## 5. Register map

None. The block has no address and is not a bus slave, so there is nothing for software to read or write.

Enabling and masking is done in the core: mie bits 16–26 for the fast lines. Acknowledging is done at each peripheral’s own status register.

## 6. Functional behaviour

### 6.1 One OR gate per peripheral

assign o_int_fast[0]  =  i_int_dma;
assign o_int_fast[1]  = |i_int_spi_dev;
assign o_int_fast[2]  = |i_int_spi_host;
assign o_int_fast[3]  =  i_int_i2c;
assign o_int_fast[4]  =  i_int_uart_0;
assign o_int_fast[5]  =  i_int_uart_1;
assign o_int_fast[6]  = |i_int_timer_1;
assign o_int_fast[7]  = |i_int_pwm;
assign o_int_fast[8]  =  i_int_wdt_wkup;
assign o_int_fast[9]  = |i_int_gpio;
assign o_int_fast[10] =  i_int_timer_0;
assign o_int_nm       =  i_int_wdt_bark;

A peripheral with one source is wired straight through; one with several is OR-reduced. Zero flip-flops, so the delay from a source asserting to the core seeing it is combinational.

### 6.2 Line assignment

The line index is the priority: Ibex resolves the lowest index first. This table therefore fixes the default priority order as well as the wiring, and changing it requires re-synthesis.

| Line | mcause | Vector | Peripheral | Sources | Shape |
| --- | --- | --- | --- | --- | --- |
| 0 | 16 | mtvec + 0x40 | DMA | 1 | level |
| 1 | 17 | mtvec + 0x44 | SPI device | 8 | level |
| 2 | 18 | mtvec + 0x48 | SPI host | 2 | level |
| 3 | 19 | mtvec + 0x4C | I²C | 1 | level |
| 4 | 20 | mtvec + 0x50 | UART0 | 1 | level |
| 5 | 21 | mtvec + 0x54 | UART1 | 1 | level |
| 6 | 22 | mtvec + 0x58 | TIMER1 | 2 | pulse |
| 7 | 23 | mtvec + 0x5C | PWM | 4 | pulse |
| 8 | 24 | mtvec + 0x60 | WDT wake-up | 1 | level |
| 9 | 25 | mtvec + 0x64 | GPIO0–3 | 4 | pulse |
| 10 | 26 | mtvec + 0x68 | TIMER0 | 1 | pulse |
| 11–14 | 27–30 | — | spare, tied to 0 | 0 | — |
| — | 31 | mtvec + 0x7C | WDT bark, on irq_nm_i | 1 | level |

26 sources on 11 lines, plus the NMI: 27. Ibex provides 15 fast lines, so four are spare.

Order follows one rule: data loss first, human time last. A missed SPI or UART event loses a byte that cannot be recovered; a missed timer tick arrives again next period. DMA takes line 0 because the rest of the system waits on a transfer completing.

### 6.3 The core identifies the source, not a register

Fast line n raises mcause 16 + n, and Ibex is permanently in vectored mode, so the trap address is mtvec + 4 × mcause. The core therefore enters the peripheral’s own handler directly. No claim register is read and no bus transaction occurs on the interrupt path.

mcause 16 and above is platform-use space in the RISC-V privileged specification, so this numbering is a local convention the specification permits.

### 6.4 The NMI bypasses this block

i_int_wdt_bark goes to o_int_nm unmodified. The bark must reach the core even if firmware has hung with interrupts disabled, and irq_nm_i is outside mstatus.MIE and mie.

Ibex ignores the NMI in Debug Mode. A bark raised while SYSDBG has the core halted therefore traps on resume, not when it occurs — and only because aon_timer holds the bark as a level.

### 6.5 A pulse can be missed, and what that costs

mip in Ibex is combinational: assign mip.irq_fast = irq_fast_i. The core latches nothing, and neither does this block. A one-cycle pulse arriving while mstatus.MIE is clear is therefore not seen.

That costs an interrupt but not information, because every source either holds its line or keeps a record:

| Source | Shape | Recovery |
| --- | --- | --- |
| DMA | level | held by DMA_ISR, write-1-to-clear |
| SPI ×2, I²C, UART ×2, WDT wake-up | level | held until the handler clears it |
| GPIO | pulse | which pin fired is in the instance’s own INTSTATUS |
| TIMER0, TIMER1, PWM | pulse | the event repeats next period |

No source destroys information. This is the condition the design depends on.

## 7. Instances

One. It is instantiated in design/top and has no parameters.

## 8. What is not provided here, and who provides it

| Function | Where it lives |
| --- | --- |
| Pending latch | the peripheral’s status register, or nowhere — 6.5 |
| Which event fired | the peripheral’s status register |
| Which peripheral fired | mcause, via the vectored trap address |
| Priority resolution | Ibex, by fast-line index |
| Per-source enable | mie bits 16–26, in the core |
| Acknowledge | writing the peripheral’s status register |

## 9. Tie-offs

| Port | Tied to | Why |
| --- | --- | --- |
| irq_fast_i[14:11] | 4'b0 | QSOC drives 11 of the 15 lines |
| irq_external_i | 0 | nothing aggregates onto it; there is no external controller |
| irq_timer_i | 0 | no CLINT, so mip.MTIP is never set. TIMER0 is an ordinary fast line |
| irq_software_i | 0 | permitted on a single-hart system |

Consequence for firmware: an RTOS ported to QSOC must supply its own timer driver rather than the standard mtime/mtimecmp one.

## 10. Requirements on others, and open items

| Item | Owner | What it blocks |
| --- | --- | --- |
| APB-to-TL-UL wrapper for aon_timer | WDT owner | two of the 27 sources, one being the NMI. Its enable is in WDOG_CTRL offset 0x1C, so without a bus path the watchdog cannot be enabled and never barks |
| Watchdog stoppable while debug_mode is asserted, or firmware told to disable it | WDT owner | a long debug session otherwise ends in a reset with nothing recording why |
| Vector table at mtvec + 0x40 … +0x68, plus +0x7C; mie bits 16–26 | firmware owner | — |
| Handlers installed before mstatus.MIE is set | firmware owner | the block is transparent out of reset, so every source is live from the first cycle |

Accepted limits, stated rather than hidden:

- A pulse arriving while mstatus.MIE is clear is lost. Recoverable for every source — 6.5.

- Priority is fixed at elaboration. Changing it is a re-synthesis.

- mcause 16–30 is platform-use space, so the numbering is a local convention rather than a portable one.

## Verification

Ten checks, none requiring a bus model:

- Each single-source input raises exactly its own line.

- Each OR group raises its line for every member, individually.

- No input raises a line other than its own.

- Spare lines [14:11] are always 0.

- i_int_wdt_bark reaches o_int_nm and no fast line.

- Simultaneous inputs on one group raise the line once.

- Simultaneous inputs on different groups raise both lines.

- Output follows input combinationally, with no cycle of delay.

- mcause/vector mapping matches section 6.2 against qnsc_pkg.

- Lint check: the module contains no i_clk_, no i_rst_n_, and no always_ff. If any appears, the design has drifted back into being a controller.