# SYSDBG

MICRO-ARCHITECTURE SPECIFICATION – V4.0

QUY NHON SEMICONDUCTORS – QNSC

# Table of Contents

# Table of Tables

Table 4-1. Upstream IP used

Table 5-1. SYSDBG interface

Table 5-2. SYSDBG parameters

Table 6-1. JTAG instructions and data registers

Table 7-1. The DBG_EN pin and the frame header’s DEBUG_PROG (QNSC_BOOT_SPEC Figure 4-2)

Table 7-2. What each reset does

Table 7-3. AXI manager behaviour

Table 7-4. Crossing signals and constraints

Table 7-5. Debug window, the first 4 KiB of ISRAM

Table 9-1. Functions this block does not provide

Table 10-1. Tie-offs

Table 11-1. Requirements on other owners

Table A-1. Acronyms

Table B-1. First review

# Table of Figures

Figure 3-1. SYSDBG block diagram

Figure 7-1. Read handshake

# Revision history

The reasoning behind each change, and every version before V3.0, is in QNSC_SYSDBG_DECISIONS.md.

| Version | Date | Author | Reviewer | Description of change |
| --- | --- | --- | --- | --- |
| V3.0 | 2026-09-23 | Nghia VT | – | Rewritten after the teacher’s reference design: JTAG data registers, native AXI4 manager, 4-phase handshake. Adds the DBG_EN pin and CPU hold |
| V3.1 | 2026-09-25 | Nghia VT | – | Port names to QNSC_RTL_Design_Naming_Rule V1.0: o_dbg_req, o_dbg_cpu_hold, AXI named per channel (o_bus_axi_ar_valid, …). No change in behaviour |
| V4.0 | 2026-10-15 | Nghia VT | – | Follows the boot flow of QNSC_BOOT_SPEC V3.4 (mentor, 12/10 and 14/10). The CPU is never held, so CPUHOLD, o_dbg_cpu_hold and the boot address mux are gone. The boot ROM loads the debug program, so SYSDBG no longer writes the window at boot: 7.2 is the four cases of the boot flow. o_dbg_en stays, for the JTAG pads only. 7.7: a halt needs the debug program in the window; a request held across a watchdog or software reset halts the core before its first instruction; breakpoints. 7.8: the layout is the contract’s debug_window, the mailbox moves to 0xF00 (QNSC_BOOT_SPEC Table 9-1). 9: DbgTriggerEn = 1, two hardware breakpoints (this document said 0, against its own _DECISIONS 5.2) |

# 1. Overview

SYSDBG is the QSOC debugger. A JTAG TAP takes commands from the host. An AXI4 manager on AXI_S0 reads and writes memory. A 4-phase handshake links the two. It also drives the Ibex debug_req input.

The CPU is never held. After every reset SCRC releases it and it runs the boot ROM, which downloads the application over UART0 and, when the frame carries one, a 4 KiB debug program into the debug window (QNSC_BOOT_SPEC). One external pin, DBG_EN, makes the ROM wait for the host after the download. The host ends the wait by writing DBG_BOOT.dbg_en_mask of SYSCSR through this block’s bus master, and can halt the CPU into the debug program at any time, while the ROM waits or while the application runs.

SYSDBG does not reset the chip, hold the CPU, gate a clock, or hold a debug ROM. It has no memory-mapped registers: everything is reached through JTAG. It captures DBG_EN once, to keep the JTAG pads on JTAG (7.1).

Block directory design/sysdbg, module m_qnsc_sysdbg, owner Nghia Van Trong.

# 2. Features

- JTAG TAP to IEEE 1149.1, 4-bit IR, six data registers.

- AXI4 manager: one-beat, 32-bit word reads and writes to any address, including while the CPU runs. One transaction at a time.

- 4-phase request/acknowledge handshake between the TCK and AXI clock domains.

- Halt through debug_req, set and cleared by the host.

- Debug while the boot ROM waits and while the application runs: the CPU is never held, and a halt enters the debug program that the ROM loaded into the window.

- Two hardware breakpoints (DbgTriggerEn = 1), also in the read-only ROM.

# 3. Block diagram

Figure 3-1. SYSDBG block diagram

Three domains, as labelled in the figure: TCK (i_jtag_tck, reset i_jtag_trst_n AND i_rst_n_por), AXI (i_clk_cpu, reset i_rst_n_sysbus) and system (i_clk_cpu, reset i_rst_n_por). i_clk_cpu is the cpu cluster clock, the same clock as S_BUS, and is never gated.

# 4. IP used

Table 4-1. Upstream IP used

| From | Module | Commit | Licence |
| --- | --- | --- | --- |
| – | – | – | – |

In house. Nothing is instantiated.

# 5. Interface

Table 5-1. SYSDBG interface

| Signal | Dir | Width | Description |
| --- | --- | --- | --- |
| i_clk_cpu | in | 1 | System and AXI clock, 20 MHz |
| i_rst_n_por | in | 1 | Power-on reset only |
| i_rst_n_sysbus | in | 1 | S_BUS domain reset. Resets the AXI domain |
| i_jtag_tck | in | 1 | JTAG clock. Asynchronous to i_clk_cpu, may stop at any time |
| i_jtag_tms, i_jtag_tdi | in | 1 | Sampled on rising TCK |
| i_jtag_trst_n | in | 1 | JTAG reset |
| o_jtag_tdo | out | 1 | Changes on falling TCK |
| o_jtag_tdo_oe | out | 1 | 1 in Shift-IR and Shift-DR only |
| i_dbg_en | in | 1 | From the DBG_EN pad. Asynchronous, captured once – 7.1 |
| o_dbg_en | out | 1 | Captured DBG_EN. Forces the JTAG pins to JTAG, in the IO MUX. Not a boot address and not a CPU hold |
| o_dbg_req | out | 1 | To Ibex debug_req_i. Level, from a flip-flop |
| o_bus_axi_ar_valid, i_bus_axi_ar_ready | out, in | 1 | AR handshake |
| o_bus_axi_ar_addr | out | 32 | Read address, [1:0] = 0 |
| i_bus_axi_r_valid, o_bus_axi_r_ready | in, out | 1 | R handshake |
| i_bus_axi_r_data | in | 32 | Read data |
| i_bus_axi_r_resp | in | 2 | Read response |
| o_bus_axi_aw_valid, i_bus_axi_aw_ready | out, in | 1 | AW handshake |
| o_bus_axi_aw_addr | out | 32 | Write address, [1:0] = 0 |
| o_bus_axi_w_valid, i_bus_axi_w_ready | out, in | 1 | W handshake |
| o_bus_axi_w_data | out | 32 | Write data |
| i_bus_axi_b_valid, o_bus_axi_b_ready | in, out | 1 | B handshake |
| i_bus_axi_b_resp | in | 2 | Write response |
| Other AXI4 signals | – | – | Constant outputs or unused inputs – section 10 |

Table 5-2. SYSDBG parameters

| Parameter | Default | Meaning |
| --- | --- | --- |
| IdcodeValue | 0x0515_3001 | Part 0x5153 (“QS”), manufacturer 0x000, bit 0 = 1 |
| AxiIdWidth | 5 | S_BUS slave-port ID width |
| SyncStages | 2 | Flip-flops per synchroniser |

# 6. Register map

No memory-mapped registers. The registers below are JTAG data registers, selected by the IR and shifted LSB first.

Table 6-1. JTAG instructions and data registers

| IR | Name | Bits | Capture-DR loads | Update-DR does | Reset |
| --- | --- | --- | --- | --- | --- |
| 0100 | ADDR | 33 | addr_reg | Load addr_reg. If bit 32 = 0 and not busy: start a read | 0 |
| 0101 | DATA | 32 | rdata_hold, the last read data | Load wdata_reg. If addr_reg[32] = 1 and not busy: start a write | 0 |
| 0110 | STATUS | 3 | {busy, resp[1:0]} | Nothing | 0 |
| 0111 | CPUDBG | 1 | dbgreq | Load dbgreq | 0 |
| 1110 | IDCODE | 32 | IdcodeValue | Nothing | – |
| 1111 | BYPASS | 1 | 0 | Nothing | – |
| others | as BYPASS | 1 | 0 | Nothing | – |

- addr_reg[32] is the direction: 0 read, 1 write. addr_reg[1:0] are ignored.

- resp is the AXI response of the last completed transaction: 00 OKAY, 10 SLVERR, 11 DECERR.

- IR is 4 bits. Capture-IR loads 0001. Test-Logic-Reset loads IDCODE.

- 1000 was CPUHOLD until V3.1. It now acts as BYPASS, like every unused code.

# 7. Functional behaviour

## 7.1 DBG_EN and the JTAG pads

The system domain synchronises i_dbg_en and captures it once, SyncStages + 1 cycles after i_rst_n_por releases. The captured value is o_dbg_en. It holds until the next power-on reset, so a later change of the pad does nothing.

o_dbg_en has one use: while it is 1 the IO MUX keeps the five JTAG pads on JTAG, so firmware that remaps pins cannot cut the debugger off in a debug session (_DECISIONS 4.3.1). It does not decide how the CPU starts. The CPU is never held and has one boot address, the ROM (QNSC_BOOT_SPEC 4).

What DBG_EN means to the boot ROM is not decided here. SYSCSR passes the pin through two flip-flops into DBG_BOOT.dbg_en, which the ROM polls (QNSC_SYSCSR_MAS 7.5). This block only ends the wait, by writing DBG_BOOT.dbg_en_mask = 1 (7.2). The pad therefore goes to two places: SYSCSR i_dbg_en and this block’s i_dbg_en.

## 7.2 Debug while the ROM waits, and while the application runs

Table 7-1. The DBG_EN pin and the frame header’s DEBUG_PROG (QNSC_BOOT_SPEC Figure 4-2)

| DBG_EN pin | DEBUG_PROG | After the download | For SYSDBG |
| --- | --- | --- | --- |
| 0 | not 0x55 | The ROM jumps to 0x2000_1080 | No debug program. The host must not raise CPUDBG |
| 1 | 0x55 | The ROM polls DBG_BOOT.dbg_en until the host writes dbg_en_mask = 1 | Debug boot. The host halts the CPU into the debug program at any time during the wait |
| 0 | 0x55 | The ROM jumps to 0x2000_1080 | Attach later. The host raises CPUDBG while the application runs |
| 1 | not 0x55 | The ROM polls as above and sends no token | Operator mistake. No debug program, so the host must not raise CPUDBG; it only writes dbg_en_mask = 1 so that the application runs |

Debug boot. The host takes these steps, in order:

- Power on with DBG_EN = 1. SCRC releases the chip and the CPU runs the ROM. The JTAG pads are on JTAG (7.1); the host may use SYSDBG from this moment.

- The PC sends the frame with DEBUG_PROG = 0x55. The ROM writes the debug program to 0x2000_0000 and answers ACKD, then writes the application and answers ACKP. The host must not raise CPUDBG before ACKD: until then the window holds nothing to enter (7.7).

- The ROM polls DBG_BOOT. The host sets CPUDBG = 1. Ibex saves the PC of the interrupted polling instruction in dpc and enters the dispatch loop at DmHaltAddr.

- The host reads and writes memory and CPU registers with sequences (7.4, 7.8) and sets breakpoints (7.7).

- The host writes CPUDBG = 0 first, then RESUME = 1. dret returns to the polling loop. A debug request suspends the wait, it does not end it.

- To stop on the application’s first instruction, the host halts the CPU, plants a breakpoint at 0x2000_1080 (7.7), writes DBG_BOOT.dbg_en_mask = 1 and resumes. The write is a word write of 0x0000_0002 to 0x8000_400C through the bus master; it needs no help from the CPU, so it also works while the CPU is halted.

Attach later is steps 3 to 5 with the application running instead of the polling loop.

The ROM itself cannot be debugged from its first instruction after a power-on: until ACKD the window is empty. After a watchdog or software reset it still holds the program, and 7.7 gives the way in.

## 7.3 Reset behaviour

Table 7-2. What each reset does

| Event | TCK domain | AXI domain | System domain | CPU |
| --- | --- | --- | --- | --- |
| Power-on | Reset | Reset | Reset: dbgreq = 0, DBG_EN captured again | SCRC releases it, it runs the ROM |
| Watchdog bite, software reset | – | Reset. A request still held is issued again on release | – (dbgreq and o_dbg_en keep their values) | SCRC releases it. With dbgreq = 1 it halts before its first instruction (7.7) |
| i_jtag_trst_n | Reset: dbgreq = 0 | – | – | – |

## 7.4 Reading and writing memory

Read of address A:

- IR = ADDR, DR = {0, A}. Update-DR starts the read.

- IR = STATUS, scan until busy = 0. Check resp.

- IR = DATA, scan. The captured value is the read data.

Write of D to address A:

- IR = ADDR, DR = {1, A}. Nothing is issued.

- IR = DATA, DR = D. Update-DR starts the write.

- IR = STATUS, scan until busy = 0. Check resp.

Rules:

- While busy = 1, Update-DR of ADDR and DATA is ignored: no register changes and no transaction starts.

- While addr_reg[32] = 1, every Update-DR of DATA starts another write, to the same address.

busy = read_req OR synchronised read_ack OR write_req OR synchronised write_ack, all on the TCK side.

## 7.5 AXI manager

Figure 7-1. Read handshake

set_ar is the rising edge of the synchronised read_req. set_aw is the same for write_req.

Table 7-3. AXI manager behaviour

| Event | Read | Write |
| --- | --- | --- |
| set_ar / set_aw | arvalid = 1, araddr = addr_reg, rready = 1 | awvalid = 1, awaddr = addr_reg, wvalid = 1, wdata = wdata_reg, bready = 1 |
| Address handshake | arvalid = 0 | awvalid = 0 |
| Data handshake | rdata_reg = rdata, resp_reg = rresp, rready = 0 | wvalid = 0 |
| Response handshake | – | resp_reg = bresp, bready = 0 |
| Acknowledge | read_ack = synchronised read_req after the delay flip-flop, AND NOT rready | write_ack = synchronised write_req after the delay flip-flop, AND NOT bready |

At most one transaction is outstanding.

## 7.6 Clock domain crossing

Each transaction is one 4-phase handshake:

- TCK side sets req.

- AXI side sees req rise and runs the transaction.

- AXI side raises ack and holds it while req = 1.

- TCK side sees ack rise, captures resp_reg into resp (and, for a read, rdata_reg into rdata_hold), then clears req.

- AXI side sees req fall and drops ack.

- TCK side sees ack fall and is no longer busy.

Table 7-4. Crossing signals and constraints

| Signal | Direction | How it crosses | Constraint |
| --- | --- | --- | --- |
| read_req, write_req | TCK to AXI | SyncStages flip-flops, then one delay flip-flop for the edge | set_max_delay -datapath_only, one i_clk_cpu period, to the first flip-flop |
| read_ack, write_ack | AXI to TCK | SyncStages flip-flops, then one delay flip-flop for the edge | the same, one TCK period |
| addr_reg, wdata_reg | TCK to AXI | No synchroniser. Stable from req rising until ack falls | set_max_delay -datapath_only, one i_clk_cpu period |
| rdata_reg, resp_reg | AXI to TCK | No synchroniser. Stable from ack rising until the next req | set_max_delay -datapath_only, one TCK period |
| dbgreq | TCK to system | SyncStages flip-flops | set_max_delay -datapath_only, one i_clk_cpu period, to the first flip-flop |

## 7.7 Halt and resume

- Halt. CPUDBG = 1 drives o_dbg_req high. Ibex saves the PC in dpc and jumps to DmHaltAddr = 0x2000_0800. The window code sets HALTED = 1; the host reads it to confirm the halt.

- A debug program must be there. Raise CPUDBG only after the ROM has sent ACKD, that is, after a frame with DEBUG_PROG = 0x55. Without it 0x2000_0800 holds whatever the RAM holds, and the halt runs it. SYSDBG does not check this.

- Halt after a reset. CPUDBG = 1 across a watchdog or software reset halts the core before its first instruction, at dpc = 0x0000_0080: debug_req is still high when SCRC releases the CPU, and the debug program is still in ISRAM, which no reset clears. After a power-on reset it does not work: CPUDBG is cleared and ISRAM holds nothing. A TRST clears CPUDBG as well.

- Resume. The host writes CPUDBG = 0 first, then RESUME = 1 in the debug window. The loop executes dret. If debug_req were still high, the core would halt again at the next instruction. The core goes back to where it was: the ROM’s polling loop, or the application.

- Breakpoints. DbgTriggerEn = 1 and DbgHwBreakNum = 2 give two hardware breakpoints, set by sequences that write tselect, tdata1 and tdata2. They work in the ROM, which cannot be patched. A software breakpoint is an ebreak written over the instruction in ISRAM and restored afterwards; it enters Debug Mode only if dcsr.ebreakm = 1, which a sequence sets once.

## 7.8 Debug window

The first 4 KiB of ISRAM, 0x2000_0000–0x2000_0FFF. The layout is the contract’s debug_window (util/qsoc_contract.yml) and the table below is generated from it. The same numbers are Ibex’s DmBaseAddr, DmAddrMask, DmHaltAddr and DmExceptionAddr, and QNSC_BOOT_SPEC Table 9-1.

Who writes what:

- The boot ROM writes the debug program, 4096 bytes from 0x2000_0000, when DEBUG_PROG = 0x55 (QNSC_BOOT_SPEC 6 and 7). The dispatch loop, the exception handler and the mailbox are part of that program.

- SYSDBG writes the sequence area at run time, one sequence at a time, and reads and writes the mailbox (7.4).

- Firmware places nothing here. The application is linked at 0x2000_1000.

Table 7-5. Debug window, the first 4 KiB of ISRAM

| Offset | Address | Size | Name | Contents |
| --- | --- | --- | --- | --- |
| 0x000–0x7FF | 0x20000000 | 2048 B | sequence area | Instruction sequences, written by the host through SYSDBG |
| 0x800–0x80F | 0x20000800 | 16 B | DmHaltAddr | Dispatch loop entry: Ibex enters here on every halt |
| 0x810–0x81F | 0x20000810 | 16 B | DmExceptionAddr | Exception handler: a fault in Debug Mode |
| 0x820–0xEFF | 0x20000820 | 1760 B | code | Rest of the dispatch loop |
| 0xF00 | 0x20000F00 | 4 B | data | Result word, the value a sequence was sent to fetch |
| 0xF04 | 0x20000F04 | 4 B | exc | Exception flag, 1 after a fault in Debug Mode |
| 0xF08 | 0x20000F08 | 4 B | cmd | Command word, non-zero when a sequence is ready at the base; the sequence clears it |
| 0xF0C | 0x20000F0C | 4 B | resume | Resume flag, non-zero tells the dispatch loop to execute dret; the loop clears it first |
| 0xF10 | 0x20000F10 | 4 B | halted | Halted flag, 1 while the core is in Debug Mode |
| 0xF14–0xFFF | 0x20000F14 | 236 B | reserved | Scratch, unused |

The code of the dispatch loop and of the sequences is host software and part of the debug program, in _DECISIONS, “Debug window code”. The ROM does not look at the content of the program, only at its CRC.

# 8. Instances

One, in design/top, with the default parameters.

# 9. What is not provided here, and who provides it

Table 9-1. Functions this block does not provide

| Function | Where it lives |
| --- | --- |
| Reset of the chip or of any domain, holding the CPU | Nowhere. The CPU is never held (V4.0): the boot ROM runs after every reset |
| CPU clock control | Not needed. The cpu cluster is never gated |
| Byte and halfword access, bursts | Nowhere. Word only; the host does read-modify-write |
| Debug ROM, program buffer | Nowhere. The debug program in ISRAM, loaded by the boot ROM, 7.8 |
| dret, stepping, ebreak, triggers | Ibex, driven by code in the debug window. Two hardware triggers (DbgTriggerEn = 1, DbgHwBreakNum = 2) |

# 10. Tie-offs

Table 10-1. Tie-offs

| Port | Tied to | Why |
| --- | --- | --- |
| o_bus_axi_ar_len, o_bus_axi_aw_len | 0 | One beat |
| o_bus_axi_ar_size, o_bus_axi_aw_size | 3'b010 | 4 bytes, word only |
| o_bus_axi_ar_burst, o_bus_axi_aw_burst | 2'b01, INCR | Legal for one beat |
| o_bus_axi_ar_id, o_bus_axi_aw_id | 0 | One transaction outstanding |
| o_bus_axi_{ar,aw}_lock, _prot, _cache, _qos, _region | 0 | No exclusive access, protection, cache hints or QoS |
| o_bus_axi_w_strb | 4'b1111 | Word only |
| o_bus_axi_w_last | 1 | One beat |
| i_bus_axi_r_id, i_bus_axi_r_last, i_bus_axi_b_id | Unused | One beat, one ID |

# 11. Requirements on others, and open items

Table 11-1. Requirements on other owners

| Item | Owner | What it blocks |
| --- | --- | --- |
| DBG_EN on a dedicated input pad, pull-down on the board, wired to SYSCSR i_dbg_en and to this block’s i_dbg_en. It may share a pad with a function the IO MUX selects after reset (UART0_CTSN): SYSCSR reads the pin all the time, this block only once | Top, pad owner | Debug boot |
| i_rst_n_por from power-on only, not from the watchdog | SCRC | DBG_EN capture, and dbgreq surviving a watchdog bite (7.7) |
| DmBaseAddr, DmAddrMask, DmHaltAddr, DmExceptionAddr taken from the contract’s debug_window, tagged // contract: debug_window.<key>. DbgTriggerEn = 1, DbgHwBreakNum = 2. DmBaseAddr/DmAddrMask act only with PMP, which is off | CPU owner | 7.7, 7.8 |
| boot_addr_i is the ROM base, no mux. fetch_enable_i tied to IbexMuBiOn: the CPU is not held | CPU owner | The CPU running the ROM after every reset |
| AXI_S0 connected to SYSDBG directly, AXI4, ID width AxiIdWidth. No axi_from_mem | Bus owner | Every bus access |
| AXI_S0 reaches SYSCSR (APB_M1) through S_BUS, AXI2APB and P_BUS: a word write to 0x8000_400C | Bus owner | Ending the ROM’s wait (7.2 step 6) |
| Remove the 0xF000_0000 SYSDBG register region from HAS Table 7-1 (already gone from qsoc_contract.yml) | HAS owner | Consistency. There are no memory-mapped registers |
| The five JTAG pads forced to JTAG while o_dbg_en = 1 | IO MUX owner | A debug session with firmware that remaps pins |
| ISRAM array has no reset and no clear-on-reset | RAM owner | The debug program surviving a watchdog or software reset (7.7) |
| QNSC_BOOT_SPEC Table 9-1 adds HALTED at 0x2000_0F10 and cites the contract’s debug_window instead of _DECISIONS 4.11 | Boot owner | The layout of 7.8 in one place |
| The 4096-byte debug program: dispatch loop, exception handler, mailbox. It goes into the frame with qsoc_image.py --debug | Lead (this block) | Every halt |
| Image linked at 0x2000_1000, reset entry 0x2000_1080; bootloader jumps to 0x2000_1080 | Firmware owner | The same image in every case of 7.2 |

Accepted limits:

- No timeout: a slave that never answers keeps busy = 1 until an S_BUS reset or power-on.

- While DBG_EN = 0, firmware can take the JTAG pins.

- No halt before the first ROM instruction after a power-on: the window is empty until the ROM loads the program (7.2, 7.7).

- SYSDBG does not check that a debug program is in the window before a halt (7.7).

# 12. Verification

- TAP: all 16 states from every state; IDCODE after reset; BYPASS and unused IR codes are 1 bit; IR changes only at Update-IR; Capture-IR loads 0001.

- Each data register of section 6: length, capture value, update effect, reset value.

- Read and write sequences of 7.4 against an AXI slave model with random ready/valid delays. Every transaction issued exactly once, with the fixed fields of section 10.

- Update-DR of ADDR or DATA while busy changes nothing.

- SLVERR and DECERR reach STATUS.resp.

- AXI protocol checker on the manager port for every test.

- CDC: TCK from 10x slower to 10x faster than i_clk_cpu, and TCK stopped mid-handshake for 10,000 cycles. CDC lint: only the signals of the crossing table cross.

- i_rst_n_sysbus asserted during a transaction: the transaction is issued again after release, and completes.

- DBG_EN: o_dbg_en = 0 until the capture, then the pin value at the capture. Toggling the pad afterwards changes nothing. o_dbg_en never glitches (it is a flip-flop output).

- With Ibex and the boot ROM, debug boot: frame with DEBUG_PROG = 0x55. After ACKD, CPUDBG = 1: HALTED = 1 and dpc is inside the ROM’s polling loop. CPUDBG = 0, RESUME = 1: HALTED = 0, the core is back in the loop and halts no more.

- With Ibex, attach later: DBG_EN = 0, the application runs; halt, read dpc, resume.

- The ROM’s wait: with the CPU halted, write DBG_BOOT = 0x0000_0002 through SYSDBG; resume; the ROM jumps to 0x2000_1080.

- Halt after a reset: CPUDBG = 1, watchdog reset; the core halts at dpc = 0x0000_0080 with the program intact. After a power-on reset it does not, and CPUDBG reads 0.

- Breakpoints: a hardware trigger in the ROM and at 0x2000_1080; an ebreak in ISRAM with dcsr.ebreakm = 1.

- DEBUG_PROG not 0x55 and DBG_EN = 1: the ROM waits, the host writes dbg_en_mask = 1, the application runs.

Acceptance: send a frame with a debug program and an application that toggles a GPIO, with DBG_EN = 1. Halt the CPU in the ROM’s wait loop and read dpc. Plant a breakpoint at 0x2000_1080, write dbg_en_mask = 1 through SYSDBG and resume. The core stops at the breakpoint; resume it and see the GPIO toggle.

# Appendix A. Acronyms

Table A-1. Acronyms

| Acronym | Description |
| --- | --- |
| CDC | Clock Domain Crossing |
| DR, IR | JTAG Data and Instruction Register |
| dpc | Debug PC: where dret returns |
| dret | Instruction that leaves Debug Mode |
| POR | Power-On Reset |
| SCRC | System Clock Reset Control |
| TAP | Test Access Port, IEEE 1149.1 |

# Appendix B. First review

Table B-1. First review

| Item | Reviewer | Response |
| --- | --- | --- |
| The block diagram does not show where the FSM and the CDC are | Teacher, 2026-09-23 | Section 3 separates the three domains. 7.6 gives the handshake, its signals and its constraints |
| Too long, and the repetition causes errors | Teacher, 2026-09-23 | Rewritten as V3.0. The reasoning moved to _DECISIONS |
| Who loads the 4 KiB debug program, and when? | Teacher, 2026-09-23 | V3.0: the host, over JTAG. V4.0: the boot ROM, over UART0, when the frame has DEBUG_PROG = 0x55 – 7.2, 7.8 |
| The debugger must control the CPU reset, selected by an external pin | Teacher, 2026-09-23 | V3.0: DBG_EN and CPUHOLD. Dropped in V4.0: the mentor’s boot flow of 12/10 never holds the CPU – 7.1, _DECISIONS V4.0 |
| Reference design VLSI_SYSDBG.drawio | Teacher, 2026-09-23 | Used as the pattern and adapted to QSOC – _DECISIONS D18 |
| CPU clock under debugger control | Teacher, 2026-09-23 | Not needed: the cpu cluster is never gated |
| IP list names axi_from_mem for the debugger | Owner, 2026-09-25 | An early pick from the V1.x memory-style port. The block is self-designed, as the teacher asks: its own AXI4 manager (7.5) connects to AXI_S0 directly, and no IP is instantiated – _DECISIONS D21 |
| The CPU is never held and the boot ROM loads the debug program | Quan (mentor), 12/10 and 14/10, in QNSC_BOOT_SPEC V3.4 | V4.0: 1, 7.1, 7.2, 7.8 |
| This document and QNSC_BOOT_SPEC Table 9-1 gave different mailbox addresses (0x700, 0xF00) | Lead, 14/10 | 7.8 and the contract: the numbers of Table 9-1 |