# I2C

MICRO-ARCHITECTURE SPECIFICATION – V3.1

QUY NHON SEMICONDUCTORS – QNSC

# Table of Contents

# Table of Tables

Table 1-1. Memory map, regions behind APB_M11

Table 3-1. Sub-blocks

Table 4-1. Upstream IP used

Table 5-1. m_qnsc_wrap_i2c interface

Table 6-1. Register map

Table 6-2. STATUS and CMD fields

Table 7-1. Command sequences

Table 7-2. Prescaler at 20 MHz (estimate without rise time)

Table 9-1. Functions this block does not provide

Table 11-1. Requirements on other owners

Table A-1. Acronyms

Table B-1. First review

# Table of Figures

Figure 3-1. I2C in QSOC

Figure 6-1. Register fields

Figure 7-1. Writing one data byte: two commands

# Revision history

The reasoning behind each change, and the research report this specification is built on, are in QNSC_I2C_DECISIONS.md.

| Version | Date | Author | Reviewer | Description of change |
| --- | --- | --- | --- | --- |
| V3.0 | 2026-09-28 | Nghia VT (lead), for Ong Bao Vinh | – | Specification written from the research report: upstream apb_i2c unmodified (the DMA registers TXCMD/RXCMD dropped), wrapper m_qnsc_wrap_i2c, open-drain as output enables to the IO MUX, 20 MHz prescaler values |
| V3.1 | 2026-10-15 | Nghia VT (lead), for Ong Bao Vinh | – | i_bus_apb_paddr is 14 bits, was 12: the contract meta.apb_paddr_width changed in #77 because the windows are 16 KiB. No change in behaviour |

# 1. Overview

I2C is one instance of pulp-platform/apb_i2c, an I2C master built on the OpenCores I2C core: START, repeated START, STOP, ACK and NACK, clock stretching, multi-master clock synchronisation and arbitration-loss detection.

The fact that shapes it: firmware drives the bus one byte per command. It writes a byte and a command, waits for the interrupt flag, and issues the next; the core has no FIFO.

The block is not an I2C slave, and has no DMA request.

Block directory design/i2c, wrapper m_qnsc_wrap_i2c, owner Ong Bao Vinh.

Table 1-1. Memory map, regions behind APB_M11

| Base | Size | Region | Port | Kind | Note |
| --- | --- | --- | --- | --- | --- |
| 0x8002C000 | 16 KiB | i2c | APB_M11 | peripheral |  |

Interrupt: fast line 3, mcause 19. SCRC domain i2c, CLK_EN[8].

# 2. Features

- Standard mode (100 kHz) and fast mode (400 kHz) from a 16-bit prescaler – 7.2.

- One command per byte: START, STOP, read, write, ACK or NACK – 7.1.

- Clock stretching by the slave; SCL synchronisation with another master – 7.3.

- Arbitration loss and bus-busy status – 7.3.

- One level interrupt, set on every completed command and on arbitration loss – 7.4.

- Zero-wait-state APB, never an error – 6.

# 3. Block diagram

Figure 3-1. I2C in QSOC

Table 3-1. Sub-blocks

| Block | Module | Function |
| --- | --- | --- |
| Registers | apb_i2c | APB slave, 6 registers, interrupt flag |
| Byte controller | i2c_master_byte_ctrl | Executes one command: shifts a byte, handles ACK |
| Bit controller | i2c_master_bit_ctrl | SCL generation, input filter, START/STOP detection, stretching, arbitration |
| Output enables | in m_qnsc_wrap_i2c | o_pad_i2c_*_oe = NOT *_padoen_o: 1 drives the line low |

The block is in the peri clock cluster.

# 4. IP used

Table 4-1. Upstream IP used

| From | Module | Commit | Licence |
| --- | --- | --- | --- |
| pulp-platform/apb_i2c | apb_i2c, i2c_master_byte_ctrl, i2c_master_bit_ctrl | 84855413 | OpenCores notice in the controller files; no LICENSE file upstream |

Facts this specification relies on, read in the vendored source:

- PADDR[5:2] is decoded; PREADY = 1 and PSLVERR = 0 always (apb_i2c.sv).

- scl_pad_o and sda_pad_o are constant 0 (i2c_master_bit_ctrl.sv): a line is either driven low or released.

- The interrupt flag is sticky: set by a completed command or arbitration loss, cleared by CMD.IACK; interrupt_o = flag AND CTRL.IEN, from a flip-flop.

- CMD is written only while CTRL.EN = 1.

# 5. Interface

Table 5-1. m_qnsc_wrap_i2c interface

| Signal | Dir | Width | Description |
| --- | --- | --- | --- |
| i_clk_peri, i_rst_n_peri | in | 1 | SCRC o_clk_i2c, o_rst_n_i2c |
| i_bus_apb_psel, i_bus_apb_penable, i_bus_apb_pwrite | in | 1 | P_BUS APB_M11 |
| i_bus_apb_paddr | in | 14 | Offset in the window (contract meta.apb_paddr_width) |
| i_bus_apb_pwdata | in | 32 | Write data |
| i_bus_apb_pstrb, i_bus_apb_pprot | in | 4, 3 | Not used |
| o_bus_apb_prdata, o_bus_apb_pready, o_bus_apb_pslverr | out | 32, 1, 1 | pready = 1, pslverr = 0 |
| o_int_i2c | out | 1 | interrupt_o, level – 7.4. To INTMAP i_int_i2c |
| i_pad_i2c_scl, i_pad_i2c_sda | in | 1 | Line level, from the pad |
| o_pad_i2c_scl_oe, o_pad_i2c_sda_oe | out | 1 | 1 = drive the line low; 0 = release. The pad drives 0 when enabled |

# 6. Register map

The core decodes offset bits 5:2: offsets 0x18–0x3C read 0 and ignore writes, and the 64-byte map repeats every 64 bytes in the window. No access returns an error.

Table 6-1. Register map

| Offset | Register | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 0x00 | PRER | RW | 0 | Prescaler, bits 15:0 – 7.2 |
| 0x04 | CTRL | RW | 0 | Bit 7 EN core enable, bit 6 IEN interrupt enable |
| 0x08 | RX | RO | 0 | Last byte received |
| 0x0C | STATUS | RO | 0 | Figure 6-1 |
| 0x10 | TX | RW | 0 | Byte to send: data, or address and R/W bit |
| 0x14 | CMD | RW | 0 | Command, Figure 6-1. Writable only while EN = 1 |

Figure 6-1. Register fields

Table 6-2. STATUS and CMD fields

| Register | Bit | Name | Meaning |
| --- | --- | --- | --- |
| STATUS | 7 | RXACK | 1 = the slave did not acknowledge (NACK) |
| STATUS | 6 | BUSY | A START has been seen on the bus and no STOP yet, from any master |
| STATUS | 5 | AL | Arbitration lost; held until the next START |
| STATUS | 1 | TIP | A transfer is in progress |
| STATUS | 0 | IF | Interrupt flag |
| CMD | 7 / 6 | STA / STO | Generate START (or repeated START) / STOP |
| CMD | 5 / 4 | RD / WR | Read / write one byte |
| CMD | 3 | ACK | When reading: 0 = send ACK, 1 = send NACK |
| CMD | 0 | IACK | Write 1 to clear IF |

CMD bits 7:4 clear themselves when the command ends or arbitration is lost; IACK clears after one cycle.

# 7. Functional behaviour

## 7.1 Commands

Figure 7-1. Writing one data byte: two commands

Each command moves one byte and ends by setting IF. START and STOP travel with the byte command they belong to.

Table 7-1. Command sequences

| Transaction | Writes, each followed by waiting for IF and writing CMD = IACK |
| --- | --- |
| Write register r of slave a with d | TX = a<<1, CMD = STA¦WR – TX = r, CMD = WR – TX = d, CMD = WR¦STO |
| Read register r of slave a | TX = a<<1, CMD = STA¦WR – TX = r, CMD = WR – TX = (a<<1)¦1, CMD = STA¦WR – CMD = RD¦ACK¦STO, then read RX |

After each write command firmware checks STATUS.RXACK = 0; after each command it checks STATUS.AL = 0.

## 7.2 SCL frequency

- A bit takes 4 phases of (PRER + 1) cycles, plus the input filter delay and the rise time of the line: TSCL ≈ [4 (PRER + 1) + 2 ((PRER >> 2) + 1) + 3] x 50 ns + trise.

- The OpenCores rule PRER = f / (5 fSCL) - 1 overshoots: at 100 kHz it gives about 109 kHz.

Table 7-2. Prescaler at 20 MHz (estimate without rise time)

| Mode | PRER | fSCL, estimated |
| --- | --- | --- |
| Standard, 100 kHz | 44 | 96.6 kHz |
| Fast, 400 kHz | 10 | 377 kHz |

The rise time only lowers the frequency; I2C_003 measures it.

## 7.3 Bus conditions

- Clock stretching: after releasing SCL the core waits until the line is high.

- Multi-master: a falling SCL driven by another master restarts the local phase, so both masters share one SCL.

- Arbitration: releasing SDA for a 1 and reading 0, or an unexpected STOP, sets AL and IF; the core releases both lines and ends the command.

- Busy: BUSY follows START and STOP on the bus, whoever sends them.

## 7.4 Interrupt

- o_int_i2c = IF AND IEN, a level from a flip-flop. IF is set by the end of every command and by arbitration loss, and is cleared only by CMD.IACK.

- With IEN = 0 firmware polls STATUS.IF.

## 7.5 Pads

- Each line is open drain: o_pad_i2c_*_oe = 1 pulls it low, 0 releases it; the board pull-up makes it high. i_pad_i2c_* is the line level, so the core sees the wired-AND of every device.

- The inputs pass the core’s own synchroniser and filter.

# 8. Instances

One m_qnsc_wrap_i2c in design/top.

# 9. What is not provided here, and who provides it

Table 9-1. Functions this block does not provide

| Function | Where it lives |
| --- | --- |
| I2C slave mode | not provided |
| FIFO, DMA request | not provided; one command per byte |
| Open-drain pad cells, pull-ups | IO MUX and pads; board |
| 10-bit addressing | firmware: two address bytes |

# 10. Tie-offs

None. Every core port reaches a wrapper port; the wrapper inverts the two output enables.

# 11. Requirements on others, and open items

Table 11-1. Requirements on other owners

| Item | Owner | What it blocks |
| --- | --- | --- |
| Open-drain pads for SCL and SDA: drive 0 when oe = 1, high impedance otherwise; line level back on the input | IO MUX, pad owner | Every transfer |
| Pull-up resistors on SCL and SDA | board | Every transfer |

Open items, I2C owner: wrapper RTL, lint, and the tests of section 12.

Accepted limits:

- An access to an unused offset returns 0 without PSLVERR; the map aliases every 64 bytes.

- The register layout differs from the original OpenCores map (separate RX/TX and STATUS/CMD); OpenCores drivers need new offsets.

# 12. Verification

- I2C_001 Reset values; CMD ignores writes while EN = 0.

- I2C_002 Write and read sequences of 7.1 against an I2C slave model, with ACK and NACK.

- I2C_003 SCL frequency for PRER = 44 and 10 with a modelled rise time.

- I2C_004 A slave stretching SCL delays the bit without error.

- I2C_005 Arbitration against a second master: AL and IF set, both lines released.

- I2C_006 o_int_i2c follows IF AND IEN; IACK clears it.

- I2C_007 o_pad_i2c_*_oe is never 1 while the core releases the line (open-drain rule).

# Appendix A. Acronyms

Table A-1. Acronyms

| Acronym | Description |
| --- | --- |
| AL | Arbitration Lost |
| NACK | Not acknowledged |
| SCL, SDA | Serial clock, serial data |

# Appendix B. First review

Table B-1. First review

| Item | Reviewer | Response |
| --- | --- | --- |
| dma_last_i floating in the wrapper | Vinh (research, F1) | DMA registers dropped with the patch |
| DMA request ports not brought out | Vinh (research, F2) | Not needed: QNSC_DMA_MAS V3.0 has no hardware request |
| OpenCores prescaler rule overshoots | Vinh (research, F3) | 7.2 |
| 1'bz in the wrapper is not synthesisable | Vinh (research, F5) | Output enables to the IO MUX, 7.5 |
| Alias every 64 B, no PSLVERR | Vinh (research, F7) | Accepted limit |
| Register map differs from OpenCores | Vinh (research, F9) | Accepted limit |
| Wrapper name, clock names, address width | lead, V3.0 | m_qnsc_wrap_i2c, i_clk_peri, 12-bit PADDR |