# PWM

MICRO-ARCHITECTURE SPECIFICATION – V1.5

QUY NHON SEMICONDUCTORS – QNSC

# Table of Contents

# Table of Figures

Figure 1-1. Figure 1 – The PWM block and where its outputs go

# Reversion and History

| Version | Date | Author/Owner | Description of Change |
| --- | --- | --- | --- |
| V1.0 | 2026-09-21 | Nghia VT | First issue. Covers the single apb_adv_timer instance that provides QSOC’s PWM outputs. The companion document is QNSC_TIMER_MAS, which covers TIMER0 and TIMER1 – a different IP at different APB ports. Two findings in this document were read from the RTL and are not in any datasheet: the four event lines are a 4-of-16 multiplexer over the channel outputs, section 4.5, and the per-timer status output is not reachable by software, section 4.6. |
| V1.1 | 2026-09-21 | Nghia VT | dft_cg_enable_i position corrected against the Day005 review. v1 ties it 0 because the review found the combined test mode / JTAG pad entry was copied from another chip and removed it – so the earlier requirement to drive it from a test-mode pin was unsatisfiable. The same review ruled the spec must not assert that QSOC has no scan, so section 5.4 now records a tie-off plus the condition for revisiting it, rather than either a hard requirement or a claim of no DFT. |
| V1.2 | 2026-09-21 | Nghia VT | External triggers settled against the team’s IOPAD_Pin_Summary and the IP’s own RTL. Four pads already exist – TIM_EXT0–3 on PIN_37–40 – so the open question narrows to which ext_sig_i bits they attach to. The selection register this document previously proposed is dropped: input_stage.sv shows each module already selects one signal with an 8-bit cfg_sel. Reading that code also found the pool is 48 signals, not 32 (EXTSIG_NUM + 16), because it includes the sixteen channel outputs – so a module can be triggered by another module’s channel, which qualifies the independence claim in section 4.1. Table 7 now cites the pad list as independent confirmation. |
| V1.3 | 2026-09-21 | Nghia VT | Three open questions closed, one narrowed. The eight pad channels are fixed as ch_0_o and ch_1_o of each module, for four independent frequencies at the pins rather than two; the four TIM_EXT pads attach to ext_sig_i[3:0] in pad order, so the mapping reads off the pinout; and centre-aligned output is recorded as available but unverified – up_down_counter.sv shows it is one bit, cfg_sawtooth_i, so removing it would be work rather than saving. The idle-level question is narrowed to a proposal plus the RTL reason it matters: out_filter.sv only updates while ctrl_active_i is high, so a stopped or gated channel holds its last level and does not return to a safe one by itself. |
| V1.4 | 2026-09-21 | Nghia VT | The idle-level item is closed by changing whose problem it is. out_filter.sv only updates its stored output while ctrl_active_i is high, so the block has no idle level to configure – a stopped channel holds the last level it drove. Section 5.9 therefore specifies a four-step firmware procedure ending in never gate a running block, and what remains for the pad owner shrinks to a one-line check that no pad pull fights the rest level. The document now has no open questions. |
| V1.5 | 2026-09-22 | Nghia VT | Removes an internal inconsistency: section 5.6 still called the ext_sig_i bit mapping a proposal left to settle, while section 5.9 had already decided it as ext_sig_i[3:0] in pad order. Section 5.6 now points at the decision. |
| V1.6 | 2026-09-22 | Nghia VT | Corrects the pad-channel mapping against the RTL. V1.3 stated the eight pad channels were ch_0_o and ch_1_o of each of the four modules, for four independent frequencies. Reading apb_adv_timer.sv shows the channels are grouped by module – u_tim0.pwm_o -> ch_0_o, u_tim1.pwm_o -> ch_1_o, u_tim2.pwm_o -> ch_2_o, u_tim3.pwm_o -> ch_3_o – so ch_i_o[3:0] is the four channels of module i. Bringing out ch_0_o and ch_1_o is therefore all four channels of modules 0 and 1, giving two independent base frequencies (four channels each), not four. Sections 4.x pad-mapping paragraph, 5.9 and the requirements table are corrected; QSOC_HAS already uses the two-frequency figure. |

# Table of Tables

| Table | Title |
| --- | --- |
| Table 1 | IP sourcing decision |
| Table 2 | What this block is not |
| Table 3 | Achievable PWM frequency and resolution at 20 MHz |
| Table 4 | Block interface |
| Table 5 | Register map |
| Table 6 | Registers of one timer module |
| Table 7 | What firmware must program before any interrupt occurs |
| Table 8 | Address decode against region size |
| Table 9 | Channel outputs against pads |
| Table 10 | Clock and reset domain |
| Table 11 | Interrupt sources contributed to INTMAP |
| Table 12 | Verification QSOC must add |
| Table 13 | Interfaces to agree with the team |
| Table 14 | Acronyms |

# Table of Figures

| Figure | Title |
| --- | --- |
| Figure 1 | The PWM block and where its outputs go |

# 1. Overview

## 1.1 Scope

This document specifies the integration of one apb_adv_timer instance as QSOC’s PWM block: its register map, the channel outputs and how many reach pads, the event lines it contributes to INTMAP, the clock and reset domain, and the address decode.

It does not cover TIMER0 or TIMER1. Those are two instances of a different IP, apb_timer_unit, at APB_M7 and APB_M8, specified in QNSC_TIMER_MAS. The distinction is stated first because confusing the two is exactly what QSOC_HAS records as a disagreement between documents, and because the confusion is easy to make: the PWM IP is called a timer and contains four of them.

Table 1 – IP sourcing decision

| Item | Value |
| --- | --- |
| IP | pulp-platform/apb_adv_timer |
| Sub-modules | timer_module, up_down_counter, comparator, prescaler, lut_4x4, out_filter, input_stage, timer_cntrl, adv_timer_apb_if |
| Instances | one |
| Bus | APB slave, plain signals, no adapter required |
| Parameters | APB_ADDR_WIDTH = 12, TIMER_NBITS = 16, EXTSIG_NUM = 32 |
| Written by QSOC | instantiation, tie-off and the pad mux; no RTL inside the block |

Table 2 – What this block is not

| Not the timebase | its counters are 16-bit and wrap every 3.2768 ms at 20 MHz |
| --- | --- |
| Not TIMER0 or TIMER1 | different IP, different ports, different clock gates |
| Not a source of mtime | QSOC has no machine timer; see QNSC_TIMER_MAS section 5.5 |
| Not readable for “which event fired” | section 4.6 |

## 1.2 Position in the system

The block is an APB slave on P_BUS, reached from the CPU through AXI2APB on AXI_M3. Unlike the timers, it does drive pads: eight of its sixteen channel outputs leave the chip through the IO mux. It also contributes four event lines to INTMAP.

Figure 1-1. Figure 1 – The PWM block and where its outputs go

# 2. Feature

## 2.1 Feature – PWM

- One apb_adv_timer instance containing four independent timer modules.

- Each module has a 16-bit up or up-down counter, an 8-bit prescaler and four comparators, giving sixteen channel outputs in total.

- Each channel has a 4-entry lookup table that combines the comparator result with the previous channel state, which is how the IP produces the different output shapes from one compare.

- Eight of the sixteen channels reach pads as PWM_0 to PWM_7, each shared with a GPIO function through the IO mux.

- Four event lines to INTMAP, each a one-cycle pulse, each selected from any one of the sixteen channel outputs.

- 32 external signal inputs able to start, stop or gate a module without CPU action; QSOC wires a subset, section 5.6.

What the block does not provide:

- No software-readable event status. Section 4.6 shows the RTL evidence. Firmware cannot ask the block which event fired or whether one was missed.

- No interrupt on its own. Out of reset the four event lines are inert until firmware programs the event configuration, section 4.5.

- No error reporting. PSLVERR behaviour is as for the other APB peripherals in QSOC: the block cannot report a bad access.

# 3. Block Diagram

Figure 1 in section 1.2 is the block diagram: the APB register file, four timer modules, the sixteen channel outputs, and the two destinations those outputs feed – the pad mux and the event multiplexer.

# 4. Micro-architecture Details

## 4.1 Structure

Four timer_module instances, each parameterised NUM_BITS = TIMER_NBITS = 16, sit behind one APB register file. Inside a module:

| Sub-module | Function |
| --- | --- |
| prescaler | 8-bit divider, field PRESC, ahead of the counter |
| up_down_counter | the 16-bit counter; counts up, or up then down for centre-aligned output |
| comparator x4 | one per channel, compares the count against that channel’s threshold |
| lut_4x4 x4 | turns the comparator result plus the current output into the next output |
| out_filter | output conditioning |
| timer_cntrl | start, stop and reset, from APB or from an external signal |

The four modules have independent counters, periods and prescalers. Two channels on the same module share a time base and can therefore be phase-related; channels on different modules are not phase-related unless firmware arranges it – which it can, because a module’s trigger pool includes the other modules’ channel outputs, section 5.6.

## 4.2 Counter width, and why 16 bits is the right choice here

The counters are 16-bit. At QSOC’s 20 MHz that is a maximum period of 3.2768 ms, so 305 Hz is the lowest frequency reachable without the prescaler, and 1.19 Hz with the prescaler at its maximum divide of 256.

That range covers every use QSOC has, with resolution to spare.

Table 3 – Achievable PWM frequency and resolution at 20 MHz

| Intended use | Frequency | Counts per period | Duty steps available |
| --- | --- | --- | --- |
| LED dimming | 1 kHz | 20 000 | 20 000 |
| Motor drive | 20 kHz | 1 000 | 1 000 |
| Audio-rate output | 44.1 kHz | 454 | 454 |
| Slowest without prescaler | 305 Hz | 65 536 | 65 536 |
| Slowest with prescaler | 1.19 Hz | 65 536 x 256 | 65 536 |

A 16-bit counter is not a limitation for PWM: the human eye resolves a few dozen brightness levels and 20 000 steps is far beyond that. The same 16 bits would be a serious limitation for a timebase, which is why the timebase is a different IP. QNSC_TIMER_MAS section 4.1 carries that comparison.

## 4.3 Block interface

Table 4 – Block interface

| Signal | Dir | Width | Note |
| --- | --- | --- | --- |
| HCLK | in | 1 | domain clock, section 5.3 |
| HRESETn | in | 1 | active low |
| PADDR | in | 12 | decoded as a register index, section 4.7 |
| PWDATA | in | 32 |  |
| PWRITE, PSEL, PENABLE | in | 1 each |  |
| PRDATA | out | 32 |  |
| PREADY, PSLVERR | out | 1 each |  |
| low_speed_clk_i | in | 1 | sampled as data, not used as a clock |
| dft_cg_enable_i | in | 1 | bypass for the internal clock gates, section 5.4 |
| ext_sig_i | in | 32 | external start, stop and gate triggers |
| events_o | out | 4 | to INTMAP; one-cycle pulses |
| ch_0_o, ch_1_o, ch_2_o, ch_3_o | out | 4 each | 16 channel outputs |

## 4.4 Register map

The block decodes an 8-bit register index. Each timer module occupies 0x40 bytes, and two global registers follow the four modules.

Table 5 – Register map

| Offset range | Contents |
| --- | --- |
| 0x000 – 0x02C | timer module 0 |
| 0x040 – 0x06C | timer module 1 |
| 0x080 – 0x0AC | timer module 2 |
| 0x0C0 – 0x0EC | timer module 3 |
| 0x100 | EVENT_CFG – selects and enables the four event lines |
| 0x104 | CH_EN – channel output enables |

Table 6 – Registers of one timer module

Offsets shown relative to the module base, which is 0x000, 0x040, 0x080 or 0x0C0.

| Offset | Name | Function |
| --- | --- | --- |
| +0x00 | CMD | start, stop, update, reset, arm |
| +0x04 | CFG | clock source, prescaler, up or up-down mode |
| +0x08 | TH | period threshold – the value the counter counts to |
| +0x0C | CH0_TH | compare threshold, channel 0 – the duty cycle |
| +0x10 | CH1_TH | compare threshold, channel 1 |
| +0x14 | CH2_TH | compare threshold, channel 2 |
| +0x18 | CH3_TH | compare threshold, channel 3 |
| +0x1C | CH0_LUT | output shape, channel 0 |
| +0x20 | CH1_LUT | output shape, channel 1 |
| +0x24 | CH2_LUT | output shape, channel 2 |
| +0x28 | CH3_LUT | output shape, channel 3 |
| +0x2C | COUNTER | current count, readable |

Period is TH, duty is CHn_TH. The two are separate registers, so changing the duty of one channel does not disturb the period or the other three channels on the same module.

## 4.5 The four event lines are a multiplexer over the channel outputs

This is the fact most likely to be assumed wrongly, so it is given its own section. The four event lines are not four dedicated comparators. They are four selections from the sixteen channel outputs, each followed by an edge detector:

assign s_event_signals = {ch_3_o, ch_2_o, ch_1_o, ch_0_o};      // 16 bits

assign events_o[0] = s_event_en[0] & r_event_sync_0[1] & ~r_event_sync_0[0];
assign events_o[1] = s_event_en[1] & r_event_sync_1[1] & ~r_event_sync_1[0];
assign events_o[2] = s_event_en[2] & r_event_sync_2[1] & ~r_event_sync_2[0];
assign events_o[3] = s_event_en[3] & r_event_sync_3[1] & ~r_event_sync_3[0];

Three consequences follow, and all three are firmware-visible.

An interrupt means “a channel I chose has just changed state”. It does not mean a period elapsed, and it does not identify which module or channel unless firmware remembers what it selected.

The pulse is exactly one HCLK cycle. The expression is the classic edge detector – present value AND the inverse of the previous value. INTMAP provides no latch, so these are the narrowest sources in QSOC and QNSC_Interrupt_Map_MAS classifies them as pulses on that basis.

Out of reset the block raises no interrupt at all. s_event_en comes from EVENT_CFG, which resets to zero. Until firmware writes a selection and an enable, the four lines stay low. This is the opposite of a fault: it means the block cannot interrupt a system that has not asked it to.

Table 7 – What firmware must program before any interrupt occurs

| Step | Register | Why |
| --- | --- | --- |
| 1 | CHn_LUT, CHn_TH, TH, CFG of the module | otherwise the channel never changes state |
| 2 | CH_EN | enable the channel output |
| 3 | EVENT_CFG – selection | choose which of the 16 channels drives each event line |
| 4 | EVENT_CFG – enable | set s_event_en for the lines wanted |
| 5 | mie bits for the PWM fast line, then mstatus.MIE | the core side, see QNSC_Interrupt_Map_MAS |

## 4.6 There is no software-readable event status

Each timer_module exposes an 8-bit status_o. In the top level it appears exactly twice – once declared, once connected:

logic [7:0] s_timer0_status;
...
    .status_o         ( s_timer0_status       )

and then nothing reads it. The APB interface module adv_timer_apb_if has no port whose name contains status. The signal is therefore not reachable by software in this version of the IP.

Consequence for QSOC. A PWM event that arrives while mstatus.MIE is clear is lost, and nothing anywhere records that it happened. There is no flag to poll and no counter to compare.

Why this is acceptable here. A PWM channel is periodic by construction: it changes state again on the next period, which at the frequencies in Table 3 is between 23 microseconds and 1 millisecond away. The recovery mechanism is periodicity, not a status register. That distinction matters, because it fails in exactly one case: a module configured for a single shot, or stopped immediately after the event. Firmware that uses PWM events as one-off notifications cannot rely on them.

This finding is the reason QNSC_Interrupt_Map_MAS must state its pulse-recovery argument in terms of periodicity rather than status registers.

## 4.7 Address decode against the region

The register file spans 0x000 to 0x107, that is 264 bytes, inside the 16 KiB region QSOC_HAS grants the block.

Table 8 – Address decode against region size

| Item | Value |
| --- | --- |
| Registers implemented | 4 modules x 12 registers, plus 2 global |
| Span actually decoded | 264 bytes, 0x000 to 0x107 |
| Region assigned | 16 KiB |
| Consequence | the decoded span aliases across the region |

As with the timers, an access beyond the implemented span neither faults nor reports an error. The recommendation is the same: accept the aliasing, document it, and do not spend logic on a fault firmware cannot observe.

# 5. Integration into QSOC

## 5.1 Address and port assignment

Taken from QSOC_HAS and not changed by this document.

| Port | Block | Base | Region |
| --- | --- | --- | --- |
| APB_M13 | PWM | 0x8003_4000 | 16 KiB |

## 5.2 Channel outputs and pads

Sixteen channels exist; eight leave the chip. The pad assignment is fixed by QSOC_HAS and every one of the eight is shared with a GPIO function.

Table 9 – Channel outputs against pads

| Pad name | Pin | Shared with |
| --- | --- | --- |
| PWM_0 | PIN_27 | GPIO1_1 |
| PWM_1 | PIN_28 | GPIO1_0 |
| PWM_2 | PIN_29 | GPIO2_7 |
| PWM_3 | PIN_30 | GPIO2_6 |
| PWM_4 | PIN_33 | GPIO2_5 |
| PWM_5 | PIN_34 | GPIO2_4 |
| PWM_6 | PIN_35 | GPIO2_3 |
| PWM_7 | PIN_36 | GPIO2_2 |

Which eight of the sixteen reach these pads is a decision this document must record and QSOC_HAS does not yet fix. In the RTL the channels are grouped by module: u_tim0.pwm_o -> ch_0_o, u_tim1.pwm_o -> ch_1_o, and so on, so ch_i_o[3:0] is the four channels of module i. Bringing out ch_0_o and ch_1_o therefore means all four channels of module 0 and all four of module 1 – two modules at the pads. Because channels within one module share that module’s counter and prescaler, this gives firmware two independent base frequencies at the pins, four channels (independent duty/phase) under each.

The remaining eight channels (modules 2 and 3) stay inside the chip and are still useful: any of the sixteen can be selected as an event source, section 4.5.

## 5.3 Clock and reset domain

Table 10 – Clock and reset domain

| Item | Value |
| --- | --- |
| Domain | D18 |
| Clock gate bit | bit 15 |
| Gate | own gate, independently gateable |
| Clock | 20 MHz system clock, no PLL in QSOC |
| Reset | synchronous release of the system reset, active low |
| low_speed_clk_i | not a clock: sampled as data, creates no second domain |

QSOC_HAS already records the treatment of low_speed_clk_i: the block samples it rather than clocking from it, so it introduces no clock domain crossing and needs no set_false_path. Tie it to the domain clock unless a slower counting rate is wanted.

Closing gate bit 15 freezes the channel outputs at their current level. For a motor drive that is not a neutral state. Firmware should stop the modules through CMD and let the outputs reach their idle level before the gate is closed, rather than gating a running block.

## 5.4 The internal clock gates and DFT

The IP instantiates clock gating cells internally and brings out dft_cg_enable_i to bypass them, so that scan can shift through logic that is otherwise gated.

For QSOC v1 this input is tied low, because there is nothing to drive it from. The pad table originally listed a combined test mode / JTAG pin, but the Day005 review of 2026-09-18 established that this entry was copied from another chip and removed it. With no test-mode pin in the pad ring, a tie-off is the only option.

This document does not claim that QSOC has no scan. The same review ruled that the spec must not assert the absence of test mode or scan, because DFT may be added later. The position recorded here is therefore narrower and is meant to be revisited:

| For v1 | dft_cg_enable_i tied 0, because no test-mode signal exists |
| --- | --- |
| What is not claimed | that QSOC will never have scan |
| What must happen if DFT is added | this input needs a real source, and the tie-off in the PWM wrapper is one of the places that must change |
| Why it is worth recording | the tie is harmless in simulation and only wrong on the tester – the same class of defect as the SRAM non-functional pins in QNSC_RAM_MAS |

The same applies to the four GPIO instances, which have the same input. Keeping the requirement visible costs one row in Table 13 and saves rediscovering it during a DFT pass.

## 5.5 Interrupt contribution

Table 11 – Interrupt sources contributed to INTMAP

| Source | From | Shape | Width |
| --- | --- | --- | --- |
| PWM event 0 | events_o[0] | pulse | 1 HCLK cycle |
| PWM event 1 | events_o[1] | pulse | 1 HCLK cycle |
| PWM event 2 | events_o[2] | pulse | 1 HCLK cycle |
| PWM event 3 | events_o[3] | pulse | 1 HCLK cycle |

Four sources on one fast line. This matches what QNSC_Interrupt_Map_MAS already assumes, so the interrupt contract needs no change from this document.

Because all four share one line, mcause identifies the block and not the event. The handler must know which channel it selected into which event slot; the block cannot tell it, section 4.6.

## 5.6 External triggers

ext_sig_i is 32 bits wide. Four of them reach pads, confirmed by the team’s IOPAD_Pin_Summary: TIM_EXT0 to TIM_EXT3 on PIN_37 to PIN_40, each sharing its pad with a GPIO function through the IO MUX.

| Pad | Pin | Shared with |
| --- | --- | --- |
| TIM_EXT0 | PIN_37 | GPIO2_1 |
| TIM_EXT1 | PIN_38 | GPIO2_0 |
| TIM_EXT2 | PIN_39 | GPIO3_7 |
| TIM_EXT3 | PIN_40 | GPIO3_6 |

The IP already selects per module, so no selection register is needed in the wrapper. An earlier revision of this document proposed one; reading the RTL shows it would duplicate what the IP does. Each timer_module takes an 8-bit cfg_sel and picks one signal out of a pool:

localparam N_TIMEREXTSIG = EXTSIG_NUM + 16;                          // 32 + 16 = 48
assign s_timer0_signal = {ch_3_o, ch_2_o, ch_1_o, ch_0_o, ext_sig_i};

and inside input_stage.sv the select walks that pool. The pool is 48 signals wide, not 32: the 32 external inputs plus the 16 channel outputs. Two consequences.

Any module can be triggered by any of the four pads, chosen at run time. The design-time decision is only which ext_sig_i bit each pad attaches to – the IOPAD_Pin_Summary left that unfixed, and section 5.9 fixes it: the lowest four, ext_sig_i[3:0], in pad order, so the mapping is guessable from the pin number rather than needing a table.

A module can also be triggered by another module’s channel output, because the pool includes all sixteen. This qualifies section 4.1: the four modules have independent counters, but they are not obliged to run unsynchronised – one can start another. No QSOC use needs it today, and it costs nothing to leave available.

The remaining 28 bits of ext_sig_i must be tied low, not left unconnected.

## 5.7 Verification QSOC must add

Table 12 – Verification QSOC must add

| # | Check | Why it is here |
| --- | --- | --- |
| 1 | Every register reads back what was written, at the documented offset | basic |
| 2 | The decoded span aliases as Table 8 states | pins the decode |
| 3 | TH sets the period and CHn_TH sets the duty, independently, on one module | the core function |
| 4 | Changing the duty of one channel disturbs neither the period nor the other three | the claim made in section 4.4 |
| 5 | Four modules run at four different frequencies simultaneously | the reason for the pad grouping in 5.2 |
| 6 | Out of reset, with EVENT_CFG untouched, events_o stays 0 | the inert-out-of-reset claim of 4.5 |
| 7 | Each event line can be selected from any of the 16 channels | proves the 4-of-16 mux |
| 8 | Every event pulse is exactly one HCLK cycle | the number INTMAP depends on |
| 9 | ext_sig_i starts and stops a module as configured | otherwise the input is untested |
| 10 | Eight channels reach the correct pads and the IO mux selects between PWM and GPIO | the only pad-facing path in this block |
| 11 | With dft_cg_enable_i tied 0, the block still functions normally; the input is proven reachable in RTL so a future DFT pass can drive it | section 5.4 – v1 has no scan, but the path must not be optimised away |
| 12 | Closing gate bit 15 freezes outputs; stopping via CMD first leaves them idle | the motor-drive hazard of 5.3 |

Test 6 asserts that nothing happens, and like test 7 in QNSC_TIMER_MAS it is the one most likely to be dropped. It is also the one that protects the boot sequence: a block that interrupts before firmware is ready is hard to debug from the symptom.

## 5.8 Interfaces to agree with the team

Table 13 – Interfaces to agree with the team

| Item | Owner | What this document proposes |
| --- | --- | --- |
| Which 8 of 16 channels reach pads | pad and IO mux owner | ch_0_o and ch_1_o – all four channels of modules 0 and 1, for two independent base frequencies |
| ext_sig_i wiring | pad / IO MUX owner | Four pads already exist (TIM_EXT0–3, PIN_37–40). This block proposes they attach to ext_sig_i[3:0] in pad order; remaining 28 bits tied low. No selection register needed – the IP selects per module |
| dft_cg_enable_i | DFT owner | tie 0 for v1 – no test-mode pin exists, Day005. Revisit if a DFT strategy is adopted; do not record QSOC as having no scan |
| low_speed_clk_i | SCRC | tie to the domain clock; the IP samples it |
| Clock gate bit 15 | SCRC | may start closed; the block does nothing useful until programmed |
| No pad pull fights the rest level | pad owner | Section 5.9 – firmware leaves each channel at a chosen level before the gate closes; a pull in the opposite direction would fight it. One-line check against the pad list |
| Four interrupt sources on one line | INTMAP | already assumed by QNSC_Interrupt_Map_MAS; no change |

## 5.9 Decisions taken

Decided – the eight pad channels are ch_0_o and ch_1_o, i.e. all four channels of module 0 and all four of module 1. In the RTL each module drives its own ch_i_o bus (u_tim0.pwm_o -> ch_0_o, u_tim1.pwm_o -> ch_1_o), so ch_i_o[3:0] is the four channels of module i. Channels within a module share that module’s counter, so two modules at the pads give firmware two independent base frequencies, four channels each. The eight that stay inside (modules 2 and 3) are not wasted: any of the sixteen can still be selected as an event source, section 4.5, or as another module’s trigger, section 5.6.

Decided – the four TIM_EXT pads attach to ext_sig_i[3:0] in pad order. So PIN_37 is bit 0, PIN_38 bit 1, PIN_39 bit 2, PIN_40 bit 3, and the remaining 28 bits are tied low. The reason is legibility rather than function: any four bits work electrically, but a mapping that follows pin order can be read off the pinout without a table, and firmware’s cfg_sel values become guessable instead of memorised.

Decided – centre-aligned output stays available but is not verified exhaustively. The mode is one configuration bit: up_down_counter.sv carries r_direction and r_sawtooth, so cfg_sawtooth_i = 1 gives edge-aligned sawtooth and 0 gives the up-then-down count that produces centre-aligned PWM. Since the capability costs nothing in an IP QSOC does not modify, removing it would be work rather than saving. QSOC’s default is sawtooth, the verification of Table 12 covers that, and centre-aligned is recorded here as available and unverified rather than claimed as supported.

Proposed, and needing the pad owner – the idle level of each PWM pad is low. This is the one question with a consequence outside the chip, so it is stated as a proposal rather than a decision. Two facts frame it:

- Stopping a module freezes its pad. out_filter.sv updates its stored output only while ctrl_active_i is high, so an inactive channel holds whatever level it last drove.

- Gating the domain freezes it too, section 5.3, and for the same reason.

So the pad does not return to a safe level by itself, and “safe” is a board question: for an LED either level is harmless, for a motor driver one of them may mean conducting. The operational rule this document asks for is therefore: stop the module through CMD and let the channel reach its idle level before the clock gate is closed, never gate a running block. Whether idle is low or high per pad is for the pad and board owners to fix before the IO MUX is frozen.

Decided, because the block cannot provide an idle level and firmware must. The earlier wording asked the pad owner to fix an idle level per pad. Reading the RTL shows that is the wrong request: out_filter.sv updates its stored output only while ctrl_active_i is high, so a stopped or gated channel holds whatever it last drove. No level is “the idle level” from the block’s point of view – there is only the last one driven.

So the rule is a firmware procedure, and it is the same for every pad:

| Step | Action |
| --- | --- |
| 1 | Set the channel’s CHn_TH so the output reaches its intended rest level |
| 2 | Let at least one full period elapse, so the level is actually driven |
| 3 | Stop the module through CMD |
| 4 | Only then may SCRC close the domain gate |

Never gate a running block. Doing so freezes the pad at an arbitrary point in the waveform, which for an LED is harmless and for a motor driver may mean conducting.

What the pad owner still needs to confirm is narrower: that none of the eight pads carries a pull-up or pull-down that fights the level firmware leaves behind. That is a one-line check against the pad list, not a design decision, and it is recorded in Table 13 rather than here.

# 6. Observations

The IP’s name is the main hazard in this block. It is called an advanced timer, it contains four timers, and it is not the timer. The previous document for these blocks specified this IP for the timer role, and QSOC_HAS still carries the resulting disagreement. The clearest defence is the one in section 4.2: 16 bits wrap in 3.28 ms, which is ample for a waveform and useless for a timebase.

Two facts in this document are not in any datasheet and change what firmware must do. The four event lines are a multiplexer over the channel outputs rather than four independent comparators, so an interrupt means a chosen channel toggled and firmware must have programmed the choice. And the per-timer status output is connected to nothing, so software cannot ask what happened. Both were found by reading the RTL rather than the documentation, and both would have been discovered late and expensively otherwise.

The absence of a status register is what forces a correction elsewhere. QNSC_Interrupt_Map_MAS argues that losing a pulse is mostly harmless because the sources keep their own record. For GPIO that is true and the register is named. For PWM it is not: the recovery mechanism is that the waveform repeats. The argument survives, but it has to be stated in terms of periodicity, and it then has a visible exception in single-shot use. A specification that had assumed a status register would have been wrong in a way no test would catch.

# Appendix A. Acronyms

Table 14 – Acronyms

| Term | Meaning |
| --- | --- |
| APB | Advanced Peripheral Bus, the AMBA bus used for QSOC’s slow peripherals |
| CDC | Clock Domain Crossing |
| CH_EN | Channel enable register, offset 0x104 |
| DFT | Design For Test |
| Duty cycle | The fraction of one period for which the output is high |
| EVENT_CFG | Event configuration register, offset 0x100 |
| HCLK | The APB clock, named by AMBA convention |
| LUT | Look-Up Table – here the 4-entry table that shapes one channel output |
| MAS | Micro-Architecture Specification |
| mcause | RISC-V CSR reporting the cause of a trap |
| Prescaler | 8-bit divider ahead of a counter, field PRESC |
| PWM | Pulse Width Modulation |
| TH | Threshold – the period register of a module |

# Appendix B. First Review

Points the author expects to be challenged on, with the answer held ready.

- Sixteen channels for eight pads – is half the block wasted? No. Any of the sixteen can be selected as an event source, and the eight that stay inside cost nothing beyond the area already committed by choosing the IP.

- Why not a smaller PWM IP, or one written in house? The alternatives on APB are not comparable: OpenTitan’s PWM is TL-UL and would need a bridge. An in-house block would have to reproduce the per-channel LUT and the up-down mode to match, which is not the small job that INTMAP was.

- Why does the block not interrupt out of reset? Because EVENT_CFG resets to zero, so the event enables are clear. This is desirable and section 5.7 test 6 protects it.

# Appendix C. References

| Claim | Where the evidence is |
| --- | --- |
| Module name, parameters, ports | pulp-platform/apb_adv_timer, rtl/apb_adv_timer.sv |
| TIMER_NBITS = 16, EXTSIG_NUM = 32 | same file, parameter list |
| Four timer_module instances, 16 channel outputs | same file, the four instantiations |
| Events are a 4-of-16 mux over channel outputs | same file: assign s_event_signals = {ch_3_o, ch_2_o, ch_1_o, ch_0_o}; |
| Event pulse is one cycle | same file: events_o[0] = s_event_en[0] & r_event_sync_0[1] & ~r_event_sync_0[0]; |
| status_o is connected to nothing readable | same file: s_timer0_status appears only at its declaration and its connection; rtl/adv_timer_apb_if.sv has no status port |
| Register offsets, module stride 0x40, EVENT_CFG at 0x100, CH_EN at 0x104 | rtl/adv_timer_apb_if.sv, the `define REG_ block |
| 8-bit prescaler | rtl/prescaler.sv: input logic [7:0] cfg_presc_i |
| Port, address, domain D18, gate bit 15 | QSOC_HAS Table 9-1, the APB port table and the clock domain table |
| PWM_0 to PWM_7 pad and pin assignment | QSOC_HAS pad table, PIN_27 to PIN_36 |
| low_speed_clk_i sampled as data, no second domain | QSOC_HAS clock list |
| Four external triggers from GPIO proposed | QSOC_HAS, timer external triggers row |
| Four interrupt sources, all pulses, one fast line | QNSC_Interrupt_Map_MAS, Table 3 |
| Most STM32 timers are 16-bit, a few 32-bit | STMicroelectronics STM32G4 general purpose timer training material |