# SYSCSR

MICRO-ARCHITECTURE SPECIFICATION – V3.2

QUY NHON SEMICONDUCTORS – QNSC

# Table of Contents

# Table of Tables

Table 4-1. Upstream IP used

Table 5-1. SYSCSR interface

Table 6-1. Memory map, regions behind APB_M1

Table 6-2. Register map

Table 6-3. RESET_CAUSE fields

Table 6-4. DOMAIN_RST_STATUS fields, all RO; the bit map is QNSC_SCRC_MAS Table 6-3

Table 6-5. CHIP_ID_REV fields

Table 6-6. DBG_BOOT fields

Table 7-1. Values read in the states firmware and the debugger meet

Table 7-2. Access results

Table 9-1. Functions this block does not provide

Table 10-1. Tie-offs inside m_qnsc_wrap_syscsr

Table 11-1. Requirements on other owners

Table 11-2. HAS text this specification departs from

Table A-1. Acronyms

Table B-1. First review

# Table of Figures

Figure 3-1. SYSCSR in the system: who writes and who reads each register

Figure 6-1. Register fields; shaded bits are reserved and read 0

Figure 7-1. RESET_CAUSE across power-on, a firmware clear and a watchdog bite

Figure 7-2. DBG_BOOT.dbg_en: the DBG_EN pin through two flip-flops, forced to 0 by the mask

# Revision history

The reasoning behind each change, the V1.0–V2.2 history and the V2.2 text are in QNSC_SYSCSR_DECISIONS.md.

| Version | Date | Author | Reviewer | Description of change |
| --- | --- | --- | --- | --- |
| V3.0 | 2026-09-28 | Nghia VT (lead), for Nguyen Hao Nam | – | V2.2 moved onto the MAS template and aligned with QNSC_SCRC_MAS V3.0: APB_M1, Naming Rule wrapper, bit 13 reserved, CHIP_ID from the contract, reset-cause write race closed, access timing and post-boot values stated |
| V3.1 | 2026-09-29 | Nguyen Hao Nam | – | Own block directory design/syscsr, wrapper m_qnsc_wrap_syscsr generated with emacs verilog-mode; CHIP_ID_REV on the new port i_cfg_chip_id, tied by design/top (sections 2, 3, 4, 5, 8, 10) |
| V3.2 | 2026-10-12 | Nguyen Hao Nam | – | Mentor’s debug boot flow of 12/10 (QNSC_BOOT_SPEC V3.3): register DBG_BOOT at 0x0C, dbg_en (RO, the DBG_EN pin through two flip-flops, forced to 0 by the mask) and dbg_en_mask (RW, written by the host over JTAG); port i_dbg_en; sub-module m_qnsc_syscsr_dbg_en; the CPU hold of debug boot no longer appears in DOMAIN_RST_STATUS (sections 1, 2, 3, 5, 6, 7.3, 7.5, 8, 11, 12) |

# 1. Overview

SYSCSR (System Control and Status Registers) is the status half of the clock and reset subsystem. Three status registers answer three questions firmware cannot answer any other way: why the chip last reset (RESET_CAUSE), which domains are in reset now (DOMAIN_RST_STATUS), and which chip this is (CHIP_ID_REV). A fourth, DBG_BOOT, tells the boot ROM whether to wait for the debugger before it starts the application, and lets the debugger end that wait (QNSC_BOOT_SPEC 7.5).

The fact that shapes it: SYSCSR is reset by power-on only, so a watchdog or software reset leaves RESET_CAUSE for the code that runs after it.

SYSCSR controls no clock and no reset; every such control bit is in SCRC (QNSC_SCRC_MAS). Its one writable control bit, DBG_BOOT.dbg_en_mask, only changes what the boot ROM reads.

Block directory design/syscsr, wrapper m_qnsc_wrap_syscsr, owner Nguyen Hao Nam. It is a block of its own, not part of SCRC: its own APB slave, its own clock cluster, one wrapper in one directory.

# 2. Features

- Four 32-bit registers on APB_M1 – 6.

- RESET_CAUSE: one sticky bit per reset source, set by hardware, write 1 to clear – 7.2.

- DOMAIN_RST_STATUS: live reset state of every SCRC domain – 7.3.

- CHIP_ID_REV: the contract constant C_CHIP_ID, ASCII ‘QSOC’ – 6.

- DBG_BOOT: the DBG_EN pin, synchronised, for the boot ROM; a mask the host sets over JTAG to let the ROM go on – 7.5.

- Reset by power-on only, on a clock that is never gated – 7.1.

- Generated by APB-CSR-Generator; one wait state per completed access – 7.4.

# 3. Block diagram

Figure 3-1. SYSCSR in the system: who writes and who reads each register

m_qnsc_wrap_syscsr wraps the generated m_qnsc_syscsr_csr, and is itself generated with emacs verilog-mode (doc/guides/EMACS_AUTO.md). It renames the ports to the Naming Rule and drives the generator’s option and constant inputs (section 10); it adds no logic of its own. The DBG_BOOT.dbg_en bit, the only logic outside the generated block, is the hand-written sub-module m_qnsc_syscsr_dbg_en: two flip-flops on the DBG_EN pin and the mask (7.5).

SYSCSR is in the cpu clock cluster of the contract, hence i_clk_cpu; the clock is SCRC o_clk_pbus, the P_BUS clock.

# 4. IP used

Table 4-1. Upstream IP used

| From | Module | Commit | Licence |
| --- | --- | --- | --- |
| nguyenquanicd/APB-CSR-Generator | generator; output m_qnsc_syscsr_csr | ee80ff54 | MentorProvided-QNSC-Course |

Generator behaviour this specification relies on, read in CSR_Generation.py at that commit, with i_slverr_en = 1, i_protect_en = 0 and the synchronous option:

- PSLVERR = 1 when PADDR[1:0] != 0, or PWRITE = 1 and PSTRB != 4'hF. Such an access changes no register.

- An access is captured in the SETUP cycle and completes with PREADY in the second ACCESS cycle; an access answered with PSLVERR completes in the first.

- A ro field is a live input with no storage; a write to it is ignored.

- A rw field is a flip-flop loaded by an APB write; its value is an output.

- A w1c field: an APB write clears the bits written 1; otherwise i_hw_we loads i_hw_wdata. The APB write wins when both occur in the same cycle – 7.2.

- An offset with no register reads 0 and ignores writes, without error.

Workbook register names are resetcause, domainrststatus, chipidrev, dbgboot; field names are those of Tables 6-3 to 6-6. The workbook and the generated file are kept in util/gen/syscsr/, because the generated port names (i_bus_rstn, i_paddr) do not follow the Naming Rule and design/syscsr/rtl/ holds only code written here.

# 5. Interface

Table 5-1. SYSCSR interface

| Signal | Dir | Width | Connected to, in design/top | Description |
| --- | --- | --- | --- | --- |
| i_clk_cpu | in | 1 | SCRC o_clk_pbus | 20 MHz, never gated |
| i_rst_n_por | in | 1 | SCRC o_rst_n_por | Power-on only. Never o_rst_n_pbus |
| i_bus_apb_psel, i_bus_apb_penable, i_bus_apb_pwrite | in | 1 | P_BUS APB_M1 | APB control. Reached by Ibex and by SYSDBG |
| i_bus_apb_paddr | in | 12 | P_BUS APB_M1 | Offset in the window (contract meta.apb_paddr_width) |
| i_bus_apb_pwdata | in | 32 | P_BUS APB_M1 | Write data |
| i_bus_apb_pstrb | in | 4 | P_BUS APB_M1 | 4'hF on a write, else PSLVERR |
| i_bus_apb_pprot | in | 3 | P_BUS APB_M1 | Not used |
| o_bus_apb_prdata, o_bus_apb_pready, o_bus_apb_pslverr | out | 32, 1, 1 | P_BUS APB_M1 | APB response – 7.4 |
| i_cause_we_wdt, i_cause_we_sw | in | 1 | SCRC o_cause_we_wdt, o_cause_we_sw | 1 for the whole WDT / SW chip reset – 7.2 |
| i_domain_rst_stat | in | 32 | bit n = ~o_rst_n_<d> of SCRC domain n; bits 13, 30:18 tied 0 | 1 = domain n in reset. Reserved bits are not read |
| i_dbg_en | in | 1 | the DBG_EN pad | Asynchronous; synchronised inside, 7.5. 1 = debug boot |
| i_cfg_chip_id | in | 32 | qnsc_pkg::C_CHIP_ID | Value of CHIP_ID_REV. A port, because the wrapper is IP and does not use qnsc_pkg |

# 6. Register map

Table 6-1. Memory map, regions behind APB_M1

| Base | Size | Region | Port | Kind | Note |
| --- | --- | --- | --- | --- | --- |
| 0x80004000 | 16 KiB | syscsr | APB_M1 | peripheral |  |

Only offset bits 11:0 reach SYSCSR, so the map repeats every 4 KiB in the window: 0x8000_5000 is RESET_CAUSE.

Table 6-2. Register map

| Offset | Register | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 0x00 | RESET_CAUSE | W1C | 0x0000_0001 | Sticky cause of the resets since last cleared |
| 0x04 | DOMAIN_RST_STATUS | RO | live | 1 = domain in reset |
| 0x08 | CHIP_ID_REV | RO | 0x5153_4F43 | Chip identifier |
| 0x0C | DBG_BOOT | RO, RW | 0x0000_0000 | Debug boot: the DBG_EN pin for the boot ROM, and its mask |
| 0x10–0xFFF | – | RSVD | 0 | Read 0, writes ignored, no error |

Figure 6-1. Register fields; shaded bits are reserved and read 0

Table 6-3. RESET_CAUSE fields

| Bits | Field | Access | Reset | Set by | Meaning when 1 |
| --- | --- | --- | --- | --- | --- |
| 0 | cause_por | W1C | 1 | power-on, by reset value | A power-on reset occurred |
| 1 | cause_wdt | W1C | 0 | i_cause_we_wdt | A watchdog reset occurred |
| 2 | cause_soft | W1C | 0 | i_cause_we_sw | A software reset (SCRC SW_RST) occurred |
| 31:3 | – | RSVD | 0 | – | – |

Table 6-4. DOMAIN_RST_STATUS fields, all RO; the bit map is QNSC_SCRC_MAS Table 6-3

| Bit | Field | Domain |
| --- | --- | --- |
| 0 | stat_sbus | S_BUS |
| 1 | stat_pbus | P_BUS |
| 2 | stat_wdt | WDT |
| 3 | stat_timer_0 | TIMER0 |
| 4 | stat_timer_1 | TIMER1 |
| 5 | stat_uart_0 | UART0 |
| 6 | stat_uart_1 | UART1 |
| 7 | stat_spi | SPI Host and SPI Device |
| 8 | stat_i2c | I2C |
| 9 | stat_gpio_0 | GPIO0 |
| 10 | stat_dma | DMA |
| 11 | stat_rom | ROM |
| 12 | stat_ram | ISRAM and DSRAM |
| 13 | – | reserved |
| 14 | stat_sysdbg | SYSDBG |
| 15 | stat_pwm | PWM |
| 16 | stat_gpio_1 | GPIO1 |
| 17 | stat_gpio_2 | GPIO2 |
| 30:18 | – | reserved |
| 31 | stat_cpu | CPU |

Table 6-5. CHIP_ID_REV fields

| Bits | Field | Access | Value | Description |
| --- | --- | --- | --- | --- |
| 31:0 | chip_id_rev | RO | 0x5153_4F43 | C_CHIP_ID: ‘Q’, ‘S’, ‘O’, ‘C’, most significant byte first. No revision field |

Table 6-6. DBG_BOOT fields

| Bits | Field | Access | Reset | Description |
| --- | --- | --- | --- | --- |
| 0 | dbg_en | RO | 0 | The DBG_EN pin through two flip-flops, 0 while dbg_en_mask = 1. The boot ROM waits while it is 1 |
| 1 | dbg_en_mask | RW | 0 | Written 1 by the host over JTAG to end the boot ROM’s wait. It can only force dbg_en to 0 |
| 31:2 | – | RSVD | 0 | – |

The mentor’s drawing of 12/10 names these SYSCSR.DEBUG_EN and SYSCSR.DEBUG_EN_MASK; the Naming Rule writes dbg (rule 1.5).

# 7. Functional behaviour

## 7.1 Clock and reset

- SYSCSR runs on o_clk_pbus, which SCRC never gates, including during a chip reset, and is reset by o_rst_n_por, which only power-on asserts.

- After power-on RESET_CAUSE = 0x1.

- A WDT or SW reset changes nothing in SYSCSR except the cause bit it sets.

- No initialisation is needed. SYSCSR is readable as soon as P_BUS and the reading master leave reset.

## 7.2 Reset cause

Figure 7-1. RESET_CAUSE across power-on, a firmware clear and a watchdog bite

- Power-on leaves cause_por = 1 by its reset value.

- Watchdog, software: i_cause_we_wdt or i_cause_we_sw is 1 for the 16 cycles of the chip reset (QNSC_SCRC_MAS 7.3), and sets its bit.

- A bit stays 1 until written 1. Several bits can be 1: each means its source has caused at least one reset since the bit was last cleared.

- Write race. An APB write to RESET_CAUSE is applied at the clock edge that ends the cycle after its SETUP cycle, and wins over a hardware set at that edge. P_BUS enters reset in the first cycle of the chip reset and starts no transfer while in reset, so at most one write lands inside the chip reset, at its first edge. The set enable lasts 16 edges, so the bit is set at the next.

## 7.3 Domain reset status

- DOMAIN_RST_STATUS[n] is i_domain_rst_stat[n], captured when the register is read. It is not stored: every SCRC output is synchronous to o_clk_pbus.

- A bit is 1 while its domain is in reset: during a chip reset, and for timer_1 until its clock first runs. Debug boot holds no domain in reset: the CPU runs the boot ROM while it waits (QNSC_BOOT_SPEC 7.5).

Table 7-1. Values read in the states firmware and the debugger meet

| State | DOMAIN_RST_STATUS | Reason |
| --- | --- | --- |
| After any reset, normal or debug boot | 0x0000_0010 | timer_1 is gated after reset, so still in reset |
| After firmware sets SCRC CLK_EN[4] | 0x0000_0000 | timer_1 clocked and released |

Ibex reads stat_cpu = 0 whenever it can read at all; the bit is for SYSDBG, which can read it during a chip reset.

## 7.4 Bus access

Table 7-2. Access results

| Access | PREADY in | PSLVERR | Effect |
| --- | --- | --- | --- |
| Aligned read | 2nd ACCESS cycle | 0 | Register value; 0 at an offset with no register |
| Aligned write, PSTRB = 4'hF, to RESET_CAUSE | 2nd ACCESS cycle | 0 | Clears the bits written 1 |
| Aligned write, PSTRB = 4'hF, to any other offset | 2nd ACCESS cycle | 0 | None |
| PADDR[1:0] != 0 | 1st ACCESS cycle | 1 | None |
| Write with PSTRB != 4'hF | 1st ACCESS cycle | 1 | None |

A byte or halfword store to SYSCSR is therefore an access fault in Ibex.

## 7.5 Debug boot status

Figure 7-2. DBG_BOOT.dbg_en: the DBG_EN pin through two flip-flops, forced to 0 by the mask

dbg_en = dbg_en_mask ? 0 : DBG_EN pin delayed by two flip-flops

- The DBG_EN pin is set on the board and is asynchronous to i_clk_cpu, so it passes two flip-flops, reset to 0 by i_rst_n_por, before anything reads it. dbg_en follows the pin two clock edges later, rising and falling: it is not latched, so the pin is read whenever the ROM reads DBG_BOOT.

- dbg_en_mask is the select of a mux whose other input is 0. Writing it 1 forces dbg_en to 0 whatever the pin; writing it 0 shows the pin again. No write can set dbg_en to 1.

- The boot ROM reads DBG_BOOT after it has received the frame and polls it while dbg_en = 1 (QNSC_BOOT_SPEC 7.5). The host ends the wait by writing DBG_BOOT = 0x2 through the SYSDBG bus master: AXI_S0, S_BUS, P_BUS.

- Like every SYSCSR flip-flop, dbg_en_mask is reset by power-on only: after a watchdog or software reset it keeps the value the host wrote, so the boot ROM waits again only after a power-on. Confirmed by the mentor on 12/10.

- The two flip-flops are behavioural RTL. design/common/tech has no synchroniser cell yet; when one is added, m_qnsc_syscsr_dbg_en instantiates it.

## 7.6 Firmware use

- At start-up, read RESET_CAUSE, keep the value, and write it back to clear the bits that were set.

- Read CHIP_ID_REV to check the program, or the host through SYSDBG, is on a QSOC.

# 8. Instances

One m_qnsc_wrap_syscsr in design/top, containing one m_qnsc_syscsr_csr and one m_qnsc_syscsr_dbg_en.

# 9. What is not provided here, and who provides it

Table 9-1. Functions this block does not provide

| Function | Where it lives |
| --- | --- |
| Clock gating, SW_RST | SCRC, APB_M0 |
| The reset-cause set enables and the domain resets | SCRC RRC and CTRLs |
| An interrupt on a reset cause | none; firmware reads RESET_CAUSE at start-up |
| Holding the CPU in debug boot | none; the boot ROM waits in software (QNSC_BOOT_SPEC 7.5) |

# 10. Tie-offs

Table 10-1. Tie-offs inside m_qnsc_wrap_syscsr

| Generated port | Tied to | Why |
| --- | --- | --- |
| i_protect_en | 0 | No privilege check; all code runs in machine mode |
| i_slverr_en | 1 | Misaligned and partial-strobe accesses answered with PSLVERR |
| i_hw_we_resetcause_cause_por | 0 | cause_por is set by its reset value |
| i_hw_wdata_resetcause_cause_* | 1 | An enable always sets the bit |
| i_chipidrev_chip_id_rev | the port i_cfg_chip_id, tied by design/top to qnsc_pkg::C_CHIP_ID | One source; hardcode_check rejects the literal |
| i_paddr[15:12] | 0 | P_BUS carries 12 address bits |
| o_resetcause_cause_* | unconnected | Generated for w1c fields; not needed |

# 11. Requirements on others, and open items

Table 11-1. Requirements on other owners

| Item | Owner | What it blocks |
| --- | --- | --- |
| i_rst_n_por = o_rst_n_por | top (lead) | RESET_CAUSE surviving WDT and SW |
| i_domain_rst_stat wired as in section 5 | top (lead) | 7.3 |
| o_cause_we_wdt, o_cause_we_sw held for the whole chip reset; P_BUS reset in its first cycle | SCRC (same owner) | 7.2 |
| The DBG_EN pad wired to i_dbg_en | top (lead) | 7.5 |
| The SYSDBG bus master reaches APB_M1 and writes DBG_BOOT | SYSDBG (lead) | the end of the debug wait, 7.5 |

Table 11-2. HAS text this specification departs from

| QSOC_HAS_Report_EN_v4_final Table 9-1, 9-2 | This specification | Why |
| --- | --- | --- |
| Clock and reset: the peripheral bus domain | reset by power-on only | A WDT or SW reset would clear RESET_CAUSE. The HAS feature list already requires “SYSCSR only reset by Power-on reset” |
| cause_por reset 0, set by a hardware write | reset 1, hardware write tied 0 | The register is held in reset during power-on, so it cannot be written then |
| domainrststatus bits 0–13; one stat_gpio for 4 GPIO; bit 13 spi_device proposed | bits 0–12, 14–17, 31; one bit per GPIO; bit 13 reserved | QNSC_SCRC_MAS domain map: 3 GPIO, one SPI domain, SYSDBG, PWM and CPU added |

Open items: none.

# 12. Verification

- SYSCSR_001 After power-on: RESET_CAUSE = 0x1, CHIP_ID_REV = 0x5153_4F43.

- SYSCSR_002 A watchdog reset sets cause_wdt and leaves the other bits unchanged.

- SYSCSR_003 A software reset sets cause_soft and leaves the other bits unchanged.

- SYSCSR_004 Writing 1 to a RESET_CAUSE bit clears only that bit; writing 0 changes nothing.

- SYSCSR_005 DOMAIN_RST_STATUS[n] = ~o_rst_n_<d> for the 18 implemented bits, in every cycle a read samples it; reserved bits read 0.

- SYSCSR_006 Writes to DOMAIN_RST_STATUS, CHIP_ID_REV and offsets with no register change nothing and complete without PSLVERR.

- SYSCSR_007 Each row of Table 7-2 gives the stated PREADY cycle, PSLVERR and effect.

- SYSCSR_008 A W1C write of 0x7 whose SETUP cycle is the last before a watchdog chip reset leaves cause_wdt = 1 after the reset.

- SYSCSR_009 Table 7-1 values after a normal boot and a debug boot (SoC test).

- SYSCSR_010 Every SYSCSR flip-flop is reset by i_rst_n_por only (netlist check).

- SYSCSR_011 With dbg_en_mask = 0, DBG_BOOT.dbg_en follows the DBG_EN pin two clock edges later, rising and falling.

- SYSCSR_012 With dbg_en_mask = 1, dbg_en reads 0 whatever the pin; writing dbg_en_mask = 0 shows the pin again.

- SYSCSR_013 A write to DBG_BOOT cannot set dbg_en, and bits 31:2 read 0.

# Appendix A. Acronyms

Table A-1. Acronyms

| Acronym | Description |
| --- | --- |
| POR | Power-On Reset |
| RRC | Reset Request Controller, in SCRC |
| SCRC | System Clock and Reset Controller |
| SYSCSR | System Control and Status Registers |
| W1C | Write 1 to clear |
| WDT | Watchdog Timer |

# Appendix B. First review

Table B-1. First review

| Item | Reviewer | Response |
| --- | --- | --- |
| CHIP_ID_REV misspelled ‘QCOS’; separate REV field | Sinh | V1.1; now C_CHIP_ID in the contract, 6 |
| MRV_BUSY premature | Sinh | Removed in V1.1 |
| RESET_CAUSE must survive WDT/SW | Nam, V2.0 | POR-only reset, 7.1 |
| SYSDBG reads SYSCSR while it holds the CPU | SYSDBG MAS V3.0 | stat_cpu, 7.3 |
| APB_S1 against contract APB_M1 | lead, V3.0 | 5, 6 |
| spi_device and GPIO3 domains removed | lead, V3.0 | bits 13 and 18 reserved |
| One-cycle cause pulse lost against a W1C write in the same cycle | lead, V3.0 | set held for the whole reset, 7.2, SYSCSR_008 |
| Generated names fail the Naming Rule | lead, V3.0 | wrapper m_qnsc_syscsr; generated file in util/gen/syscsr, 4 |
| Post-boot DOMAIN_RST_STATUS not stated | lead, V3.0 | 7.3 |
| SYSCSR.DEBUG_EN status bit, from the pin through two flip-flops, and DEBUG_EN_MASK written over JTAG to force it to 0 | Quan (mentor), 12/10 drawing | DBG_BOOT, 6, 7.5 |
| Only power-on resets the debug part | Quan (mentor), 12/10, confirmed | 7.5: dbg_en_mask and the synchroniser on i_rst_n_por |