# RAM

MICRO-ARCHITECTURE SPECIFICATION – V2.5

QUY NHON SEMICONDUCTORS – QNSC

# Table of Contents

# Table of Tables

Table 1-1. Memory map, regions behind AXI_M1 and AXI_M2

Table 4-1. Upstream IP used

Table 5-1. RAM interface

Table 5-2. IP parameters, fixed inside each wrapper

Table 7-1. FIFO buffers

Table 7-2. Timing at the block ports, no contention, FIFOs empty

Table 7-3. Burst address generation

Table 8-1. The two blocks

Table 9-1. Functions this block does not provide

Table 10-1. Tie-offs

Table 11-1. Requirements on other owners

Table A-1. Acronyms

Table B-1. First review

# Table of Figures

Figure 3-1. The RAM block: wrapper, unmodified controller and macro. Heavy outline: the strobe FIFO QSOC adds

# Revision history

The reasoning behind each change is in QNSC_RAM_DECISIONS.md.

| Version | Date | Author | Reviewer | Description of change |
| --- | --- | --- | --- | --- |
| V2.0 | 2026-09-23 | Nghia VT | – | Rewritten as specification only; memory map generated from the contract |
| V2.1 | 2026-09-24 | Nghia VT | – | ISRAM contents survive every reset except power loss. PARA_ID_WD = 7, from the HAS |
| V2.2 | 2026-09-24 | Nghia VT | – | Wrapper interface, strobe FIFO, timing, reset and tie-offs specified; figure and macro address corrected |
| V2.3 | 2026-09-25 | Nghia VT | – | Port names to QNSC_RTL_Design_Naming_Rule V1.0: AXI named per channel (i_bus_axi_aw_valid, …), macro port o_mem_* / i_mem_rdata (was o_sram_*, o_sram_bwe is o_mem_be). No change in behaviour |
| V2.4 | 2026-09-28 | Nghia VT | – | Two blocks, two wrappers, per Naming Rule V1.1 rule 2.1: m_qnsc_wrap_isram in design/isram, m_qnsc_wrap_dsram in design/dsram (was one m_qnsc_wrap_axi4_sram in design/ram). No change in behaviour |
| V2.5 | 2026-10-15 | Nghia VT | – | Wrappers written and simulated. PARA_LEN_WD is 9, not 8: with 8, a 256-beat burst (AxLEN = 255) is dropped with no response (found in simulation, 7.2). Section 12 carries test IDs RAM_001 to RAM_012 and says which items are not simulated; the timing table of 7.1 is confirmed as written |

# 1. Overview

ISRAM and DSRAM are the on-chip RAMs of QSOC. They are two blocks with one design: m_qnsc_wrap_isram and m_qnsc_wrap_dsram, each the upstream AXI4-SRAM-CONTROLLER, unmodified, plus a write-strobe FIFO, in front of an SMIC 28 nm single-port SRAM macro. The block does not decode addresses, never returns an error response, and has no registers.

Table 1-1. Memory map, regions behind AXI_M1 and AXI_M2

| Base | Size | Region | Port | Kind | Note |
| --- | --- | --- | --- | --- | --- |
| 0x20000000 | 4 KiB | isram_dbg | AXI_M1 | memory | The debug window: the first 4 KiB of ISRAM, not a block of its own |
| 0x20001000 | 60 KiB | isram | AXI_M1 | memory | The downloaded application |
| 0x30000000 | 32 KiB | dsram | AXI_M2 | memory | Stack, heap and variables |

isram_dbg and isram are one macro on one port; the block sees one 64 KiB region.

Files design/isram/rtl/m_qnsc_wrap_isram.sv and design/dsram/rtl/m_qnsc_wrap_dsram.sv, owner Nghia Van Trong.

# 2. Features

- AXI4 subordinate, 32-bit data and address, 7-bit ID, on AXI_M1 or AXI_M2.

- Bursts INCR of 1 to 256 beats and FIXED of 1 to 16 (the AXI4 limit); responses in order – 7.1, 7.2.

- Byte, halfword and word writes through WSTRB – 7.3.

- Eight-entry FIFO on every channel; one SRAM read in flight – 7.1.

- 64 KiB ISRAM and 32 KiB DSRAM, set by PARA_SRAM_DEPTH – section 8.

- Macro array not reset: contents survive watchdog and software reset – 7.6.

# 3. Block diagram

Figure 3-1. The RAM block: wrapper, unmodified controller and macro. Heavy outline: the strobe FIFO QSOC adds

Clock i_clk_mem and reset i_rst_n_mem come from the mem cluster. The clock is never gated; the reset is asserted by power-on, watchdog and software reset.

# 4. IP used

Table 4-1. Upstream IP used

| From | Module | Commit | Licence |
| --- | --- | --- | --- |
| nguyenquanicd/AXI4-SRAM-CONTROLLER | m_vlsi_axi4_sram and four sub-modules | 503d7cd | MentorProvided-QNSC-Course |
| SMIC 28 nm | single-port SRAM macro | PDK | foundry |

# 5. Interface

Table 5-1. RAM interface

| Signal | Dir | Width | Description |
| --- | --- | --- | --- |
| i_clk_mem | in | 1 | mem cluster clock, 20 MHz |
| i_rst_n_mem | in | 1 | Asynchronous active-low reset, 7.6 |
| i_bus_axi_aw_valid, o_bus_axi_aw_ready | in, out | 1 | AW handshake. awready = address FSM idle and AWFIFO not full |
| i_bus_axi_aw_addr, i_bus_axi_ar_addr | in | 32 | Byte address |
| i_bus_axi_aw_len, i_bus_axi_ar_len | in | 8 | Beats minus one |
| i_bus_axi_aw_burst, i_bus_axi_ar_burst | in | 2 | 00 FIXED, 01 INCR; 10 and 11 see 7.4 |
| i_bus_axi_aw_id, i_bus_axi_ar_id | in | 7 | Returned on bid, rid |
| i_bus_axi_w_valid, o_bus_axi_w_ready | in, out | 1 | W handshake. wready = WFIFO not full |
| i_bus_axi_w_data | in | 32 | Write data |
| i_bus_axi_w_strb | in | 4 | Byte lane enables, 7.3 |
| i_bus_axi_w_last | in | 1 | Connected to the IP, not used, section 10 |
| o_bus_axi_b_valid, i_bus_axi_b_ready | out, in | 1 | B handshake. bvalid = BFIFO not empty |
| i_bus_axi_ar_valid, o_bus_axi_ar_ready | in, out | 1 | AR handshake. arready = address FSM idle and ARFIFO not full |
| o_bus_axi_r_valid, i_bus_axi_r_ready | out, in | 1 | R handshake. rvalid = RFIFO not empty |
| o_bus_axi_r_data | out | 32 | Read data |
| o_bus_axi_b_id, o_bus_axi_r_id | out | 7 | ID of the request |
| o_bus_axi_b_resp, o_bus_axi_r_resp | out | 2 | Always 00, OKAY |
| o_bus_axi_r_last | out | 1 | Last beat of the burst |
| o_mem_addr | out | 14 / 13 | Word address, IP byte address [15:2] (ISRAM) or [14:2] (DSRAM) |
| o_mem_wdata | out | 32 | Write data |
| o_mem_we | out | 1 | High for one cycle per write beat |
| o_mem_be | out | 4 | Byte write enables, active high; bit n writes wdata[8n+7:8n]. Zero when o_mem_we = 0 |
| o_mem_oe | out | 1 | High in the read-issue cycle |
| i_mem_rdata | in | 32 | Read data, sampled in the cycle after o_mem_oe |

Table 5-2. IP parameters, fixed inside each wrapper

| Parameter | ISRAM | DSRAM | Meaning |
| --- | --- | --- | --- |
| PARA_DATA_WD | 32 | 32 | Data width in bits |
| PARA_ADDR_WD | 32 | 32 | AXI address width |
| PARA_ID_WD | 7 | 7 | AXI ID width, the S_BUS master-port ID width of HAS Table 5-1 |
| PARA_LEN_WD | 9 | 9 | Width of the controller’s beat counter, which holds AxLEN + 1: 9 bits, bit 8 of AxLEN tied 0. At 8, a 256-beat burst is dropped – 7.2 |
| PARA_FIFO_DEPTH | 8 | 8 | Depth of all six FIFOs, a power of two |
| PARA_SRAM_DEPTH | 16384 | 8192 | Macro depth in words; sets the o_mem_addr width to $clog2(PARA_SRAM_DEPTH) |

# 6. Register map

None. Configuration is the parameters of section 5, fixed at elaboration.

# 7. Functional behaviour

## 7.1 FIFOs, arbitration and timing

Table 7-1. FIFO buffers

| FIFO | Width | Contents | Push | Pop |
| --- | --- | --- | --- | --- |
| AWFIFO | ADDR + ID + 1 | address, ID, last flag | write FSM, one per beat | write grant |
| WFIFO | DATA | write data | wvalid & wready | write grant |
| STRBFIFO | 4 | wstrb | wvalid & wready | o_mem_we |
| ARFIFO | ADDR + ID + 1 | address, ID, last flag | read FSM, one per beat | read issue |
| RFIFO | DATA + ID + 2 + 1 | data, ID, RRESP, last flag | cycle after read issue | rvalid & rready |
| BFIFO | ID + 2 | ID, BRESP | write grant of a last beat | bvalid & bready |

A write is granted when AWFIFO and WFIFO are both non-empty; one entry is popped from each and written to the macro in the same cycle. When a write and a read request in the same cycle, the arbiter’s round-robin toggle picks one; the toggle flips after every grant. A lone request is granted in the cycle it appears.

A read is issued when ARFIFO is non-empty, RFIFO is not full and no read is pending. At most one SRAM read is in flight: the address is presented in one cycle and the data is pushed into RFIFO in the next.

B and R responses return in the order their addresses were accepted, each with the ID of its request.

Table 7-2. Timing at the block ports, no contention, FIFOs empty

| Quantity | Value |
| --- | --- |
| rvalid after the AR handshake cycle, single beat | 4 cycles |
| bvalid after the AW handshake cycle, W accepted in the same cycle | 3 cycles |
| Read burst | 1 beat per 2 cycles |
| Write burst | 1 beat per cycle |
| Address handshakes on one channel, N-beat bursts | 1 per N + 2 cycles |

## 7.2 Burst address generation

Table 7-3. Burst address generation

| AxBURST | Next address |
| --- | --- |
| 00 FIXED | unchanged |
| 01 INCR | addr + 4 |
| 10 WRAP, 11 reserved | (addr + 4) & ~3 – 7.4 |

The first beat uses AxADDR as given; the increment is 4 bytes whatever AxSIZE. The last beat is flagged by a counter loaded from AxLEN; WLAST is not used.

The counter is loaded with AxLEN + 1 and is PARA_LEN_WD bits wide. With the IP’s default of 8 bits, AxLEN = 255 loads 0: the burst is finished before it starts, no beat reaches the FIFOs and no response comes back, and a write leaves the write-data FIFO full. The wrappers therefore set PARA_LEN_WD = 9 and tie bit 8 of AxLEN to 0, which the controller accepts unmodified. Found in simulation (RAM_012).

The macro word address is IP address bits [$clog2(PARA_SRAM_DEPTH)+1:2]. Bits [1:0] never reach the macro, so a single-beat access at an unaligned address reads the containing word, or writes the bytes WSTRB selects in it.

## 7.3 Byte-enable path

The IP has no WSTRB input. The wrapper adds u_strbfifo, an m_vlsi_fifo 4 bits wide with PARA_DEPTH = $clog2(PARA_FIFO_DEPTH), so it holds PARA_FIFO_DEPTH entries, the same as WFIFO:

- Push i_bus_axi_w_strb on i_bus_axi_w_valid & o_bus_axi_w_ready, the WFIFO push.

- Pop on o_mem_we, which is the WFIFO pop.

- Output o_mem_be = o_mem_we ? head : 4'b0000.

STRBFIFO and WFIFO hold the same number of entries in every cycle, so the head of STRBFIFO is the strobe of the beat being written. A beat with wstrb = 4'b0000 writes no byte and still completes.

## 7.4 WRAP

AxBURST = 2'b10 or 2'b11 is accepted. addr_wrap in m_vlsi_axfsm.sv computes (addr + 4) & ~3, which aligns the address and does not wrap it. For a 4-byte-aligned start address a WRAP burst produces the same addresses as INCR.

## 7.5 Error handling

BRESP and RRESP are always 2'b00, OKAY. The block never returns SLVERR or DECERR. Address bits above the macro word address are ignored.

## 7.6 Reset

i_rst_n_mem (asserted asynchronously, released synchronously by SCRC) resets every flip-flop in the controller and in u_strbfifo: address FSMs to S_IDLE, FIFO pointers and storage to zero, the arbiter toggle and the read-pending flag to zero. After reset awready, arready and wready are 1, and bvalid and rvalid are 0. A transaction in flight when reset asserts is dropped without a response.

The macro array has no reset and no clear-on-reset: its contents are unchanged by any assertion of i_rst_n_mem while power is applied.

# 8. Instances

Table 8-1. The two blocks

| Block | Port | PARA_SRAM_DEPTH | o_mem_addr |
| --- | --- | --- | --- |
| ISRAM | AXI_M1 | 16384 | [15:2] |
| DSRAM | AXI_M2 | 8192 | [14:2] |

The two wrappers differ only in PARA_SRAM_DEPTH; neither has a parameter. One of each in design/top. Each decode window (section 1) equals its macro size, so every routed address selects exactly one word.

# 9. What is not provided here, and who provides it

Table 9-1. Functions this block does not provide

| Function | Where it lives |
| --- | --- |
| Address decode, DECERR for unmapped addresses | S_BUS |
| WRAP bursts | Nowhere; masters must not issue them – 7.4 |
| Narrow bursts (AxSIZE below 4 bytes with AxLEN above 0) | Nowhere – 7.2 |
| Exclusive and locked access | Nowhere |
| Error detection, SECDED | Nowhere |
| Macro margin, retention and test pins | Physical design – section 10 |
| ISRAM contents | ROM bootloader, or the host through SYSDBG |

# 10. Tie-offs

Table 10-1. Tie-offs

| Port | Tied to | Why |
| --- | --- | --- |
| S_BUS awsize, arsize | open | The IP has no AxSIZE; the increment is 4 bytes |
| S_BUS awprot, arprot, awcache, arcache, awlock, arlock, awqos, arqos | open | No protection, cache, exclusive or QoS function |
| IP i_wlast | i_bus_axi_w_last, unused inside | The last beat comes from AxLEN |
| IP BRESP, RRESP source | 2'b00 inside the IP | No error source; firmware sees OKAY for every access |
| Macro margin pins | databook default | Margin at default |
| Macro retention pins | inactive | Retention off |
| Macro test and BIST pins | inactive | Test off |

# 11. Requirements on others, and open items

Table 11-1. Requirements on other owners

| Item | Owner | What it blocks |
| --- | --- | --- |
| CPU2AXI carries the Ibex byte enables to wstrb | bus owner | Every sub-word CPU store – 7.3 |
| DECERR for unmapped addresses; decode windows as Table 1-1 | bus owner | Out-of-range accesses – 7.5 |
| S_BUS master-port ID width 7, equal to PARA_ID_WD | bus owner | Response ID matching |
| Does the DMA issue WRAP or narrow bursts into RAM? If yes, axi_burst_unwrap goes in front of this block | DMA owner | DMA transfers – 7.2, 7.4 |
| First 4 KiB of ISRAM reserved, main image linked at 0x20001000 | firmware owner | The SYSDBG debug window |
| Macro pins tied as section 10 | physical design | Silicon function |

Open: timing and area of both instances after synthesis.

# 12. Verification

The IP’s regression runs unchanged against the controller alone: nguyenquanicd/AXI4-SRAM-CONTROLLER at 503d7cd98352378c09dff3930f7fdb0590ea643d, 46 directed tests in sim/vcs/tests/ on the Synopsys AXI VIP, self-checking in sim/vcs/env/axi_svt_basic_env.sv. Its macro model sim/vcs/env/m_vlsi_sram_sp.sv has no byte enables and resets its array; the checks below use a model with byte enables and no reset.

Checks QSOC adds. dv/isram and dv/dsram run the same tests, in dv/ram_common, against a macro model with byte enables and no reset; make sim BLOCK=isram and make sim BLOCK=dsram. Both blocks pass all of them.

- RAM_001 Each of the 15 non-zero wstrb patterns writes only its bytes; 0000 none – 7.3.

- RAM_002 Assertion: STRBFIFO and WFIFO occupancy are equal in every cycle – 7.3. The assertion runs in every cycle of every test; the test fills the FIFOs on purpose.

- A C program writing uint8_t and uint16_t data leaves neighbours intact – 7.3. Not simulated: it needs the CPU and CPU2AXI. RAM_001 covers the same bytes at the port.

- RAM_004 Every value of the timing table of 7.1, measured at the block ports.

- RAM_005 B and R return in acceptance order, each with its request’s ID – 7.1.

- RAM_006 Against an independent address model, aligned WRAP gives INCR addresses – 7.4.

- RAM_007 o_mem_addr is [15:2] / [14:2]; an unaligned single beat hits its word – 7.2.

- RAM_008 BRESP and RRESP are 00 for every access – 7.5.

- RAM_009 Reset values of 7.6; a pattern survives an i_rst_n_mem pulse – 7.6.

- Macro pin tie-offs match the databook – section 10. Not simulated: the testbench has a behavioural model, not the macro. It belongs to physical design.

- The vendored rtl/ files match commit 503d7cd – section 4. Checked by CI (Vendor tree unmodified), not by simulation.

- RAM_012 (added) INCR bursts of 1, 2, 3, 16, 17 and 256 beats, FIXED of 1 to 16, and 300 random bursts against a reference memory with B and R stalled at random – 7.2.

# Appendix A. Acronyms

Table A-1. Acronyms

| Acronym | Description |
| --- | --- |
| BWE | Byte write enable |
| FIFO | First-In, First-Out buffer |
| DECERR | AXI decode error response |
| DSRAM | Data RAM, 32 KiB on AXI_M2 |
| ISRAM | Instruction RAM, 64 KiB on AXI_M1 |
| SECDED | Single Error Correct, Double Error Detect |
| SLVERR | AXI slave error response |
| WSTRB | AXI write strobes, the byte enables |

# Appendix B. First review

Table B-1. First review

| Item | Reviewer | Response |
| --- | --- | --- |
| Too long, and the redundancy causes wrong information | Teacher, 2026-09-23 | Rewritten as specification only; the memory map is generated from util/qsoc_contract.yml |
| Is the missing WSTRB a defect in the IP? | – | No. The IP’s README states it as a boundary. The wrapper adds it – 7.3 |
| Do the four passing wrap_* tests prove WRAP works? | – | No. The IP’s reference model computes the same address – 7.4 |
| Is ROM the same block? | Teacher | No. Separate block design/rom, same controller with the write channel idle |