# TIMER

MICRO-ARCHITECTURE SPECIFICATION – V2.0

QUY NHON SEMICONDUCTORS – QNSC

# Table of Contents

# Table of Tables

Table 1-1. Instance assignment

Table 3-1. TIMER sub-modules

Table 4-1. Upstream IP used

Table 4-2. Why the PWM IP cannot serve as the timer

Table 5-1. Block interface

Table 6-1. Register map of one instance

Table 6-2. CFG_REG_LO and CFG_REG_HI bit fields

Table 7-1. Counting rate options

Table 7-2. Interrupt shape by mode

Table 7-3. Address decode against region size

Table 9-1. Functions this block does not provide

Table 10-1. Constraints on firmware and integration

Table 11-1. Requirements on other owners

Table 12-1. Verification QSOC must add

Table A-1. Acronyms

Table B-1. First review

# Table of Figures

Figure 3-1. The timer block and its two counters

# Revision history

V2.0 is a rewrite. The reasoning behind each decision, and how every open question closed, is in QNSC_TIMER_DECISIONS.md.

| Version | Date | Author | Reviewer | Description of change |
| --- | --- | --- | --- | --- |
| V2.0 | 2026-09-23 | Nghia VT | – | Rewritten as specification only, onto the template the other blocks follow. Tables and figures are numbered by the build |

# 1. Overview

TIMER0 and TIMER1 are the two general-purpose timers of QSOC, each an integration of pulp-platform/apb_timer_unit behind an APB4 slave. One instance holds two 32-bit counters that can be chained into one 64-bit counter.

QSOC has no CLINT, so there is no mtime. TIMER0 is the chip’s timebase, and firmware reaches it through these registers rather than through the RISC-V standard machine timer – section 11.

Block directory design/timer, module m_qnsc_wrap_apb_timer_unit, owner Nghia Van Trong.

Table 1-1. Instance assignment

| Instance | Region | APB port | Mode | Interrupt lines |
| --- | --- | --- | --- | --- |
| TIMER0 | 0x8001C000 | APB_M7 | 64-bit, both counters chained | 1 – irq_lo_o only |
| TIMER1 | 0x80020000 | APB_M8 | two independent 32-bit | 2 – irq_lo_o, irq_hi_o |

# 2. Features

- Two 32-bit counters per instance, chainable into one 64-bit counter.

- 8-bit prescaler ahead of each counter, and an optional reference-clock input sampled as data.

- Periodic or one-shot at the compare, selected per counter.

- Hardware start from an event input, with no CPU action.

- No status register and no interrupt flag – section 6, and the fact QNSC_Interrupt_Map_MAS depends on.

# 3. Block diagram

Figure 3-1. The timer block and its two counters

Table 3-1. TIMER sub-modules

| Sub-module | Count | Function |
| --- | --- | --- |
| timer_unit_counter | 2 | the 32-bit counters, lo and hi |
| timer_unit_counter_presc | 2 | 8-bit prescaler ahead of each counter |
| APB register file | 1 | ten addresses, section 6 |
| Interrupt logic | 1 | combinational, section 7.3 |

In 64-bit mode the two counters are chained, and the RTL raises the carry explicitly:

s_enable_count_hi = ( s_timer_val_lo == 32'hFFFFFFFF );

so counter_hi advances exactly once per wrap of counter_lo – a true 64-bit count, not two loosely coupled halves.

# 4. IP used

Table 4-1. Upstream IP used

| From | Module | Commit | Licence |
| --- | --- | --- | --- |
| pulp-platform/apb_timer_unit | apb_timer_unit and three sub-modules | pinned in vendor/manifest.yml | SolderPad 0.51 |

Why not the PWM IP, which QSOC also instantiates. An earlier revision specified apb_adv_timer for this role; it cannot perform it, and the reason is arithmetic:

Table 4-2. Why the PWM IP cannot serve as the timer

|  | apb_adv_timer (PWM) | apb_timer_unit (this block) |
| --- | --- | --- |
| Counter width | 16-bit | 32-bit, or 64-bit combined |
| Wrap at 20 MHz, no prescaler | 3.2768 ms | 214.7 s, or ~29 000 years at 64-bit |
| Channel output pins | 16 | none |
| Suited to | short repeating waveforms | a timebase that must not wrap |

A timebase wrapping every 3.28 ms forces firmware to count wraps continuously, and one missed wrap corrupts every later measurement. A PWM block never needs a long count. The two roles want opposite counter widths, which is why QSOC instantiates both IPs.

# 5. Interface

Table 5-1. Block interface

| Signal | Dir | Width | Note |
| --- | --- | --- | --- |
| HCLK | in | 1 | peri cluster clock – section 7.5 |
| HRESETn | in | 1 | active low |
| PADDR | in | 12 | only PADDR[5:0] is decoded – section 7.6 |
| PWDATA | in | 32 |  |
| PWRITE, PSEL, PENABLE | in | 1 each |  |
| PRDATA | out | 32 |  |
| PREADY | out | 1 | tied to PSEL & PENABLE inside the IP |
| PSLVERR | out | 1 | tied 0 inside the IP |
| ref_clk_i | in | 1 | sampled as data, not used as a clock |
| event_lo_i, event_hi_i | in | 1 each | start a counter without CPU action |
| irq_lo_o, irq_hi_o | out | 1 each | to INTMAP |
| busy_o | out | 1 | either counter enabled |

PREADY tied and PSLVERR tied pull in opposite directions:

- The block can never stall P_BUS. No access can hang the bus, which removes it from the list of blocks a watchdog must protect against.

- The block can never report an error. A write to an unimplemented offset is accepted silently and a read returns zero, so firmware gets no signal that it addressed the block wrongly.

# 6. Register map

Ten addresses per instance, at the region base.

Table 6-1. Register map of one instance

| Offset | Name | Access | Function |
| --- | --- | --- | --- |
| 0x00 | CFG_REG_LO | RW | configuration of lo; also holds the 64-bit mode bit |
| 0x04 | CFG_REG_HI | RW | configuration of hi |
| 0x08 | TIMER_VAL_LO | RW | current count of lo; a write loads it |
| 0x0C | TIMER_VAL_HI | RW | current count of hi; a write loads it |
| 0x10 | TIMER_CMP_LO | RW | compare target for lo |
| 0x14 | TIMER_CMP_HI | RW | compare target for hi |
| 0x18 | TIMER_START_LO | WO | any write starts lo |
| 0x1C | TIMER_START_HI | WO | any write starts hi |
| 0x20 | TIMER_RESET_LO | WO | any write clears lo |
| 0x24 | TIMER_RESET_HI | WO | any write clears hi |

Reads return only six of the ten – the two CFG, the two VAL, the two CMP. The four command addresses are write-only and read as zero.

There is no status register and no interrupt flag. This is the fact section 11 and QNSC_Interrupt_Map_MAS both depend on: nothing in this block records that an interrupt happened, so nothing here can acknowledge one either.

Table 6-2. CFG_REG_LO and CFG_REG_HI bit fields

| Bit | Name | Function |
| --- | --- | --- |
| 0 | ENABLE | counter runs |
| 1 | RESET | clears the counter; self-clearing |
| 2 | IRQ_EN | allows the compare to raise the interrupt |
| 3 | IEM | lets event_*_i set ENABLE |
| 4 | CMP_CLR | on compare, clear the counter – this is what makes it periodic |
| 5 | ONE_SHOT | on compare, clear ENABLE |
| 6 | PRESC_EN | insert the prescaler |
| 7 | REF_CLK_EN | count ref_clk_i edges instead of every core clock |
| 15:8 | PRESC | 8-bit prescaler divisor |
| 31 | MODE_64 | CFG_REG_LO only: chain both counters into one 64-bit timer |

MODE_64 is read from CFG_REG_LO everywhere the RTL uses it, including the paths controlling the hi counter. Writing bit 31 of CFG_REG_HI has no effect and must not be relied on.

# 7. Functional behaviour

## 7.1 Counting rate

Table 7-1. Counting rate options

| REF_CLK_EN | PRESC_EN | Counter advances | Wrap of 32 bits at 20 MHz |
| --- | --- | --- | --- |
| 0 | 0 | every core clock | 214.7 s |
| 0 | 1 | every PRESC+1 core clocks | up to 256 x that |
| 1 | 0 | on each rising edge of ref_clk_i | set by ref_clk_i |
| 1 | 1 | prescaled ref_clk_i edges | set by both |

ref_clk_i passes through a four-stage shift register and an edge detector inside the IP. It is sampled as data, never used as a clock, so it creates no second clock domain and needs no CDC constraint.

## 7.2 One counting tick is the interrupt pulse width

One counting tick is one HCLK cycle with no prescaler and no reference clock, PRESC+1 cycles with the prescaler, or one ref_clk_i period with REF_CLK_EN set. The narrowest case is one core clock, and that is the case INTMAP must assume.

## 7.3 The interrupt is combinational, and 64-bit mode drives only one line

if ( s_cfg_lo_reg[`MODE_64_BIT] == 1'b0 ) begin       // 32-bit mode
   irq_lo_o = s_target_reached_lo & s_cfg_lo_reg[`IRQ_BIT];
   irq_hi_o = s_target_reached_hi & s_cfg_hi_reg[`IRQ_BIT];
end else begin                                        // 64-bit mode
   irq_lo_o = s_target_reached_lo & s_target_reached_hi & s_cfg_lo_reg[`IRQ_BIT];
end

In 64-bit mode irq_hi_o is never driven, so the instance contributes one source, not two. That is why TIMER0 contributes one line and TIMER1 two.

The 64-bit compare is correct, and it is worth showing why, because the expression looks as though two unrelated events must coincide. target_reached_hi is high for the whole epoch in which counter_hi equals its target, and that epoch lasts one full wrap of counter_lo. Within it, counter_lo passes through every value exactly once, so it equals its own target for exactly one tick. The AND therefore produces exactly one pulse, at the intended 64-bit value.

## 7.4 The compare is equality, not greater-or-equal

target_reached is asserted when the counter equals the compare value. The standard RISC-V machine timer is specified the other way – pending while mtime >= mtimecmp – specifically so that a target already passed still fires.

This block has no such protection, and the failure is silent:

| Mode | If firmware writes a target the counter has just passed |
| --- | --- |
| 32-bit at 20 MHz | next hit after a full wrap, 214.7 s late |
| 64-bit at 20 MHz | next hit after a 64-bit wrap, ~29 000 years – never |

Firmware must compute the next target from the current count with enough margin to cover its own interrupt latency, and must not write a target derived from a count it read earlier. In 64-bit mode this is a correctness matter, not a performance one. Section 12 turns it into a test.

## 7.5 One-shot mode holds the interrupt as a level

The comparator is a flop not gated by the counter enable:

// timer_unit_counter.sv -- COMPARATOR
always_ff@(posedge clk_i, negedge rst_ni)
   if ( s_count == compare_value_i ) target_reached_o <= 1'b1;
   else                              target_reached_o <= 1'b0;

and when the counter is stopped, s_count holds its value. So a mode that stops the counter on the compare leaves s_count == compare_value_i true indefinitely, and the interrupt is held rather than pulsed.

Table 7-2. Interrupt shape by mode

| CMP_CLR | ONE_SHOT | What happens at the target | Shape | Width |
| --- | --- | --- | --- | --- |
| 1 | 0 | counter clears and runs on – periodic | pulse | 1 counting tick |
| 0 | 0 | counter runs past the target | pulse | 1 tick; next hit after a full wrap |
| x | 1 | ENABLE cleared, counter stops on the target | held level | until firmware clears or restarts it |

The held level is useful rather than awkward. One-shot is exactly the mode in which “the event repeats next period” is not available as a recovery mechanism. Because the IP holds the line, a one-shot alarm missed while mstatus.MIE was clear is still pending when interrupts are re-enabled. QNSC_Interrupt_Map_MAS classifies the two modes separately for this reason.

Firmware must clear a one-shot interrupt by acting on the block – writing TIMER_RESET_*, moving TIMER_CMP_*, or clearing IRQ_EN. Returning from the handler without doing so re-enters it immediately.

## 7.6 The register file aliases 256 times across its region

Table 7-3. Address decode against region size

| Item | Value |
| --- | --- |
| Registers implemented | 10 addresses, 0x00 to 0x24 |
| Address bits decoded | PADDR[5:0] |
| Span actually decoded | 64 bytes |
| Region assigned | 16 KiB |
| Consequence | the 64 bytes alias 256 times across the region |

So 0x80020000 and 0x80020040 are the same register. Harmless if firmware uses the base address, and a trap if anyone assumes an access above the first 64 bytes will fault – it will not, PSLVERR being tied low.

The recommendation is to accept the aliasing and document it, rather than narrow the decode in the wrapper: narrowing costs logic and buys only a fault firmware has no way to observe.

# 8. Instances

Two, differing in the mode firmware selects rather than in any parameter – see the table in section 1. TIMER0 is configured 64-bit as the chip timebase; TIMER1 is left as two independent 32-bit counters.

# 9. What is not provided here, and who provides it

Table 9-1. Functions this block does not provide

| Function | Where it lives |
| --- | --- |
| Interrupt status or flag | Nowhere. The block has none – section 6 |
| Interrupt acknowledge | Firmware acting on the block: TIMER_RESET_*, TIMER_CMP_* or IRQ_EN – 7.5 |
| Error response on a bad offset | Nowhere. PSLVERR is tied 0 – section 5 |
| Greater-or-equal compare | Nowhere. Firmware must add the margin – 7.4 |
| mtime and mtimecmp | Nowhere. QSOC has no CLINT – section 11 |
| Aggregation onto a CPU interrupt line | INTMAP |

# 10. Constraints this block imposes

Table 10-1. Constraints on firmware and integration

| On | Constraint | Consequence if missed |
| --- | --- | --- |
| Firmware | Compute the next compare target from the current count, with margin | A target just passed is missed for a full wrap – 214.7 s, or never in 64-bit mode |
| Firmware | Clear a one-shot interrupt by acting on the block | The handler re-enters immediately – 7.5 |
| Firmware | Do not write MODE_64 in CFG_REG_HI | Silently ignored – section 6 |
| Firmware | Use the region base; do not rely on a fault above 0x24 | Aliasing, silent – 7.6 |
| SCRC | Do not gate peri while a timer interrupt is asserted | The handler cannot clear its own source |

# 11. Requirements on others, and open items

Table 11-1. Requirements on other owners

| Item | Owner | What it blocks |
| --- | --- | --- |
| APB paddr width for the wrapper | bus owner | The wrapper’s port list. The IP declares 12 bits; the region is 16 KiB, so 14 |
| CLK_EN bit position for this block | SCRC owner | The wrapper’s clock port |
| TIMER0 on line 10, TIMER1 on line 6 of irq_fast_i | INTMAP owner (this author) | Settled in util/qsoc_contract.yml |
| What drives event_lo_i and event_hi_i, if anything | top-level owner | Hardware start. Tied 0 if nothing drives them |
| What drives ref_clk_i, if anything | top-level owner | The reference-clock counting modes of 7.1 |
| Timer driver in place of the standard mtime one | firmware owner | Any RTOS port – see below |

What QSOC gives up by having no mtime. The RISC-V privileged specification puts the machine timer at mtime/mtimecmp in a CLINT, and every RTOS port and most bare metal examples assume it. QSOC has no CLINT, so mip.MTIP is never set and irq_timer_i is tied off. The consequence is confined to software: firmware drives TIMER0 through the registers of section 6 instead, and an RTOS port must supply its own timer driver. Nothing in hardware is missing – a 64-bit counter with a compare is exactly what mtime is – but the interface is this block’s, not the standard one.

Accepted limits, stated rather than hidden:

- Equality compare, with no catch-up – 7.4. The most serious limit in the block.

- No interrupt status anywhere – section 6.

- No error response – section 5.

- 64 bytes aliasing across 16 KiB – 7.6.

- irq_hi_o unused in 64-bit mode – 7.3.

Open on this block: whether event_*_i and ref_clk_i are driven at all, and the gate count, which waits on synthesis.

# 12. Verification

The IP arrives with no testbench, so all of this is QSOC’s to write.

Table 12-1. Verification QSOC must add

| What is checked | Why it matters |
| --- | --- |
| A target written just after the counter passed it is missed, and the documented margin rule avoids it | The block’s most serious limit – 7.4. The test exists to prove the rule is necessary, not that the hardware is wrong |
| 64-bit chaining: counter_hi advances exactly once per wrap of counter_lo | The carry of section 3 |
| 64-bit mode raises one pulse at the intended value, and irq_hi_o stays low | 7.3 |
| One-shot holds the line until firmware acts; periodic pulses for one tick | 7.5, and the shape INTMAP assumes |
| Pulse width is one counting tick in each of the four rate modes | 7.1, 7.2 |
| MODE_64 in CFG_REG_HI has no effect | Section 6 |
| Reads of the four command offsets return zero | Section 6 |
| 0x00 and 0x40 are the same register | 7.6 |
| No APB access can stall P_BUS, for any offset in the 16 KiB region | Section 5 |

Acceptance criterion: TIMER0 in 64-bit mode raises exactly one interrupt at a programmed 64-bit target, and a target written late is proven to be missed – because a timebase that silently stops being a timebase is the failure this block can cause.

# Appendix A. Acronyms

Table A-1. Acronyms

| Acronym | Description |
| --- | --- |
| APB | Advanced Peripheral Bus, AMBA APB4 |
| CLINT | Core Local Interruptor – the RISC-V standard timer block. Not present in QSOC |
| CMP_CLR | Clear the counter on compare, making it periodic |
| IEM | Interrupt Event Mask: lets event_*_i start the counter |
| MODE_64 | CFG_REG_LO bit 31: chain both counters |
| mtime, mtimecmp | The RISC-V standard machine timer registers, in a CLINT |
| ONE_SHOT | Clear ENABLE on compare, stopping the counter |
| PRESC | Prescaler divisor |
| PSLVERR | APB slave error response. Tied 0 by this IP |

# Appendix B. First review

Table B-1. First review

| Item | Reviewer | Response |
| --- | --- | --- |
| Too long, and the redundancy causes wrong information | Teacher, 2026-09-23 | V2.0: 571 lines to this, with the record kept whole in _DECISIONS |
| Why two different timer IPs in one chip? | – | Section 4: 16-bit wraps in 3.28 ms, which cannot be a timebase. The roles want opposite counter widths |
| Is the 64-bit compare expression correct? | – | Yes, and 7.3 shows why the AND of two conditions still gives exactly one pulse |
| Does the block need an interrupt status register? | – | It has none and cannot be given one – it is IP. Section 9 says where acknowledgement happens instead |