QUY NHON SEMICONDUCTORS – QNSC

PULP APB GPIO

MICRO-ARCHITECTURE SPECIFICATION

V1.2

Contents

Contents	2

Revision and History	3

Table of Tables	4

Table of Figures	5

1.	Overview	6

2.	Features and Operation	6

2.1.	Implementation Configuration	6

2.2.	Basic Operation	7

2.3.	Supported Features	7

2.4.	Hardware Interfaces	8

2.5.	Register Map	9

2.6.	APB4 Compatibility and Required Adaptation	10

2.7.	SoC Connections	11

2.8.	Programming Sequences	12

3.	Block Diagram	13

4.	Verification Plan	14

5.	References	15

Appendix A.	Acronyms	16

Appendix B.	First Review	17

- Revision and History

| Version | Date (mm/dd/yyyy) | Author | Reviewer | Description of Change |
| --- | --- | --- | --- | --- |
| V1.0 | 09/14/2026 | Hieu Vu Minh Bui | - | Technical specification for PULP apb_gpio and APB4 integration requirements |
| V1.1 | 09/17/2026 | Hieu Vu Minh Bui | - | Aligned with source RTL: grouped sampling, PADIN latency, interrupt priority, reset/address behavior and pending APB4 adaptation. |
| V1.2 | 09/23/2026 | Hieu Vu Minh Bui | - | Updated the target MCU configuration from four 8-pin GPIO banks (32 pins) to three 8-pin GPIO banks (24 pins), with three APB selections and three interrupt outputs. |

Table of Tables

Table 2-1. GPIO hardware interfaces and SoC connections	8

Table 2-2. GPIO register map for one 8-pin bank	9

Table 2-3. GPIO APB4 compatibility assessment	10

# Table of Figures

Figure 3-1. GPIO block diagram and SoC connections	13

# Overview

The PULP apb_gpio peripheral provides memory-mapped control and status for digital general-purpose I/O. This specification describes the upstream RTL and the additional APB4 integration requirements. The target MCU configuration is three independent eight-pin banks, providing 24 GPIO signals; integration and verification of that configuration remain pending.

Source IP: pulp-platform/apb_gpio. Repository: https://github.com/pulp-platform/apb_gpio. This specification references commit f82caeb7f7d89427f05e9af5ed31e0675efe0d83.

The source RTL is distributed under the Solderpad Hardware License 0.51. Use, modification and redistribution are subject to the license terms, including preservation of applicable notices and identification of modified files.

# Features and Operation

## Implementation Configuration

The source module is apb_gpio. The target instance parameters are PAD_NUM=8, NBIT_PADCFG=4 and APB_ADDR_WIDTH=12. The upstream default PAD_NUM is 32; it does not automatically create three eight-pin banks.

The planned MCU integration uses three instances for GPIO banks 0 through 2, each with an independent APB selection, address window and interrupt output. This document does not establish that the three-instance top-level RTL has been implemented.

The upstream APB interface contains PADDR, PWDATA, PWRITE, PSEL, PENABLE, PRDATA, PREADY and PSLVERR. PSTRB and PPROT are not present in the upstream module.

APB4 adaptation is required but is not implemented in the reviewed upstream RTL. The project shall implement byte-enabled register updates and define PPROT handling before claiming compatibility with its APB4 interface.

The GPIO register and signal widths in this document describe one PAD_NUM=8 instance unless otherwise specified.

The MCU integration shall verify APB4 setup and access phases, byte-enabled writes, address decoding, reset behavior and interrupt connectivity for all three instances.

## Basic Operation

PADOUT stores output data; PADDIR uses 0 for input and 1 for output. PADOUTSET and PADOUTCLR are write aliases that modify PADOUT without a software read-modify-write. GPIOEN does not gate output writes. The input path is gpio_in to r_gpio_sync0 to r_gpio_sync1 to r_gpio_in. gpio_in_sync exposes r_gpio_sync1; PADIN reads r_gpio_in, one enabled sampling stage later.

GPIOEN sampling is grouped by four pins: OR(GPIOEN[3:0]) enables all three input registers for pins 0 through 3; OR(GPIOEN[7:4]) does the same for pins 4 through 7. A disabled group holds its previous samples. Interrupt qualification remains per pin.

Each pin event is qualified by its GPIOEN bit, INTEN bit and INTTYPE field. INTTYPE encodes 00 falling, 01 rising and 10 both edges. Encoding 11 is reserved and generates no event in this RTL revision. Use INTEN to mask events. PADDIR does not participate in event qualification.

interrupt is the OR of qualified events, not the OR of stored status. INTSTATUS accumulates qualified event bits. If an event and a status read occur at the same HCLK edge, event capture takes priority: next_status = old_status OR qualified_events, and the read does not clear any old bits. Without an event, a read access clears the stored status.

The RTL contains no debounce filter, level-sensitive interrupt mode, internal bidirectional pad or pin multiplexer. PADCFG stores configuration bits only; electrical behavior is implemented by the external pad library.

## Supported Features

The selected parameterization provides eight GPIO signals per instance; three planned instances provide 24 GPIO signals.

Direction control and output data are programmable independently for each GPIO signal.

PADOUTSET and PADOUTCLR support atomic modification of output bits.

Two input synchronization stages and one previous-sample register are updated together when the corresponding four-pin group is enabled.

Interrupt enable and falling-edge, rising-edge or both-edge selection are programmable per GPIO signal.

PADCFG provides four configuration bits per GPIO signal. The electrical function of these bits is defined by the selected I/O pad library.

The upstream RTL implements a zero-wait-state APB response with PREADY=1 and PSLVERR=0.

## Hardware Interfaces

Signal directions in Table 2-1 are specified relative to one PAD_NUM=8 GPIO instance.

Table 2-1. GPIO hardware interfaces and SoC connections

| Signal | Direction | Function | SoC connection |
| --- | --- | --- | --- |
| HCLK | Input | Clock for registers, synchronizers and interrupt logic | PCLK from APB clock domain |
| HRESETn | Input | Asynchronous active-low reset | PRESETn / Reset Controller |
| dft_cg_enable_i | Input | DFT clock-gating control port; unused by current RTL | Tie to agreed DFT value |
| PADDR[11:0] | Input | Local byte address; decoder uses PADDR[6:2] | P_BUS address low bits |
| PWDATA[31:0] | Input | APB write data | P_BUS |
| PWRITE | Input | Read/write direction | P_BUS |
| PSEL | Input | Select this GPIO slave | APB decoder output |
| PENABLE | Input | APB access phase | P_BUS |
| PRDATA[31:0] | Output | APB read data | P_BUS read mux |
| PREADY | Output | Transfer ready; tied to 1 | P_BUS |
| PSLVERR | Output | Slave error; tied to 0 | P_BUS |
| gpio_in[7:0] | Input | Raw logic levels from pads | IO MUX / pad input |
| gpio_in_sync[7:0] | Output | r_gpio_sync1; two stages, enabled per four-pin group | Optional internal observation / IO logic |
| gpio_out[7:0] | Output | Output value for each pin | IO MUX / pad output |
| gpio_dir[7:0] | Output | Direction: 0 input, 1 output | IO MUX / pad output-enable |
| gpio_padcfg[7:0][3:0] | Output | Four configuration bits per pad | Pad-control block; meaning is technology-specific |
| interrupt | Output | OR of qualified events; not a sticky pending level | Interrupt Controller / CPU |

## Register Map

A register address is GPIO_BANK_BASE plus its offset. The GPIO0_BASE through GPIO2_BASE values, system P_BUS address width and IRQ assignments remain subject to the final integration map. The instance receives a 12-bit local address; the upstream register decoder uses only PADDR[6:2].

Table 2-2. GPIO register map for one 8-pin bank

| Register | Offset | Access | Used bits | Function |
| --- | --- | --- | --- | --- |
| PADDIR | 0x00 | RW | [7:0] | 0=input; 1=output |
| GPIOEN | 0x04 | RW | [7:0] | Sampling enable per four-pin group; interrupt qualification per pin |
| PADIN | 0x08 | RO | [7:0] | Read r_gpio_in, one enabled stage after gpio_in_sync |
| PADOUT | 0x0C | RW | [7:0] | Output value register |
| PADOUTSET | 0x10 | WO alias | [7:0] | Write 1 to set PADOUT bits; reads return 0 |
| PADOUTCLR | 0x14 | WO alias | [7:0] | Write 1 to clear PADOUT bits; reads return 0 |
| INTEN | 0x18 | RW | [7:0] | Enable interrupt detection per pin |
| INTTYPE | 0x1C | RW | [15:0] | 2 bits/pin: 00 falling, 01 rising, 10 both, 11 reserved (no event) |
| INTTYPE16_31 | 0x20 | Unused | - | No implemented bits for eight pins; read 0, writes ignored |
| INTSTATUS | 0x24 | RC | [7:0] | Sticky event status; read clears only if no simultaneous event |
| PADCFG | 0x28 | RW | [31:0] | Four pad-control bits per pin |

Reset values: all implemented direction, enable, output, interrupt-type, pad-configuration, status and input-pipeline state resets to zero. PADOUTSET and PADOUTCLR have no independent storage and read as zero. Unimplemented bits read as zero and ignore writes. Writes to PADIN and INTSTATUS have no effect.

Address behavior: PADDR[1:0] and PADDR[11:7] are ignored by the core. With PSEL asserted, offsets separated by 0x80 alias; 0xA4 has the same read-clear side effect as 0x24. For PAD_NUM=8, offsets 0x20 and 0x2C through 0x6C have no implemented pin state; 0x70 through 0x7C select the default case. Reads return zero and writes have no effect at those word indices. The core never reports an address error. Any stricter decoding belongs to the custom integration.

## APB4 Compatibility and Required Adaptation

Table 2-3. GPIO APB4 compatibility assessment

| Item | Upstream IP | Required project treatment |
| --- | --- | --- |
| 32-bit data | Supported | Use 32-bit read/write data; apply the byte-write behavior specified below. |
| PSTRB[3:0] byte enables | Not present | Add byte enables to register updates, preserving unselected bytes. SET/CLR require operation-specific masking. Not yet implemented in reviewed RTL. |
| PPROT[2:0] | Not present | Define the system access-control policy. Enforcement may be upstream or peripheral-side. Document unused PPROT explicitly; adding the port alone does not implement protection. |
| PREADY | Always 1 | Upstream response is zero-wait. A sequencing adapter shall define its own PREADY behavior and prevent repeated core accesses. |
| PSLVERR | Always 0 | Upstream never reports an error. Define custom error responses and side effects if the integration requires access checks. |
| PADDR width | 12-bit local | Pass the 12-bit local address after system decoding. Final system address width and base addresses remain pending; 24 bits is only a bus-generator example. |
| protect_en / slverr_en | Not standard APB4 ports | These controls remain in the P_BUS interconnect unless a peripheral-side contract is defined. |

Byte-write requirements below apply to the project adaptation, not to the upstream implementation. Expand each PSTRB bit into eight byte-mask bits. For an RW register, next = (old & ~byte_mask) | (PWDATA & byte_mask). For PADOUTSET, next_out = old_out | (PWDATA & byte_mask). For PADOUTCLR, next_out = old_out & ~(PWDATA & byte_mask). PSTRB=0000 shall cause no write-side state change.

For eight pins, PADDIR, GPIOEN, PADOUT, SET, CLR and INTEN use byte 0; INTTYPE uses bytes 0 and 1; PADCFG uses all four bytes. INTTYPE pin i occupies bits [2*i+1:2*i]; PADCFG pin i occupies bits [4*i+3:4*i]. PSTRB selects register bytes, not pin numbers.

Example: PADCFG=0x11223344, PWDATA=0x000000AA and PSTRB=0001 shall produce 0x112233AA. Masking PWDATA alone would incorrectly clear the other three bytes.

To preserve zero-wait operation, byte enables can be added directly to register-update logic. An external read-modify-write adapter instead requires sequencing, wait-state control and an atomicity policy; it shall not blindly read registers with side effects. If an adapter rejects a transfer, it shall prevent unintended core side effects. No such adapter is verified by this specification.

## SoC Connections

Each planned GPIO bank shall connect to P_BUS with an independent PSEL and base-address window. Bus address width, IRQ indices and pad assignments shall be recorded in the top-level integration specification.

HCLK connects to PCLK, and HRESETn connects to PRESETn or an equivalent active-low reset source.

gpio_in, gpio_out, gpio_dir and gpio_padcfg connect to the I/O multiplexer and pad-control logic. The physical I/O pads are external to the GPIO IP.

The RTL drives gpio_dir directly. Translate its polarity if the selected pad uses an active-low output enable. GPIOEN is not a tri-state output-enable signal.

Each interrupt output shall connect to a controller capable of capturing the qualified event signal. The output is not a pending level derived from INTSTATUS. A single isolated edge normally produces one HCLK cycle under stable sampling and enable conditions; consecutive events or enable changes need separate verification.

gpio_padcfg connects to the SoC pad-control implementation. If pad configuration is not implemented, its treatment is defined by the selected I/O pad library and top-level integration contract.

## Programming Sequences

Output sequence: software writes PADOUT, PADOUTSET or PADOUTCLR to establish the output value, then writes the corresponding PADDIR bit to 1. The resulting gpio_out and gpio_dir signals are forwarded through the I/O multiplexer and padframe.

Input sequence: select input direction and configure the external pad, then set the required GPIOEN bit. Allow the enabled sampling pipeline to settle before reading PADIN. For an input stable before the first enabled HCLK edge, gpio_in_sync updates after edge 2 and the PADIN source after edge 3 in ideal RTL sampling; APB read completion adds bus timing.

Interrupt initialization: keep the target INTEN bit clear; configure input direction and pad controls; program INTTYPE; set GPIOEN; allow the sampling pipeline to settle; read INTSTATUS to remove stale status; then set INTEN. Event capture starts when INTEN becomes effective. Input transitions during initialization are not guaranteed to be retained.

Interrupt service: read INTSTATUS to identify recorded pins. A valid read clears status only when no qualified event is captured at the same edge. Status may remain set after the interrupt output returns low; software and the interrupt controller shall not assume that reading status deasserts a held IRQ level.

# Block Diagram

Figure 3-1 is a functional diagram derived from the reviewed upstream RTL, not an official PULP diagram. It shows the unmodified core; PSTRB and PPROT adaptation is outside this figure. External pad and interrupt-controller blocks lie outside the IP boundary. Dashed sampling-control paths represent register enables, not a separate generated clock.

The read mux selects register data, r_gpio_in for PADIN, or r_status for INTSTATUS. PREADY=1 and PSLVERR=0 are independent response constants. The edge detector compares r_gpio_sync1 with r_gpio_in; qualified events branch separately to the status register and the interrupt OR reduction.

Figure 3-1. GPIO block diagram and SoC connections

# Verification Plan

1. Reset: verify zero state for RW registers, status and all input stages; SET/CLR aliases read zero. This is a required test, not a recorded PASS result.

2. APB and address decoding: verify setup/access behavior, PRDATA=0 outside read access, PREADY=1 and PSLVERR=0. Check low-address-bit and 0x80 aliases in upstream RTL, reserved locations and ignored writes to read-only registers.

3. APB4 adaptation: test all 16 PSTRB patterns, including 0000 and noncontiguous masks, on RW and SET/CLR writes. Unselected bytes shall retain their values. Verify the agreed PPROT and error policy; these tests require the custom implementation.

4. Output and direction: verify PADOUT, SET and CLR behavior, alias reads, direction bits and external pad-enable polarity. GPIOEN shall not prevent output-register writes.

5. Input sampling: verify the two synchronization stages and additional PADIN stage. With GPIOEN=0x01, pins 0 through 3 update and pins 4 through 7 hold. When all enables in a group are zero, all three stages hold their last values.

6. Interrupt: test falling, rising and both-edge selection, reserved type 11 and per-pin INTEN/GPIOEN masks. Verify that PADDIR does not mask events and that the controller captures event pulses even though INTSTATUS remains set afterward.

7. Read-clear: without a simultaneous qualified event, an INTSTATUS read returns stored status and clears it at the access edge. With a simultaneous event, next status shall equal old_status OR qualified_events; all old bits remain set and clear is suppressed.

8. Integration: verify independent selection of all three banks, final base addresses, 24 pad connections, three IRQ routes, reset and byte-write behavior. If the adapter inserts waits or errors, ensure no repeated or rejected transfer changes core state.

# References

PULP APB GPIO repository: https://github.com/pulp-platform/apb_gpio

Reviewed commit: https://github.com/pulp-platform/apb_gpio/tree/f82caeb7f7d89427f05e9af5ed31e0675efe0d83

Top-level RTL: https://github.com/pulp-platform/apb_gpio/blob/f82caeb7f7d89427f05e9af5ed31e0675efe0d83/rtl/apb_gpio.sv

License: https://github.com/pulp-platform/apb_gpio/blob/f82caeb7f7d89427f05e9af5ed31e0675efe0d83/LICENSE

Team P_BUS generator: https://github.com/nguyenquanicd/APB-BUS-Generator

Acronyms

| Acronym | Description |
| --- | --- |
| APB | Advanced Peripheral Bus |
| APB4 | AMBA APB Protocol version 2.0 / commonly called APB4 |
| CPU | Central Processing Unit |
| DFT | Design for Test |
| GPIO | General Purpose Input/Output |
| IP | Intellectual Property core |
| IO | Input/Output |
| MCU | Microcontroller Unit |
| PSTRB | APB write byte strobes |
| SoC | System on Chip |

First Review

| No. | Reviewer | Response |
| --- | --- | --- |