# Overview

QSOC is a 32-bit RISC-V microcontroller, a training project at Quy Nhon Semiconductors: single core, on-chip memory only, AXI4 system bus, APB4 peripheral bus, sixteen peripheral slaves out to forty-eight pads.

Two constraints shape the design, both from the project's tool licences, not from a product requirement. No flash: the application is downloaded into RAM over a serial port after every reset and run from there. No phase-locked loop (PLL): the whole chip runs from one external clock.

The device uses open-source IP wherever a suitable block exists: the core, the buses, the memory controllers, most peripherals. In-house RTL covers only the clock and reset controller, the debug block, and adapters where two IPs differ in protocol. Each block has an owner and its own spec; this document is the system view and stays consistent with them.

# Features

Technology:

– SMIC 28nm, 20MHz

Core: 32-bit RV32IMC RISC-V CPU (lowRISC Ibex), 20 MHz

– Two-stage pipeline, in-order, machine mode; no branch predictor, 1 cycle penalty on a taken branch

– 1 cycle for add, subtract, shift, logic and compare; 3 for multiply, 4 for the high-half forms; 37 for divide and remainder

– 32 x 32-bit register file in flip-flops

Memory: 98 Kbytes on chip, no external memory interface

– 2 Kbytes ROM holding the serial bootloader, read-only

– 64 Kbytes instruction RAM: 4 Kbytes debug entry code plus 60 Kbytes application

– 32 Kbytes data RAM, split into ISRAM and DSRAM

Reset and supply

– Active-low reset, 3 sources: power-on, watchdog and software

– Sticky reset cause register; SYSCSR only reset by Power-on reset to keep the value of register

Clock: one external input on a dedicated pad

– One frequency for the whole chip

– Clock gating is allowed for peripherals only, never for the CPU, the buses or the memories

Bootloader preloaded in ROM

– Serial download after every reset: one frame of header, payload and CRC32

– Bootloader fits the 2 Kbyte ROM; Table 13-4 carries the size estimate

– The debug interface is the second load path, used when the ROM itself is faulty

Debug over JTAG: halt, resume, memory access without the CPU

– In-house block, control registers reachable only over JTAG and deliberately off the system bus

– 1 hardware breakpoint (works in read-only memory). Debug entry code occupies the first 4 KiB of ISRAM — no separate debug memory, no program buffer. One master port serves memory access, run control and register reads.

48 pins in four sides of twelve, 35 carrying peripheral functions

– Each muxed pin selects one of 4 functions through a two-bit field, all defaulting to the first after reset

– 32 general-purpose I/O in 4 blocks of 8, one interrupt per block; 8 power and ground pins; 3 no-connect pads; 2 dedicated for clock and reset

Interrupts: 27 sources on 12 wires to the core

– INTMAP is an OR tree: one OR gate per peripheral

– The core identifies the source from mcause, so no register read is needed. Table 8-1 gives the mapping

– The watchdog bark is the one non-maskable interrupt and bypasses INTMAP

– Eleven sources are pulses. Each peripheral records its own event, so none is lost

DMA: three modes on one engine, register job, descriptor chain and eight peripheral channels, with one completion interrupt

– Separate AXI read and write ports merged before the system bus; reaches the ROM, both RAMs and the peripheral window

– The chosen front-end has no completion interrupt and no enable input

12 peripheral instances on the peripheral bus

– 2x serial port (UART0/1); UART0 carries the boot download

– 1x SPI master and 1x SPI target sharing one pin group

– 1x I2C, controller or target

– 4x 8-pin GPIO

– TIMER0 and TIMER1: apb_timer_unit, two compare outputs each. TIMER0 chains them into one 64-bit counter, so it raises 1 interrupt; TIMER1 raises 2

– PWM: a separate apb_adv_timer, 16 channels, 8 to pads, 4 event lines, 4 external triggers

– Watchdog: bark interrupt at the first threshold, bite reset at the second. Disabled after reset, lockable. Largest window 3.58 minutes

# Block diagram

Ibex core

– Executes the program. Provides two memory-style ports, one for instruction fetch and one for data, plus sixteen usable interrupt inputs

CPU2AXI

– Converts each core port to the system bus protocol and merges them into one bus port, so the core takes one slave port instead of two

SYSDBG

– The debug block. Takes JTAG, halts and resumes the core, reads and writes memory over its own bus port without involving the core

– The block's control registers respond only over JTAG and are deliberately not placed on the system bus, so software running on the core cannot grant itself debug access

S_BUS

– The AXI4 crossbar. Routes three access sources to four destinations by address

– Returns a decode error for every unmapped address, which the memories themselves cannot do

ROM

– Holds the bootloader. Its content is fixed when the design is built, and it is made read-only by tying its write channel to the idle state

ISRAM

– Instruction memory. Holds the debug entry code in the first 4 Kbytes and the downloaded application in the upper part

DSRAM

– Data memory. Being separate from the instruction memory means a stack overflow cannot overwrite the running program

AXI2APB

– The bridge between the two buses, built as two modules in series: full AXI4 to the lite version of AXI, then lite AXI to APB

– This block also splits bursts, which the peripheral protocol has no concept of and which only the DMA generates

P_BUS

– The APB4 router. Subtracts each target's base address before forwarding, so every peripheral sees its registers starting at 0

– Answers an unmapped address itself in the same cycle instead of hanging while waiting for a peripheral that will never respond

SCRC (System Clock Reset Control)

– The system clock and reset controller. Takes the single external clock, merges the three reset sources, and distributes gated clocks and synchronised reset to every domain

– Software reaches it through two register windows on the peripheral bus: the control registers at APB_M0 and the status registers, SYSCSR, at APB_M1. The clock and reset logic itself is not addressable - only those registers are

SYSCSR

– The status half of SCRC: reset cause surviving a watchdog or software reset, which domains are currently held in reset, and the device identification code. Read-only; every control bit - clock gating per peripheral and the software reset bit - lives in the control window at APB_M0

INTMAP

– An OR tree, one gate per peripheral, combining 26 sources onto the eleven fast interrupt lines irq_fast_i[10:0]. It is not a bus slave: no registers, no APB interface, no address, and zero flip-flops, so it adds no clock cycle on the path from a peripheral to the core

– QSOC writes the combining logic itself: an OR tree and a concatenation, with no state. It latches nothing and has no status register, which is what removes the bus port and releases AXI_M4

IO MUX

– Selects which peripheral signal reaches each pad, and holds the select registers

– Not a block in the usual sense: it is a wide combinational select network with no clock or reset of its own, plus the registers that control it

WDT

– The watchdog. Raises an early warning interrupt at the first threshold and a reset request at the second

TIMER0 and TIMER1

– Two apb_timer_unit instances, two compare outputs each, no channel pin. TIMER0 in 64-bit mode drives only irq_lo_o → 1 source; TIMER1 as two 32-bit units → 2 sources. Interrupt is a pulse when periodic, a held level in one-shot. TIMER0's clock gate is open out of reset (it is the timebase), TIMER1's starts closed.

PWM

– One apb_adv_timer instance: four independent 16-bit modules, four channels each → sixteen channel outputs (ch_i_o[3:0] are the four channels of module i). Eight reach pads as PWM_0–PWM_7 — the four channels of module 0 and module 1 — giving two independent base frequencies, four channels each. Four event lines to INTMAP, each a one-cycle pulse selecting any one of the sixteen channels.

– Different IP from TIMER0/TIMER1 (apb_timer_unit, no channel pin). Naming trap: the four modules inside apb_adv_timer are also called Timer0–Timer3. Integration: TIM_EXT0–TIM_EXT3 → ext_sig_i[3:0], upper 28 bits tied 0; closing the PWM clock gate freezes the outputs at their current level — not a safe state for a power stage.

UART0 and UART1

– 16550-compatible serial ports. The first port carries the boot download

SPI master and SPI target

– Two separate IPs sharing one peripheral port and one pin group, so only one role is active at a time

I2C

– The two-wire bus controller. Needs three signals per line and open-drain pads with external pull-up resistors

GPIO0 to GPIO3

– Eight pins per block, with direction control, input synchronisation and interrupt generation

DMA

– Moves blocks of data between the memories without the core. Configured over the peripheral bus, moves the data on the system bus

# Clock and Reset

## Clock List

One root buffer takes the pin clock and fans out by cluster - core and buses, peripherals, memories - with one integrated clock gate per domain. Five domains are never gated, and that is a design rule rather than a default: their gate enables are hardwired active so no software mistake can stop them.

Table 4-1. Clock gating rule

| Domain group | Gate | Why |
| --- | --- | --- |
| CPU, System BUS, Peripheral BUS, ROM, RAM | Hardwired on, not writable | Gating any of these removes the agent that would have to re-enable them. Nothing is left awake to recover |
| Real peripherals: WDT, TIMER0/1, UART0/1, SPI, I2C, GPIO, DMA, SPI device, PWM | Writable through CLK_EN in SCRC | These are the only blocks whose clock can be stopped for power without stranding the system |

A gated peripheral must still answer the bus. A small always-powered responder inside SCRC watches CLK_EN and, when an APB transaction targets a gated domain, returns an error immediately instead of letting the transfer hang. It runs on the peripheral bus clock, which is one of the five that are never gated. It does not re-enable the clock and does not retry: software reads the error, writes CLK_EN, then repeats the access itself.

## Reset List

Reset arrives in two tiers. Tier 1 combines the global sources with a plain OR gate, fully asynchronous so it needs no running clock. Tier 2 gives every domain its own synchroniser clocked by that domain, released under SCRC control.

Table 4-2. Reset sources

| Source | Tier | Note |
| --- | --- | --- |
| Power-on reset | 1 | Asynchronous. Resets everything |
| Watchdog request | 1 | Asynchronous. Same OR gate as power-on |
| Software reset | 2 | A single CSR bit in SCRC. Resets every domain except the CPU, so the CPU survives to observe the result |

Three sources, not five: the external reset pin and the debugger request were both removed in the design review of 18/09/2026. There is no priority between sources and no mux - with an OR gate the result does not depend on which source arrived first, and RESET_CAUSE can latch more than one bit when two sources coincide. SYSCSR itself is reset only by power-on, so the cause survives a watchdog or software reset and can be read after restart.

Two ordering constraints are mandatory and neither can be verified by simulating a block on its own. The debug block must leave reset before the CPU, because halting the core before it executes its first ROM instruction is the only way to debug a fault inside the bootloader. And the debug block must sit outside every reset tree except power-on: if a system reset dropped the debug block too, issuing that reset would kill the JTAG link at the moment it is needed and the host would see the target disappear with no explanation.

## Clock Relationship

Every domain derives from the same root, so all of them are synchronous to each other and no clock-domain crossing is needed anywhere in the functional path. Two crossings exist and both belong to the debug block: the JTAG TAP runs on the adapter's own clock, which is free to stop between commands, and the system side runs on the chip clock. Neither side may wait on an edge of the other.

SCRC is not a state machine. It is a small MCU: a single-cycle RV32I core with a 13-instruction subset, a dedicated instruction ROM, a bus interface and an APB generator, built from library cells. Its program has two regions - a power-up sequence that runs once and brings the domains up in order, and a runtime handler entered whenever the CPU writes SOFT_RST_CTRL or CLK_EN. The handler reads the register, applies the change to the addressed domain, then returns to its idle loop. One consequence is worth stating in the software notes: the interrupt into that core is edge-triggered with no busy flag, so two writes issued back to back can lose the second one, and firmware should not write these two registers in immediate succession.

# BUS

QSOC has two buses joined by a bridge. The system bus carries the heavy traffic - instruction fetch, data reads and writes, debug accesses, DMA transfers. The peripheral bus carries register accesses, which are sparse and not worth a dedicated AXI port per block.

## S_BUS

Table 5-1. S_BUS configuration

| Item | Value | Reason |
| --- | --- | --- |
| Protocol | AXI4 full | - |
| Address width | 32 bits | Matches the CPU core |
| Data width | 32 bits | Matches the CPU core and the memories. A wider bus would need packing logic at every 32-bit endpoint |
| Slave ports | 3 | CPU core, debug block, DMA |
| Master ports | 4 | ROM, ISRAM, DSRAM, AXI2APB bridge. Adding a fifth master port does not widen the identifier width, because the crossbar scales with the number of slave ports, not with the number of master ports |
| Identifier width | 5 bits at the slave ports, 7 at the master ports | The crossbar extends the identifier by the logarithm of the number of slave ports so that they stay unique after merging |
| Outstanding transactions | 2 per port | Enough to overlap the address phase with the data phase; a deeper queue costs extra registers for traffic that this design does not generate |
| Register stage | Cut on every address and data channel | Breaks the combinational path through the crossbar, at the cost of one cycle of latency per channel |
| Connectivity | Every slave port reaches every master port | The DMA must reach all of the memory and the peripheral window, and the debug block must reach everything the CPU core reaches |

Table 5-2. S_BUS ports

| Port | Connects to | Note |
| --- | --- | --- |
| AXI_S0 | SYSDBG | The debug block's master port is not AXI but a req/gnt/rvalid handshake interface with byte enables, the same kind as the CPU core's memory port, so this port must have a protocol adapter |
| AXI_S1 | CPU2AXI | The CPU core's two interfaces are converted separately and then merged by a multiplexer, so one port carries both instruction fetch and data traffic |
| AXI_S2 | DMA | The DMA exposes a separate read port and write port, so they have to be joined ahead of this port |
| AXI_M0 | ROM | 0x0000_0000 to 0x0000_07FF |
| AXI_M1 | ISRAM | 0x2000_0000 to 0x2000_FFFF |
| AXI_M2 | DSRAM | 0x3000_0000 to 0x3000_7FFF |
| AXI_M3 | AXI2APB bridge | 0x8000_0000 to 0x8003_FFFF |

The two buses use opposite conventions for a region's end address, so Table 5-2 and Table 5-4 do not go into code the same way. The crossbar decoder treats the end address as exclusive: a rule takes the table value plus one, and using the table value as is makes the last byte of every region raise a decode error. The peripheral bus decoder treats it as inclusive. The crossbar overlap check exists only in simulation; on silicon the rule that appears later in the map silently wins.

## P_BUS

Table 5-3. P_BUS configuration

| Item | Value | Reason |
| --- | --- | --- |
| Protocol | APB4 | Most peripheral IP exposes an APB slave |
| Address width | 18 bits | Sixteen slaves at 16 KiB each is 256 KiB, which needs eighteen bits |
| Data width | 32 bits | Matches the system bus |
| Slave ports | 16 ports, 16 KiB each | A uniform address stride makes an address recognisable by eye and lets the decoder compare a fixed field |
| Byte strobe | Present on the bus | Needed so that a byte write does not rewrite the whole register; two peripherals do not implement it, so they need a wrapper |
| Error response | Yes | Used by the router itself for unmapped addresses |

Table 5-4. Slave allocation on P_BUS

| Port | Block | Base address |
| --- | --- | --- |
| APB_M0 | SCRC | 0x8000_0000 |
| APB_M1 | SYSCSR | 0x8000_4000 |
| APB_M2 | WDT | 0x8000_8000 |
| APB_M3 | GPIO0 | 0x8000_C000 |
| APB_M4 | GPIO1 | 0x8001_0000 |
| APB_M5 | GPIO2 | 0x8001_4000 |
| APB_M6 | GPIO3 | 0x8001_8000 |
| APB_M7 | TIMER0 | 0x8001_C000 |
| APB_M8 | TIMER1 | 0x8002_0000 |
| APB_M9 | UART0 | 0x8002_4000 |
| APB_M10 | UART1 | 0x8002_8000 |
| APB_M11 | SPI wrapper | 0x8002_C000 |
| APB_M12 | I2C | 0x8003_0000 |
| APB_M13 | PWM | 0x8003_4000 |
| APB_M14 | DMA configuration | 0x8003_8000 |

## Protocol Adapter

Some IP does not speak the protocol of the bus it attaches to, so an adapter is mandatory at each point: the CPU core emits no AXI; three peripheral blocks use a different register bus protocol; the debug block exposes a handshake interface; the DMA exposes two manager ports where the crossbar gives one; None is optional; most exist in the IP families the project uses, so the work is picking and parameterising them.

Table 5-5. The four debug parameters of the core

| Parameter | Value | Why this value |
| --- | --- | --- |
| DmBaseAddr | 0x2000_0000 | Base of the debug window, which is the first 4 KiB of the instruction RAM. Reserved by QNSC_RAM_MAS, so the main image links at 0x2000_1000 |
| DmAddrMask | 0x0000_0FFF | Size minus one. The core matches the window as address AND NOT mask equals base, so the window must be a power of two aligned to its own size |
| DmHaltAddr | 0x2000_0800 | Where the core jumps on every halt, so it must always hold valid code. The debug block writes the dispatch loop there over JTAG at the start of a session |
| DmExceptionAddr | 0x2000_0810 | Halt address plus 16. The core's default is plus 8, which in the reference debug module is the resume entry point, so leaving the default makes an exception inside debug mode jump into the resume path |

Two of the four only take effect when memory protection is enabled, and it is disabled on this chip, as Table 6-4 row 11 records. They are still set to the values in Table 5-5 rather than left at their defaults, because a later decision to enable protection would otherwise inherit an address range that does not belong to QSOC.

Table 5-6. Mandatory protocol conversions

| Location | From | To | Why it cannot be skipped |
| --- | --- | --- | --- |
| S_BUS slave port AXI_S1 (CPU2AXI) | Ibex req/gnt/rvalid memory interface, two ports | AXI4 full, one port | The CPU core emits no AXI: its two memory ports use a req/gnt/rvalid handshake with address, byte enables and data, no channels, no identifiers, no bursts. Convert each port with axi_from_mem and then merge them with axi_mux. On naming: the Ibex RTL does not use the string OBI anywhere and axi_from_mem calls this interface mem, so calling this point OBI2AXI follows the habit of the PULP IP family rather than the name in the RTL |
| S_BUS slave port AXI_S0 | req/gnt/rvalid with byte enables (debug block) | AXI4 full | The debug block's master port is not AXI but a plain handshake interface, the same kind as the CPU core's memory port. It shares one adapter form with the conversion point at the CPU port: use axi_from_mem from pulp-platform/axi with MaxRequests set to 1, since the debug block keeps exactly one command outstanding. The adapter belongs to the bus owner, not to the debug block |
| S_BUS slave port AXI_S2 | Two AXI4 ports (DMA read and write) | One AXI4 port | Not a protocol change but a port join: the DMA exposes a separate read manager and write manager while the crossbar provides only one port. Adding a fourth crossbar port would avoid the join and would not widen the bus identifier, but it costs one more decoder and one more error-response block |
| P_BUS port APB_M2 (WDT) | APB4 | TL-UL | The watchdog is a TileLink Uncached Lightweight slave, not an APB slave. It has registers only, with a single outstanding transfer per side, so no queue or burst handling is needed |
| Inside APB_M11 (SPI master) | APB4 | TL-UL | The SPI master is also a TileLink slave. Its register file declares a single block six address bits wide and has no buffer window; the two data buffers are read and written through ordinary registers, so the adapter only has to convert register accesses |

# CPU

## CPU Block Diagram

The pipeline has two stages. Stage one generates the program counter, fetches, expands compressed instructions, decodes and reads the register file; stage two executes, does the memory access if there is one, accesses the control registers and writes back - there is no separate writeback stage. A prefetch buffer keeps the instruction fetch port busy; with no branch predictor, a taken branch discards the prefetch and costs one cycle.

## Interfaces

Table 6-1. Interface groups

| Group | Signal count | Data direction | Purpose |
| --- | --- | --- | --- |
| Instruction | 6 | Read only | Instruction fetch. Request and address go out; grant, valid, data and error come back. No write path exists |
| Data | 9 | Read and write | Data reads and writes. Adds a write enable signal, write data and four byte enables to that same handshake mechanism |
| Interrupts | 5 signals, 19 wires | In | Software interrupt, timer interrupt and external interrupt, fifteen fast interrupts, one non-maskable interrupt |
| Debug | 1 | In | Request to enter debug mode |
| Clock, reset, boot and status | 5 | Both directions | Clock, active-low reset, boot address, hart identifier, sleep status output |

The core speaks a simple request-and-grant memory interface and the system bus speaks AXI4, so an adapter is mandatory. The core also has two memory ports, one for instruction fetch and one for data, while the bus accepts one transaction at a time - so the adapter both translates and queues. Table 6-2 lists what it has to get right; signal names are in the RTL rather than here.

Table 6-2. Requirements on CPU2AXI

| No. | Requirement | How it is done | If it is missing |
| --- | --- | --- | --- |
| 1 | Choose between the two ports when both request in the same cycle | Fixed priority, data first. A stalled data access stalls the core in the same cycle, while the instruction path has a prefetch buffer that absorbs one cycle | One of the two requests is dropped |
| 2 | Keep the byte enables on a write | The byte enables from the data port are passed straight to the bus. The instruction port has no write path | A single-byte write rewrites the whole word, and the memory controller cannot repair it |
| 3 | Return each response to the port that asked | Only one transaction is outstanding, so the adapter's own state says which port is being served and no tag table is needed | An instruction fetch response returned on the data path corrupts data with no error raised |
| 4 | Never drive a write from the instruction port | Instruction requests take the read path only | A write would be issued on a port that has no write path |
| 5 | Transfer one word at a time | Every access is a single beat of four bytes. The core never bursts; only the DMA does | A burst would have to be split again at the peripheral bridge, for no gain |
| 6 | Report a bus error to the core | The bus response is wired to the core's error inputs on both the instruction and the data side | A failed access looks like a successful one and the core continues on whatever the bus returned |
| 7 | Note for later: give the ports different identifiers before allowing them to overlap | Not implemented and not needed while one transaction is outstanding. Both ports use the same identifier today | Two overlapping transactions with one identifier may have their responses returned in either order, with nothing to tell them apart |

## CPU Operation Modes

Table 6-3 lists the states the core itself can be in. The chip-level modes are a different unit and are in Table 12-1. What follows the table is how the core enters and leaves sleep, because that is where firmware has to get the register state right.

Table 6-3. Core states

| State | How it is entered | What runs | What is active |
| --- | --- | --- | --- |
| Machine mode | The state after reset, and the state returned to from any trap | All code: bootloader, application, interrupt handlers | Everything. Every instruction and every control register is usable. User mode exists in the core and is not used: running only in machine mode is a firmware convention, not a hardware boundary |
| Sleep | Executing the wait-for-interrupt instruction while no enabled interrupt is pending | Nothing. The pipeline is halted and the core stops its own clock | The clock gate inside the core is closed and the sleep status output is high. Five events reopen it: an enabled interrupt, the non-maskable interrupt, a debug request, already being in debug mode, or single-step being active. The core never wakes itself |
| Debug mode | The debug block raises the request input, a breakpoint instruction is executed, or a hardware trigger matches | The dispatch loop and command sequences at the debug halt address | Ordinary interrupts are not taken, and the non-maskable interrupt is ignored as well. Memory is read and written over the debug block's own bus port and needs no halt. Reading a CPU register does need the halt, because a register has no bus address: the debug block writes a short instruction sequence into the debug window, the core executes it and stores the value to a known address, and the debug block then reads that address |

Entering sleep takes three steps and no register write. Firmware executes the wait-for-interrupt instruction; the controller leaves its decode state for an intermediate state where it drops its busy flag, stops issuing instruction requests and flushes the decode stage; on the next cycle it enters the sleep state and stays there. With the busy flag clear the core's clock enable falls, and the clock gate instantiated inside the core stops the core clock, which is where the saving comes from. The sleep status output is the inverse of that clock enable.

Leaving sleep needs one of five events: an enabled interrupt, the non-maskable interrupt, a debug request, already being in debug mode, or single-step being active. The core then re-fetches and, if the interrupt is also enabled globally, takes the handler. Nothing else wakes it: there is no timer inside the core, and no register a host can write to resume it.

Two properties of the wake-up condition are worth stating because firmware depends on them. The pending signal is qualified by the per-source enable register alone and not by the global interrupt enable bit, so clearing a per-source enable removes that source's ability to wake the core at all - a sleep entered with every per-source enable clear is left only by the non-maskable interrupt or by a debug request. Conversely, clearing the global enable still allows the wake-up: the core resumes at the instruction after the wait, without taking the handler, which is the usual way to wait for an event without writing an interrupt service routine. In debug mode and in single-step the wait instruction behaves as a no-operation.

The clock the core stops is its own. QSOC's clock controller keeps the CPU domain hardwired on, as Table 4-1 records, so the wait instruction does not interact with it. The sleep status leaves the wrapper as o_pwr_sleep, and it has no destination today because QSOC has no power manager.

## Features and Configurations

Each row states the option, QSOC's pick, what the pick brings and what is lost without it. Rows differing from the published reference configuration for a core of this size class say so.

Table 6-4. CPU configuration

| No. | Feature | Options | QSOC | Reason: when present, and when absent |
| --- | --- | --- | --- | --- |
| 1 | Base integer instruction set | RV32I / RV32E | RV32I | RV32I has thirty-two registers, the reduced set has only sixteen. With RV32I the standard compiler and the prebuilt libraries run as they are, with no modification. Without it, sixteen registers save flip-flops but the compiler has to spill registers to memory more often, which inflates the code and increases memory traffic - and code size is the resource that is constrained here, because the program lives in RAM. The saving is paid for twice. |
| 2 | Multiply and divide | None / slow / fast / single cycle | Fast | When present, a multiply takes three cycles and a divide iterates in hardware. With no multiplier at all the compiler emits library functions and a multiply costs tens of cycles, which the bootloader's checksum computation and every timer computation cannot tolerate. The single-cycle option removes the three stall cycles but puts a wide multiplier array on the critical path, risking the frequency target in exchange for a benefit that no expected workload needs. |
| 3 | Bit manipulation | None / subsets | None | Leaving it out costs nothing here: none of the chosen libraries need it and the compiler does not emit it unless asked. Enabling it adds shift and mask hardware and would shorten some bitwise operations, but it is not part of the base architecture, so code that uses it would no longer build for a plain RV32I target. |
| 4 | Compressed instructions | Zca / Zca+Zcb / Zca+Zcb+Zcmp | Zca | Zca provides the standard 16-bit encodings and typically cuts code size by about a quarter, which directly buys more space for the application. Without it every instruction is four bytes. The larger subsets compress more - byte and half-word forms, plus push and pop sequences for function entry and exit - and since code size is the deciding constraint they deserve measurement rather than dismissal. They were not chosen for the first version because they make the decoder larger and the reference configuration for this size class uses Zca only. This version chooses Zca: Zca matches the rv32imc architecture string the toolchain emits by default, so no new compiler is needed and nobody has to check the support level. At 28nm the larger decoder is no longer the reason; the real reason to defer is toolchain risk |
| 5 | Register file style | Flip-flop / latch | Flip-flop | Flip-flops synthesise on every technology and on FPGA with no special handling. A latch-based register file is smaller for ASIC but needs a clock-gating structure that the technology may not support, and it does not map into FPGA fabric at all, so it would have to be swapped back for prototyping. |
| 6 | Third pipeline stage | Yes / no | No | Without it the register write happens in the second stage and the core keeps a depth of two stages, which means it is smaller and has a shorter branch penalty. With it the result of a data read can be forwarded while the next instruction executes, raising throughput on read-heavy code, in exchange for area and one more stage of forwarding logic. Not worth it for a microcontroller running peripheral drivers. |
| 7 | Branch target adder | Yes / no | No | When present, it computes the branch target address in a separate adder and removes one cycle for taken branches, in exchange for a second adder. When absent, taken branches pay that cycle. The reference configuration for this size class leaves it out and this design follows, because there is no measurement yet to justify the extra area. |
| 8 | Branch predictor | Yes / no | No | When absent, every taken branch empties the prefetch buffer. When present, correctly predicted branches avoid that bubble, but each misprediction costs more than the bubble it replaces, and the storage for the prediction state is significant relative to a core this small. At this frequency the bubble is cheap. |
| 9 | Instruction cache | Yes / no | No | There is nothing useful to cache: the program lives in on-chip RAM and is reachable in a few cycles, so a cache would only add tag memory, data memory and a fill state machine to shorten an access that is already short. It would also make instruction fetch timing variable, complicating the boot and debug behaviour described elsewhere. |
| 10 | Security hardening | Yes / no | No | Without it the core is a single copy of the logic. With it the whole core is duplicated and the two copies are compared every cycle, and memory error correction is enabled as well, which widens both memory interfaces from 32 bits to 39 and forces every memory in the system to carry those extra bits. That is a system-wide change for a threat model this project does not have. |
| 11 | Memory protection | Yes / no | No | Disabled. The wrapper instantiates the core with PMPEnable = 0, so the debug window is protected by convention rather than by hardware. Enabling it would cost three of the four checkers and would require the wrapper parameter to change with it |
| 12 | Number of protection regions | Up to 16 | 0 | Follows from row 11. The parameter is left at its default and no region is instantiated |
| 13 | Protection granularity | 0 and up | - | Not applicable while protection is disabled |
| 14 | Debug support | Always present, not parameterisable | Yes | This is not a configuration choice: the core has no parameter that removes the debug part. The debug request pin, debug mode and the dcsr and dpc registers are always present. Only the number of triggers is selectable, see row 15 of this table. This row is kept in the table to state plainly that QSOC uses that debug part, not to record a decision. |
| 15 | Hardware breakpoints | 0 and up | 2 | Two triggers, one more than the core's default. A breakpoint can be placed in code that cannot be modified, including the ROM, and the second one is what lets a breakpoint still be set while the core is already stopped at the first |
| 16 | Performance counters | 0 to 10 | 0 | The cycle counter and the retired instruction counter are always available and are enough to measure program load time and boot time. Additional event counters would be useful for tuning cache behaviour and branch behaviour, neither of which exists here. |
| 17 | Privilege level | Both levels always present, not parameterisable | Software convention: machine only | This is not a configuration choice. The core declares user mode hard-wired in the misa register and there is no parameter that removes it; the mstatus.MPP field even resets to the user value, so software only has to write mstatus and execute mret to enter user mode. QSOC's choice to run in machine mode only is a software convention, which means the firmware has to uphold it itself, not a boundary enforced by hardware. Worth knowing if the firmware ever does enter user mode: on this core interrupts are still taken while in user mode regardless of the global enable bit. |
| 18 | Boot address | Any value aligned to 256 bytes | 0x0000_0000 | Points at the bottom of read-only memory so that the first instruction fetch lands in boot code. It must be aligned to 256 bytes - the core checks this - because the low byte places the post-reset entry point above the trap vector table. |
| 19 | Debug window base address and mask | Any window aligned to a power of two | 0x2000_0000, 4 KiB | It is anchored to the bottom of instruction memory, so the value depends only on the base address. Anchored to the top it would be the base address plus the size minus the window, and every change of memory size would shift it - and since these are build-time parameters rather than registers, shifting one means rebuilding the core. |
| 20 | Debug halt and debug exception addresses | Any address within the window | 0x2000_0800 and 0x2000_0810 | The halt address is where the core fetches instructions when it is requested to stop. The exception address must be sixteen bytes away from it, not eight: the debug block places the resume entry point at plus eight, so the core's own default of plus eight would send an error raised during debug onto the resume path, silently taking the core out of debug mode and losing control of the device. |
| 21 | Hart identifier | Any value | 0 | There is only one core, so the value only has to be unique among cores. Zero is the common convention and lets published example code run as it is, with no modification. |

# Memory map

## Table of address map

Table 7-1. Complete address table

| No. | Start | End | Size | Block | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | 0x0000_0000 | 0x0000_07FF | 2 KiB | ROM | Boot code, read-only, executable |
| 2 | 0x0000_0800 | 0x1FFF_FFFF | - | Reserved | Unused |
| 3 | 0x2000_0000 | 0x2000_0FFF | 4 KiB | ISRAM (debug) | Debug window: halt address 0x2000_0800, exception address 0x2000_0810. Firmware must not place anything here |
| 4 | 0x2000_1000 | 0x2000_FFFF | 60 KiB | ISRAM (program) | The downloaded application program, executable |
| 5 | 0x2001_0000 | 0x2FFF_FFFF | - | Reserved | Unused, kept for ISRAM expansion |
| 6 | 0x3000_0000 | 0x3000_7FFF | 32 KiB | DSRAM | Stack, heap and variables |
| 7 | 0x3000_8000 | 0x3FFF_FFFF | - | Reserved | Unused, kept for DSRAM expansion |
| 8 | 0x4000_0000 | 0x7FFF_FFFF | - | Reserved | Unused |
| 9 | 0x8000_0000 | 0x8000_3FFF | 16 KiB | SCRC | System clock and reset control registers |
| 10 | 0x8000_4000 | 0x8000_7FFF | 16 KiB | SYSCSR | System status registers |
| 11 | 0x8000_8000 | 0x8000_BFFF | 16 KiB | WDT | Watchdog timer |
| 12 | 0x8000_C000 | 0x8000_FFFF | 16 KiB | GPIO0 | General purpose IO port 0 |
| 13 | 0x8001_0000 | 0x8001_3FFF | 16 KiB | GPIO1 | General purpose IO port 1 |
| 14 | 0x8001_4000 | 0x8001_7FFF | 16 KiB | GPIO2 | General purpose IO port 2 |
| 15 | 0x8001_8000 | 0x8001_BFFF | 16 KiB | GPIO3 | General purpose IO port 3 |
| 16 | 0x8001_C000 | 0x8001_FFFF | 16 KiB | TIMER0 | Plain counter, an apb_timer_unit instance |
| 17 | 0x8002_0000 | 0x8002_3FFF | 16 KiB | TIMER1 | Plain counter, an apb_timer_unit instance |
| 18 | 0x8002_4000 | 0x8002_7FFF | 16 KiB | UART0 | Serial port essential for boot |
| 19 | 0x8002_8000 | 0x8002_BFFF | 16 KiB | UART1 | General purpose serial port |
| 20 | 0x8002_C000 | 0x8002_FFFF | 16 KiB | SPI wrapper | SPI master at wrapper offset 0x0000-0x0FFF, reserved 0x1000-0x1FFF, SPI slave at 0x2000-0x3FFF |
| 21 | 0x8003_0000 | 0x8003_3FFF | 16 KiB | I2C | I2C controller |
| 22 | 0x8003_4000 | 0x8003_7FFF | 16 KiB | PWM | Advanced timer instance used for the PWM outputs |
| 23 | 0x8003_8000 | 0x8003_BFFF | 16 KiB | DMA config | Register interface of the DMA |
| 24 | 0x8004_0000 | 0xEFFF_FFFF | - | Reserved | Unused |
| 25 | 0xF000_0000 | 0xF000_000B | 12 B | SYSDBG regs (JTAG only) | Three registers - CTRL, STATUS, ID - decoded inside the debug block and never put on S_BUS, so software cannot halt the core or grant itself debug access. The crossbar must leave this region unmapped and answer DECERR; a slave here would shadow them. Everything up to 0xFFFF_FFFF is otherwise unmapped |
| 26 | - | - | - | INTMAP | Combinational interrupt logic. It has NO address: no register file, no APB interface, and it is not a bus slave. Decided in the design review of 18/09/2026 |

# Interrupt map

# System (SYS) CSR

The system status registers are the software-readable half of SCRC, the system clock and reset controller. Three registers answer three questions that firmware cannot answer any other way: why the chip restarted, which domains are currently held in reset, and which chip the program is running on. They report state and start nothing; every control bit lives in the control register block on the neighbouring port.

The block is generated from one register description rather than written by hand, so the field types and reset values in this chapter and the RTL come from the same source and cannot drift apart. It shares the peripheral bus clock and reset domain rather than owning one, which has a consequence worth stating: it cannot report on the domain it itself depends on.

Table 9-1. SYSCSR configuration

| Item | Value | Note |
| --- | --- | --- |
| Protocol | APB4 | Same peripheral bus as every other register block |
| Data width | 32 bits | Matches the peripheral bus |
| Address width | 16 bits | Three registers in one 16 KiB window |
| Base address | 0x8000_4000 | Port APB_M1, Table 5-4 |
| Clock and reset | Peripheral bus domain | Not an independent domain: it shares D02, so it cannot report on a domain it depends on |
| Generated or hand written | Generated from the register description | Field types and reset values come from one description, so the RTL and this table cannot drift apart |

Table 9-2 is the register definition as the generator consumes it, so it is the same table the RTL is built from, not a restatement of it.

Table 9-2. Register definition

| REGISTER | OFFSET | BIT NAME | FIELD WIDTH | BIT TYPE | RESET VALUE | DESCRIPTION |
| --- | --- | --- | --- | --- | --- | --- |
| resetcause | 0x0000 |  |  |  | 0x0 |  |
|  |  | cause_por | [0] | w1c | 0x0 | Set by Reset Manager when POR triggers a reset (i_hw_we_resetcause_cause_por=1, i_hw_wdata=1). SW writes 1 to clear (W1C) after reading. |
|  |  | cause_wdt | [1] | w1c | 0x0 | Set by Reset Manager when WDT triggers a reset (i_hw_we_resetcause_cause_wdt=1, i_hw_wdata=1). SW writes 1 to clear (W1C) after reading. |
|  |  | cause_soft | [2] | w1c | 0x0 | Set by Reset Manager when SOFT triggers a reset (i_hw_we_resetcause_cause_soft=1, i_hw_wdata=1). SW writes 1 to clear (W1C) after reading. |
|  |  | reserved | [31:3] | ro | 0x0 | Reserved. Read-as-zero, write ignore. |
| domainrststatus | 0x0004 |  |  |  | 0x0 |  |
|  |  | stat_sysbus | [0] | ro | 0x0 | 1 = System BUS domain held in reset, 0 = released. = ~o_rst_sysbus_n, driven directly by Reset Manager. |
|  |  | stat_perbus | [1] | ro | 0x0 | 1 = Peripheral BUS domain held in reset, 0 = released. = ~o_rst_perbus_n. |
|  |  | stat_wdt | [2] | ro | 0x0 | 1 = WDT domain held in reset, 0 = released. = ~o_rst_wdt_n. |
|  |  | stat_timer0 | [3] | ro | 0x0 | 1 = TIMER0 domain held in reset, 0 = released. = ~o_rst_timer0_n. |
|  |  | stat_timer1 | [4] | ro | 0x0 | 1 = TIMER1 domain held in reset, 0 = released. = ~o_rst_timer1_n. |
|  |  | stat_uart0 | [5] | ro | 0x0 | 1 = UART0 domain held in reset, 0 = released. = ~o_rst_uart0_n. |
|  |  | stat_uart1 | [6] | ro | 0x0 | 1 = UART1 domain held in reset, 0 = released. = ~o_rst_uart1_n. |
|  |  | stat_spi | [7] | ro | 0x0 | 1 = SPI (spi_host) domain held in reset, 0 = released. = ~o_rst_spi_n. |
|  |  | stat_i2c | [8] | ro | 0x0 | 1 = I2C domain held in reset, 0 = released. = ~o_rst_i2c_n. |
|  |  | stat_gpio | [9] | ro | 0x0 | 1 = GPIO domain (4 instances, shared) held in reset, 0 = released. = ~o_rst_gpio_n. |
|  |  | stat_dma | [10] | ro | 0x0 | 1 = DMA domain held in reset, 0 = released. = ~o_rst_dma_n. |
|  |  | stat_rom | [11] | ro | 0x0 | 1 = ROM domain held in reset, 0 = released. = ~o_rst_rom_n. |
|  |  | stat_ram | [12] | ro | 0x0 | 1 = RAM domain held in reset, 0 = released. = ~o_rst_ram_n. |
|  |  | stat_spi_device | [13] | ro | 0x0 | [PROPOSED] SPI_DEVICE domain held in reset, 0 = released. = ~o_rst_spi_device_n. Usage pending Hieu confirmation. |
|  |  | reserved | [15:14] | ro | 0x0 | Reserved. Read-as-zero, write ignore. |
| chipidrev | 0x0008 |  |  |  | 0x51534F43 |  |
|  |  | chip_id_rev | [31:0] | ro | 0x51534F43 | Fixed constant 'QSOC', tied at integration - not connected to the Clock/Reset Manager. No separate REV field. |

On resetcause. The three bits match the three reset sources fixed by the design review of 18/09/2026: power-on, watchdog and software. There is no bit for an external reset pin, because the pin is how power-on reset is implemented rather than a fourth source, and none for a debug-initiated reset, because QNSC_SYSDBG_MAS removed it. The bits are write-one-to-clear and set by hardware, so firmware reads the cause, writes the same value back, and the register is ready for the next restart. Without the clear step a later restart cannot be told apart from the one just handled. More than one bit can be set at once, so firmware must treat the register as a set of flags rather than a number.

On domainrststatus. Each bit is the inverse of the reset output that drives that domain, so the register is a wire view of the reset manager rather than a copy that could go stale. The order matches the bit assignment in QNSC_SCRC: bit 0 is the system bus, bit 13 is the SPI device block. Five domains have no bit and the reasons differ. The CPU and this register block itself cannot be soft-reset at all, so a status bit would report on something software cannot change. The debug domain and the IO multiplexer are not in the soft-reset list. The PWM domain is the one gap: it has a dedicated soft-reset bit in the control register but no matching status bit here, so software can reset it and cannot confirm the reset took effect.

On chipidrev. The value reads as the four ASCII characters of the project name with the most significant byte first. It is tied at integration and is not driven by the reset manager, so it is readable whatever state the rest of the chip is in - which is what makes it useful as the first register a bring-up script reads. There is no separate revision field: all thirty-two bits carry the identifier, so a silicon revision would need a register of its own.

Two properties of the register map are worth recording because they are easy to get wrong later. Bits 16 and above of domainrststatus are not described in Table 9-2 and are not covered by its reserved field, so their read value is undefined by this document; firmware must mask the register to its low sixteen bits rather than compare the whole word. And bit 14 of that register carried a hardware busy flag in an earlier revision of the register specification, which the design review rejected and its owner then removed; it is reserved here so that a future addition does not silently inherit the old meaning.

# DMA

The DMA moves blocks of data between memories without spending CPU cycles; without it every transfer is a copy loop in firmware, occupying the core and slowing interrupt response. Firmware configures it through one slave port on the peripheral bus, and data moves through one master port on the system bus; the paths are independent, so a transfer runs while the CPU works elsewhere. The block is the generated iDMA backend behind a frontend written for this chip, specified in QNSC_iDMA V2.0, and it merges what were three separate front-ends into one module: a register job mode, a descriptor chain mode and eight fixed peripheral channels.

Table 10-1. DMA interfaces

| Interface | Protocol | Role | Connects to |
| --- | --- | --- | --- |
| Configuration | APB4, 32-bit | Slave | P_BUS master port APB_M14 at 0x8003_8000. One port serves all three engines through a unified register block |
| Data read | AXI4 | Master | Generated backend read port, into the 3:1 multiplexer |
| Data write | AXI4 | Master | Generated backend write port, into the 3:1 multiplexer |
| Descriptor fetch | AXI4, read only | Master | The descriptor engine fetches its own chain, into the same multiplexer |
| System bus port | AXI4 | Master | S_BUS slave port AXI_S2, identifier width 7. One physical port after the multiplexer |
| Peripheral request | Handshake | Slave | Eight fixed channels. Six wired in RTL (UART0, UART1, I2C); the two SPI channels are allocated but not connected |
| Interrupt | Level | Master | One line dma_irq_o into INTMAP on fast line 0 |

Three master ports have to become one: the backend read port, the backend write port, and the read port the descriptor engine uses to fetch its own chain. A crossbar port for each would cost three decoders, three error responders and three matrix rows, and identifier width does not decide it because the crossbar widens identifiers the same either way. They are merged by a 3:1 multiplexer taken from the same IP family, which costs one port's bandwidth when two engines want the bus at once.

Neither stock front-end could be used, and the reason is worth recording because it is not a preference. The descriptor front-end of the source IP has its registers hardwired to a 64-bit peripheral bus while QSOC's is 32-bit, and it depends on a generated register package that is not in the repository; the register front-end has no interrupt output at all, only identifiers for firmware to poll. So the frontend is written for this chip and is 32-bit native, while the data backend is still generated by the IP's own tool rather than written by hand. Configuration stays on the peripheral bus, which is what the address map, the peripheral allocation and the interrupt map all rest on.

Table 10-2. DMA configuration

| No. | Feature | Option | QSOC | Description and reason |
| --- | --- | --- | --- | --- |
| 1 | Data width | Any power of two | 32 bit | Matches the system bus. A wider setting would need packing logic at every 32-bit endpoint |
| 2 | Address width | Any | 32 bit | Must cover the whole address space, otherwise there are regions the DMA cannot reach |
| 3 | Front-end | reg / desc64 / in-house | In-house, unified | Neither stock front-end is usable: desc64 has its registers hardwired to a 64-bit APB while QSOC's is 32-bit, and reg has no interrupt output at all. The three modes are merged into one module instead of three behind an arbiter that was never built |
| 4 | Operating modes | One or several | Three: register job, descriptor chain, eight peripheral channels | One register block and one arbiter serve all three, so no top-level arbiter is needed |
| 5 | Backend | Hand written / generated | Generated | Produced by the IP's own util/gen_idma.py rather than written by hand, so it stays comparable with upstream |
| 6 | Number of dimensions | 1D / 2D / 3D | 1D | One dimension is enough for contiguous block moves |
| 7 | Error handling | None / report / report and wait | Report and wait | On a bus error the transfer stops and the cause stays readable in the status register |
| 8 | Completion interrupt | Present / absent | Present, one line | dma_irq_o is error_irq OR (IRQ_MODE AND completion_irq), both built from DMA_ISR flags. It replaces the eleven per-peripheral lines of the earlier design |
| 9 | Interrupt shape | Pulse / level | Level, held | DMA_ISR flags are write-1-to-clear, so the line stays asserted until firmware clears the flag. This is what keeps the DMA out of the pulse-loss case in Table 8-3 |
| 10 | Port count on the system bus | One or several | One | Read, write and descriptor fetch are merged by a 3:1 multiplexer, so the crossbar spends one port rather than three |

Firmware writes the source address, the destination address and the length over the peripheral bus, then starts the transfer. The block issues read bursts from the source and write bursts to the destination for the full length, then sets the matching flag in its status register and raises dma_irq_o if that flag is enabled. Because the flags are write-1-to-clear, the handler learns which event fired by reading the register and clears it by writing back, so one interrupt line still distinguishes a completion from an error and identifies which of the eight peripheral channels was involved.

Accessible regions are ROM, SRAM and peripherals; an unmapped address returns a decode error. A write into ROM is neither errored nor ignored: the array write enable fires while the bus response is a hardcoded OKAY, so the write lands and the bus reports success - a silent failure. Return SLVERR for a write into ROM. Only the DMA bursts, so the peripheral bridge splits them into single cycles.

Two limits are stated by the block owner and belong in this report. The RTL of the new frontend has been reviewed by hand only and has not been simulated, so every interface width and field name in this chapter comes from a reading of the source rather than from a passing testbench. And two of the eight peripheral channels, the SPI transmit and receive pair, are allocated but not yet connected, because the SPI blocks sit in a vendored tree outside the scope of that work. Neither affects the interrupt count, which is one line in both cases.

# IO PAD

## Function Select Encoding

## Pad List

# Operation mode

Three modes describe the chip as a whole. The core's own states - machine mode, sleep and debug mode - are a different unit and are in Table 6-3; privilege level is not an operating mode and is in Table 6-4 row 17. Sleep is not a chip mode either: the core stops its own clock, but QSOC has no power manager, so no domain is switched off and the chip stays in normal operation.

Table 12-1. Operation modes

| Mode | Entered by | Left by | What is running |
| --- | --- | --- | --- |
| Reset | Any of the three reset sources: power-on, watchdog bite, or the software reset bit | The clock is qualified, then SCRC releases the domains in order, and last the core's fetch enable is asserted | SCRC only. Every other domain is held in reset, so the buses do not respond |
| Normal | The first instruction fetch after the fetch enable is asserted | Any reset source, or a debug entry | The ROM bootloader first, then the downloaded application. The core may stop its own clock with the wait-for-interrupt instruction without leaving this mode, because QSOC has no power manager and the rest of the chip keeps running |
| Debug | A request from the debug block over JTAG, a breakpoint instruction, or a hardware trigger match | The debug return instruction, which is only reached after the two ordered writes described below | The core executes only what the debug block places in the debug window. Peripherals and the buses keep running, so a watchdog left enabled still counts |

On leaving reset. The core has a fetch enable input that gates every instruction request, so releasing a domain's reset is not by itself enough to start the program. This fixes part of the boot order: the ROM domain must be running before the fetch enable is asserted, otherwise the first fetch is issued at a memory that cannot answer. Which block drives that signal, and at which step, is not yet assigned - the CPU wrapper exposes it as an input and no specification claims it.

On resuming from debug. The resume request alone does not release the core, because debug mode is left only by the debug return instruction and the core is sitting in a dispatch loop rather than executing one. Two writes are needed and their order is fixed: first clear the debug request so it is no longer asserted, then set the resume flag in the debug window. In the other order the core executes the return, still sees the request asserted, and halts again at the next instruction boundary - which the host sees as a resume that was ignored, with nothing explaining why.

# Boot flow