QUY NHON SEMICONDUCTORS – QNSC

RTL Design Naming Rule

This document defines mandatory naming rules for RTL design. The rules apply to modules, instances, ports, internal signals, constants, and integration interfaces. Exceptions shall be explicitly reviewed and agreed before implementation.

V1.2

Contents

Contents	Error! Bookmark not defined.

1.	GENERAL DESIGN PRINCIPLES	6

1.1.	Identifier Format	6

1.2.	Port Direction Prefix	6

1.3.	Active-Low Naming	7

1.4.	Numeric Instance and Channel Naming	7

1.5.	Standard Vocabulary	7

2.	MODULE, INSTANCE, AND INTERNAL RTL NAMING	10

2.1.	Module Naming	10

2.2.	Instance Naming	10

2.3.	Internal Signal Naming	11

2.4.	Parameter, Constant, and State Naming	11

2.5.	Memory Naming	12

3.	PORT AND INTERFACE NAMING	13

3.1.	Clock	13

3.2.	Reset	13

3.3.	APB	13

3.4.	AXI	14

3.4.1.	AW Channel	14

3.4.2.	W Channel	14

3.4.3.	B Channel	14

3.4.4.	AR Channel	14

3.4.5.	R Channel	14

3.5.	CSR	14

3.6.	Interrupt	15

3.7.	GPIO	15

3.8.	Pad	15

3.9.	Memory	16

3.10.	DMA	16

3.11.	JTAG	16

3.12.	Debug	17

3.13.	Boot	17

3.14.	Power	17

3.15.	DFT	18

3.16.	Analog	18

3.17.	Test	18

4.	COMPLETE EXAMPLES AND ENFORCEMENT	19

4.1.	Complete QNSC Wrapper Example	19

4.2.	Complete Naming Example	19

4.3.	Enforcement Checklist	20

Revision

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 15/09/2026 | Tam Nguyen | Initial version |
| 1.1 | 28/09/2026 | Nghia VT | 2.1 and 4.3: a wrapper is m_qnsc_wrap_<ip>, the IP's short name in the chip contract (m_qnsc_wrap_uart, m_qnsc_wrap_isram), not the vendor module name |
| 1.2 | 01/10/2026 | Nghia VT | 2.1: chip top m_qnsc_top / m_qnsc_chip, nested wrapper, packages qnsc_<function>_pkg. 2.3: function locals and struct fields. New 2.6 type, 2.7 file, 2.8 technology cell. 3.1/3.2: role as domain in generic modules. 3.3/3.4: buses with several ports, structs inside a block. 4.3: checklist. Examples qnsc_clk_gate, qnsc_rst_sync, u_rst_sync follow 1.5 |

GENERAL DESIGN PRINCIPLES

Identifier Format

All RTL identifiers shall:

- Use lowercase English letters.

- Use digits where required.

- Use underscore _ as the only word separator.

- Not use CamelCase, PascalCase, or mixed-case naming.

- Use meaningful and consistent terminology throughout the project.

Examples

| Valid | Invalid |
| --- | --- |
| m_qnsc_timer | M_QNSC_TIMER |
| i_bus_apb_paddr | iApbAddr |
| o_int_timer_0 | o_irq_timer0 |
| i_rst_n_sys | rstn_sys |
| u_timer_0 | uTimer0 |

Port Direction Prefix

All module ports shall use the following prefixes:

| Prefix | Meaning | Example |
| --- | --- | --- |
| i_ | Input | i_clk_sys |
| o_ | Output | o_int_timer_0 |
| io_ | Bidirectional | io_i2c_sda |

The direction prefix shall always be placed at the beginning of the port name.

Active-Low Naming

Active-low signals shall contain _n immediately after the signal meaning.

Examples:

i_rst_n_sys

i_rst_n_cpu

o_rst_n_peri

_n shall not be replaced by inconsistent forms such as:

rstn

resetn

reset_b

rst_bar

Numeric Instance and Channel Naming

Numeric instances or channels shall use an underscore before the index.

Valid

timer_0

timer_1

uart_0

uart_1

dma_0

dma_1

Invalid

timer0

timer1

uart0

dma0

Standard Vocabulary

Project-wide standard vocabulary shall be used consistently.

| Function | Mandatory terminology |
| --- | --- |
| Interrupt | int |
| Power | pwr |
| Design-for-Test | dft |
| Analog | ana |
| Multiplexer | mux |
| Clock | clk |
| Reset | rst |
| Configuration | cfg |
| Debug | dbg |
| Test | test |
| Boot | boot |
| Memory | mem |
| Peripheral | peri |
| UART | uart |
| SPI | spi |
| I2C | i2c |
| GPIO | gpio |
| DMA | dma |

Standard protocol vocabulary shall be preserved, including:

paddr pwdata prdata pready psel penable pwrite awaddr awvalid awready wdata wstrb wvalid wready bresp bvalid bready araddr arvalid arready rdata rresp rvalid rready

MODULE, INSTANCE, AND INTERNAL RTL NAMING

Module Naming

QNSC functional modules shall use:

m_qnsc_<function>

Examples:

m_qnsc_timer

m_qnsc_dma

m_qnsc_gpio

m_qnsc_uart

QNSC wrapper modules shall use:

m_qnsc_wrap_<ip>

where <ip> is the short name of the IP as the chip contract names it, without an index, not the name of the vendor module. The name stays the same when the IP is replaced. Two configurations of one IP are two IPs in the contract, each with its own wrapper.

Examples:

m_qnsc_wrap_cpu

m_qnsc_wrap_uart

m_qnsc_wrap_timer

m_qnsc_wrap_isram

m_qnsc_wrap_dsram

Generic modules shall use:

qnsc_<function>

Examples:

qnsc_fifo_sync

qnsc_arbiter

qnsc_clk_gate

qnsc_rst_sync

The chip top shall use:

m_qnsc_top

m_qnsc_chip

m_qnsc_top instantiates every block wrapper; m_qnsc_chip is m_qnsc_top plus the pad ring. Both are defined in design/top/rtl/ and nowhere else. The top of a block is its wrapper, m_qnsc_wrap_<ip>; no _top suffix is added.

A wrapper nested inside a block wrapper, around one IP of a block built from several, shall use:

m_qnsc_wrap_<ip>_<part>

Examples:

m_qnsc_wrap_cpu_ibex

m_qnsc_wrap_cpu_cpu2axi

Packages shall use:

qnsc_<function>_pkg

Examples:

qnsc_pkg

qnsc_cpu2axi_pkg

Packages share one namespace with every vendored package (axi_pkg, tlul_pkg); the prefix keeps ours apart. qnsc_pkg is the package generated from the chip contract.

Instance Naming

Module instances shall use:

u_<function>_<index>

Examples:

u_timer_0

u_timer_1

u_uart_0

u_dma_0

u_fifo_0

For a single non-indexed instance:

u_timer

u_cdc

u_rst_sync

Internal Signal Naming

Internal registered signals shall use:

r_<function>

Internal combinational signals shall use:

w_<function>

Examples:

r_timer_count

r_timer_enable

r_state

w_timer_done

w_timer_match

w_next_state

Example

reg  [31:0] r_timer_count;

wire        w_timer_done;

A variable local to a function or a task, and a field of a struct, is not an internal signal and takes no prefix.

Parameter, Constant, and State Naming

Parameters shall use:

P_<FUNCTION>

Examples:

P_DATA_WIDTH

P_ADDR_WIDTH

P_DEPTH

Local constants shall use:

C_<FUNCTION>

Examples:

C_IDLE

C_BUSY

C_ERROR

FSM states shall use:

S_<STATE>

Examples:

S_IDLE

S_WAIT

S_WRITE

S_READ

Example:

localparam P_DATA_WIDTH = 32;

localparam C_IDLE  = 2'd0;

localparam C_BUSY  = 2'd1;

Memory Naming

Memory arrays shall use:

mem_<function>

Examples:

mem_data

mem_tag

mem_fifo

mem_instruction

Type Naming

Types shall use:

<function>_t

Examples:

state_t

addr_t

axi_req_t

The members of an enum are constants or FSM states and follow 2.4:

typedef enum logic [1:0] {S_IDLE, S_BUSY, S_DONE} state_t;

File Naming

Each file shall hold one module, package or interface, and shall be named after it:

m_qnsc_wrap_uart.sv     module m_qnsc_wrap_uart

qnsc_cpu2axi_pkg.sv     package qnsc_cpu2axi_pkg

The source of a wrapper generated with emacs is <module>.src.sv. Simulation, synthesis and connectivity flows take the top of a block from the file name, so a file named otherwise selects the wrong top.

Technology Cell Naming

RTL shall not instantiate a cell of a technology library (standard cell, I/O pad, memory macro) directly. It instantiates a generic module, which each technology implements with the same name and ports:

qnsc_<function>

Examples:

qnsc_clk_gate

qnsc_clk_buf

qnsc_clk_inv

Inside an implementation, the library cell keeps its own name and pins, and its instance shall use:

u_size_only_<function>

so that synthesis can preserve it by name. How a technology is added and chosen: design/common/tech/README.md.

PORT AND INTERFACE NAMING

Clock

Clock ports shall use:

i_clk_<domain>

o_clk_<domain>

Examples:

i_clk_sys

i_clk_cpu

i_clk_peri

o_clk_peri

In a generic module (qnsc_<function>), which belongs to no domain, <domain> names the role of the clock: i_clk_src, i_clk_dst, o_clk_gated. A port named i_clk alone is not allowed.

Reset

Reset ports shall use:

i_rst_n_<domain>

o_rst_n_<domain>

Examples:

i_rst_n_sys

i_rst_n_cpu

i_rst_n_peri

o_rst_n_peri

Every reset port is active low. In a generic module, <domain> names the role, as for clocks: i_rst_n_dst.

APB

APB ports shall use:

i_bus_apb_<signal>

o_bus_apb_<signal>

Examples:

i_bus_apb_paddr

i_bus_apb_psel

i_bus_apb_penable

i_bus_apb_pwrite

i_bus_apb_pwdata

o_bus_apb_prdata

o_bus_apb_pready

o_bus_apb_pslverr

A module with several APB ports names the port after the protocol, as the chip contract names it (APB_M8 is m_8):

o_bus_apb_m_8_psel

i_bus_apb_m_8_prdata

AXI

AXI naming shall be normalized by individual channel and signal.

The format shall be:

i_bus_axi_<channel>_<signal>

o_bus_axi_<channel>_<signal>

AW Channel

i_bus_axi_aw_addr

i_bus_axi_aw_valid

o_bus_axi_aw_ready

W Channel

i_bus_axi_w_data

i_bus_axi_w_strb

i_bus_axi_w_valid

o_bus_axi_w_ready

B Channel

o_bus_axi_b_resp

o_bus_axi_b_valid

i_bus_axi_b_ready

AR Channel

i_bus_axi_ar_addr

i_bus_axi_ar_valid

o_bus_axi_ar_ready

R Channel

o_bus_axi_r_data

o_bus_axi_r_resp

o_bus_axi_r_valid

i_bus_axi_r_ready

A module with several AXI ports names the port after the protocol, as the chip contract names it (AXI_S0 is s_0, AXI_M1 is m_1):

i_bus_axi_s_0_aw_addr

o_bus_axi_m_1_aw_addr

A module inside a block may carry a bus as one packed struct per direction: o_bus_axi_req, i_bus_axi_rsp (and the same for APB). The ports of a wrapper stay one per signal, as above.

CSR

CSR ports shall use:

i_csr_<function>

o_csr_<function>

Examples:

i_csr_timer_enable

i_csr_timer_prescaler

o_csr_timer_status

o_csr_timer_count

Interrupt

Interrupt ports shall use:

i_int_<source>

o_int_<source>

Examples:

o_int_timer_0

o_int_uart_0

o_int_dma_0

i_int_external

i_int_software

i_int_timer

GPIO

GPIO ports shall use:

i_gpio_<function>

o_gpio_<function>

io_gpio_<function>

Examples:

i_gpio_uart_rx

o_gpio_spi_sclk

o_gpio_i2c_sda

Pad

Pad ports shall use:

i_pad_<function>

o_pad_<function>

io_pad_<function>

Examples:

i_pad_usi_0

o_pad_can_sel

io_pad_i2c_scl

Memory

Memory ports shall use:

i_mem_<function>

o_mem_<function>

Examples:

o_mem_addr

o_mem_wdata

o_mem_we

o_mem_be

i_mem_rdata

DMA

DMA ports shall use:

i_dma_<function>

o_dma_<function>

Examples:

o_dma_req

i_dma_gnt

i_dma_rvalid

o_dma_addr

o_dma_wdata

i_dma_rdata

JTAG

JTAG ports shall use:

i_jtag_<function>

o_jtag_<function>

Examples:

i_jtag_tck

i_jtag_tms

i_jtag_tdi

o_jtag_tdo

Debug

Debug ports shall use:

i_dbg_<function>

o_dbg_<function>

Examples:

i_dbg_req

i_dbg_enable

o_dbg_ack

o_dbg_halted

Boot

Boot ports shall use:

i_boot_<function>

o_boot_<function>

Examples:

i_boot_addr

i_boot_sel

o_boot_done

Power

Power-related ports shall use:

i_pwr_<function>

o_pwr_<function>

Examples:

i_pwr_sleep

i_pwr_isolation

o_pwr_good

DFT

DFT ports shall use:

i_dft_<function>

o_dft_<function>

Examples:

i_dft_scan_en

i_dft_bypass

o_dft_scan_out

Analog

Analog ports shall use:

i_ana_<function>

o_ana_<function>

io_ana_<function>

Examples:

i_ana_comp_out

o_ana_trim

io_ana_test

Test

Test-related ports shall use:

i_test_<function>

o_test_<function>

Examples:

i_test_en

i_test_mode

o_test_status

COMPLETE EXAMPLES AND ENFORCEMENT

Complete QNSC Wrapper Example

module m_qnsc_wrap_timer (

input  wire        i_clk_sys,

input  wire        i_rst_n_sys,

input  wire [15:0] i_bus_apb_paddr,

input  wire        i_bus_apb_psel,

input  wire        i_bus_apb_penable,

input  wire        i_bus_apb_pwrite,

input  wire [31:0] i_bus_apb_pwdata,

output wire [31:0] o_bus_apb_prdata,

output wire        o_bus_apb_pready,

output wire        o_bus_apb_pslverr,

output wire        o_int_timer_0

);

Internal RTL:

reg  [31:0] r_timer_count;

reg         r_timer_enable;

wire        w_timer_done;

wire        w_timer_match;

localparam P_DATA_WIDTH = 32;

localparam C_IDLE = 2'd0;

localparam C_BUSY = 2'd1;

reg [1:0] r_state;

Instance:

timer u_timer_0 (

...

);

Complete Naming Example

A timer module integrated through APB shall follow:

Module

m_qnsc_wrap_timer

Clock

i_clk_sys

Reset

i_rst_n_sys

APB

i_bus_apb_paddr

i_bus_apb_psel

i_bus_apb_penable

i_bus_apb_pwrite

i_bus_apb_pwdata

o_bus_apb_prdata

o_bus_apb_pready

o_bus_apb_pslverr

Interrupt

o_int_timer_0

Internal registered signal

r_timer_count

Internal combinational signal

w_timer_done

Parameter

P_DATA_WIDTH

Constant

C_IDLE

Instance

u_timer_0

Enforcement Checklist

Before RTL is committed, the following rules shall be satisfied:

| Check | Mandatory requirement |
| --- | --- |
| Identifier case | All identifiers shall be lowercase |
| Word separator | Words shall use _ |
| Port direction | Ports shall use i_, o_, or io_, matching the direction |
| QNSC module | Shall use m_qnsc_<function> |
| QNSC wrapper | Shall use m_qnsc_wrap_<ip> |
| Instance | Shall use u_<function>_<index> where indexed |
| Registered signal | Shall use r_<function> |
| Combinational signal | Shall use w_<function> |
| Parameter | Shall use P_<FUNCTION> |
| Constant | Shall use C_<FUNCTION> |
| FSM state | Shall use S_<STATE> |
| Numeric instance | Shall use timer_0, not timer0 |
| AXI | Shall be normalized by channel and signal |
| Pad | Shall use i_pad_, o_pad_, io_pad_ |
| Power | Shall use pwr |
| DFT | Shall use dft |
| Analog | Shall use ana |
| Mux | Shall use mux |
| Third-party IP | Shall be normalized at wrapper boundary |
| Chip top | Shall be m_qnsc_top or m_qnsc_chip, in design/top/rtl/ |
| Package | Shall use qnsc_<function>_pkg |
| Type | Shall use <function>_t; enum members S_<STATE> or C_<FUNCTION> |
| File | One module, package or interface per file, named after it |
| Clock and reset | Shall use i_clk_<domain> and i_rst_n_<domain> |
| Bus with several ports | Shall use <i¦o>_bus_<protocol>_<port>_..., <port> as in the contract |
| Technology cell | Shall be instantiated through qnsc_<function>; the library cell only inside, as u_size_only_<function> |