# BOOT

BOOT FLOW SPECIFICATION – V3.4

QUY NHON SEMICONDUCTORS – QNSC

# Table of Contents

# Table of Tables

Table 3-1. Memory map, regions behind APB_M1 and APB_M8 and AXI_M0 and AXI_M1 and AXI_M2

Table 4-1. Boot stages

Table 4-2. Steps of Figure 4-1

Table 5-1. UART0 registers used, base 0x8002_0000

Table 6-1. Frame fields

Table 7-1. Steps of Figure 7-1

Table 7-2. Tokens, 4 ASCII bytes, sent by the ROM

Table 7-3. Timeouts

Table 8-1. Limits, enforced by the build

Table 8-2. Source, design/rom/boot/

Table 9-1. Debug program layout, fixed by the Ibex debug parameters

Table 10-1. Requirements on other owners

Table A-1. Acronyms

Table B-1. First review

# Table of Figures

Figure 3-1. Blocks on the boot path

Figure 4-1. Reset to application

Figure 6-1. Boot frame; every field little-endian

Figure 7-1. Bootloader flow

Figure 7-2. PC loader and boot ROM

# Revision history

The reasoning behind each change, the V1.0–V2.2 history and the V2.2 text are in QNSC_BOOT_DECISIONS.md.

| Version | Date | Author | Reviewer | Description of change |
| --- | --- | --- | --- | --- |
| V3.4 | 2026-10-14 | Nguyen Hao Nam | – | Mentor feedback of 14/10 (Appendix B): the ROM loads the debug program when DEBUG_PROG is 0x0000_0055 and skips it for any other value; 0xAA is no longer checked, so FHDR is for LENGTH only; Figures 6-1 and 7-1, Tables 6-1, 7-1, 7-2 and 11 with them |
| V3.3 | 2026-10-12 | Nguyen Hao Nam | – | Mentor’s debug boot flow of 12/10 (Appendix B): the ROM loads the debug program too, over UART0; JTAG no longer loads the image or holds the CPU, and the boot address mux is gone, so both modes run the ROM; header 16 bytes with DEBUG_PROG (0x55: a 4 096-byte debug program and its DBG_CRC follow the header and go to 0x2000_0000; 0xAA: none); tokens ACKD and FDCR; after ACKP the ROM waits while SYSCSR DBG_BOOT.dbg_en = 1, which the DBG_EN pin sets and the host clears over JTAG; a debug request during the wait enters the debug program at DmHaltAddr 0x2000_0800 and dret returns to the wait; the debug program follows the SYSDBG debug window layout (9); Figures 4-1, 6-1, 7-1, 7-2 redrawn and Tables 4-2, 7-1 with them |
| V3.2 | 2026-10-09 | Nguyen Hao Nam | – | Mentor review of V3.1 (Appendix B): ENTRY dropped from the header, for the same entry as debug boot; LOAD_ADDR dropped with it, so the payload always goes to 0x2000_1000 and the ROM jumps to 0x2000_1080; header 12 bytes; LENGTH 132 to 61 440; FUAR dropped and the UART error bits no longer read, since a CRC or the timeout catches every UART error; Figure 4-1 redrawn (two SCRC steps, the DBG_EN pin, no CRC test) and Table 4-2 with it |
| V3.1 | 2026-09-30 | Nguyen Hao Nam | – | Mentor review 29/09 (Appendix B): no QRDY after reset, only after a failure; 19 200 baud; Figure 4-1 shows the CPU release and tests CRC pass; debug boot explained by the boot address mux; steps of Figures 4-1 and 7-1 described (Tables 4-2, 7-1); how each failure is detected; timeouts given as minimums; UART errors before MAGIC get no reply; Figure 7-1 redrawn with one decision per check; the PC stops on a failure token; RX_TIMEOUT and DRAIN_IDLE raised |
| V3.0 | 2026-09-28 | Nghia VT (lead), for Nguyen Hao Nam | – | V2.2 moved onto the template: UART0 address corrected to 0x8002_0000, hardware sequencing referenced to SCRC/SYSDBG, bootloader image limits moved here from the ROM, timing recomputed, monochrome figures |

# 1. Overview

QSOC has no Flash. After every reset the boot ROM downloads the application from a PC into ISRAM over UART0, checks it, and jumps to it. In debug boot it also downloads a debug program, and waits for the debugger before it jumps. This specification is the bootloader, the frame and handshake between it and the PC loader, and what the application can rely on.

It is not a block and has no RTL. The hardware it runs on is QNSC_ROM_MAS, QNSC_SCRC_MAS (reset and release order), QNSC_SYSCSR_MAS (DBG_BOOT, the DBG_EN pin) and QNSC_SYSDBG_MAS (the JTAG access to the chip).

Source design/rom/boot/, output rom_image.hex; owner Nguyen Hao Nam.

# 2. Features

- The same flow after every reset, in both modes: power-on, watchdog, software – 4.

- UART0 at 19 231 baud (19 200 nominal) 8N1, polled – 5.

- Frame: 16-byte header with its own CRC; an optional 4 096-byte debug program with its own CRC; payload, payload CRC – 6.

- A 4-byte token answers every step; any failure returns to QRDY – 7.

- Nothing written to RAM before the header is checked – 7.1.

- Application up to 60 KiB, loaded at 0x2000_1000 and entered at 0x2000_1080; debug program up to 4 KiB, loaded at 0x2000_0000 – 9.

- Debug boot: with the DBG_EN pin at 1 the ROM waits after ACKP until the host releases it over JTAG; the CPU runs the whole time – 7.5.

# 3. Boot path

Figure 3-1. Blocks on the boot path

Table 3-1. Memory map, regions behind APB_M1 and APB_M8 and AXI_M0 and AXI_M1 and AXI_M2

| Base | Size | Region | Port | Kind | Note |
| --- | --- | --- | --- | --- | --- |
| 0x00000000 | 2 KiB | rom | AXI_M0 | memory | Boot code, read-only, executable |
| 0x20000000 | 4 KiB | isram_dbg | AXI_M1 | memory | The debug window: the first 4 KiB of ISRAM, not a block of its own |
| 0x20001000 | 60 KiB | isram | AXI_M1 | memory | The downloaded application |
| 0x30000000 | 32 KiB | dsram | AXI_M2 | memory | Stack, heap and variables |
| 0x80004000 | 16 KiB | syscsr | APB_M1 | peripheral |  |
| 0x80020000 | 16 KiB | uart_0 | APB_M8 | peripheral | Carries the boot download |

The bootloader runs from the ROM, keeps its stack in DSRAM, writes ISRAM only from 0x2000_1000, or from 0x2000_0000 when the frame carries a debug program, and accesses no peripheral other than UART0 and one register of SYSCSR, DBG_BOOT, which it only reads.

# 4. Sequence

Figure 4-1. Reset to application

Table 4-1. Boot stages

| Stage | Done by | Specified in | Ends when |
| --- | --- | --- | --- |
| 1. Chip reset | SCRC RRC | QNSC_SCRC_MAS 7.3 | chip reset releases |
| 2. Power-up | SCRC MCPU | QNSC_SCRC_MAS 7.5 | CPU released; UART0 clocked and reachable |
| 3. ROM start | Ibex, crt0.S | QNSC_ROM_MAS 6, section 8 | boot_main called |
| 4. Download | bootloader | section 7 | every CRC passes, ACKP sent |
| 5. Debug wait | bootloader, host | 7.5 | DBG_BOOT.dbg_en reads 0 |
| 6. Jump | bootloader | 7.5 | first instruction at 0x2000_1080 |

Both modes run the same stages. The DBG_EN pin does not change where the CPU starts and does not hold it in reset: the CPU always starts in the ROM, and the pin only makes the ROM wait at stage 5. SYSCSR passes the pin through two flip-flops into DBG_BOOT.dbg_en, which the ROM reads; the host clears it by writing DBG_BOOT.dbg_en_mask = 1 over JTAG (QNSC_SYSCSR_MAS 7.5). The debug program, if the frame carries one, is in ISRAM from 0x2000_0000 before the wait starts, so the host can halt the CPU into it at any time: Ibex enters it at DmHaltAddr, 0x2000_0800.

Table 4-2. Steps of Figure 4-1

| Step of Figure 4-1 | What happens | Specified in |
| --- | --- | --- |
| Power on | The power-on reset. A WDT bite or SW_RST resets the chip too and enters at the next step. | QNSC_SCRC_MAS 7.3 |
| SCRC: hold the whole chip in reset, clock the IPs | The RRC holds the chip in reset: 16 cycles after a WDT bite or SW_RST; after POR it releases one cycle later. Then MCPU runs the CRM program, which first starts the clocks of the peripherals that CLK_EN asks for: after reset, every peripheral except TIMER1. | QNSC_SCRC_MAS 7.3, 7.5 |
| SCRC: release the reset of every IP except the CPU | At least 3 cycles later MCPU releases the reset of every domain except the CPU. At least 16 cycles after that it opens the APB guards of the peripherals whose clock runs, the TIMER1 guard staying closed, so UART0 is reachable before the CPU runs. Register values: QNSC_SCRC_MAS 7.5. | QNSC_SCRC_MAS 7.5 |
| SCRC releases the CPU reset | Last, MCPU releases the CPU reset, in both modes. | QNSC_SCRC_MAS 7.5 |
| Ibex fetches 0x0000_0080 (ROM) | Ibex starts at its boot address + 0x80: 0x0000_0080 in the ROM. The start-up code points the stack at the top of DSRAM, 0x3000_8000, and calls the bootloader, boot_main. | QNSC_ROM_MAS 6, section 8 |
| init UART0 | Sets 19 200 baud, 8 data bits, no parity and 1 stop bit, turns the FIFOs on and the interrupts off (section 5). The ROM starts listening for MAGIC at once. | 5 |
| receive the frame from the PC over UART0: DEBUG_PAYLOAD to 0x2000_0000 (only when DEBUG_PROG = 0x55), PAYLOAD to 0x2000_1000 | Receive and check the frame, Figure 7-1 and Table 7-1. The step ends when the header CRC, LENGTH, the debug program CRC if there is one, and the payload CRC all pass. On any failure the ROM sends a failure token, empties the line and sends QRDY, and the PC resends the whole frame. | 7 |
| SYSCSR.DBG_EN = 1? | The ROM reads SYSCSR DBG_BOOT. Bit 0, dbg_en, is the DBG_EN pin through two flip-flops, forced to 0 once the host has written dbg_en_mask = 1. | 7.5, QNSC_SYSCSR_MAS 7.5 |
| CPU polling: read SYSCSR.DBG_EN again (1: debug boot) | The ROM reads DBG_BOOT in a loop. The CPU is not held in reset: it runs this loop, so the host can halt it into the debug program at any time: Ibex enters it at DmHaltAddr, 0x2000_0800, and dret brings it back into this loop. The host ends the wait by writing DBG_BOOT.dbg_en_mask = 1 through the SYSDBG bus master. With DEBUG_PROG other than 0x55 the ROM waits all the same and sends no token. | 7.5 |
| jump to 0x2000_1080 (ISRAM) (0: normal boot) | The ROM clears the instruction prefetch and jumps to 0x2000_1080. | 7.5 |
| application runs from ISRAM | Section 9. If the frame carried a debug program, the host can halt the CPU into it while the application runs. | 9 |

# 5. UART0 configuration

UART0 is pulp-platform/apb_uart around obi_uart: 16550 registers at byte offset 4 x n, 16-byte RX FIFO.

Table 5-1. UART0 registers used, base 0x8002_0000

| Offset | Register | Use |
| --- | --- | --- |
| 0x00 | RBR / THR / DLL | receive, transmit; DLL = 65 while DLAB = 1 |
| 0x04 | IER / DLM | IER = 0; DLM = 0 while DLAB = 1 |
| 0x08 | FCR | 0x07: FIFOs on, both cleared |
| 0x0C | LCR | 0x80 (DLAB) while writing the divisor, then 0x03: 8 bits, no parity, 1 stop |
| 0x14 | LSR | bit 0 data ready; bit 5 THRE; bit 6 TEMT. Bits 1–3, the line errors, are not read |

Divisor 65: 20 MHz / (16 x 65) = 19 231 baud, 0.16 % above 19 200, a standard rate on the PC. One byte takes 520 µs, 10 400 cycles. The low rate keeps a wide margin against framing errors on the link.

The ROM polls UART0 and uses no interrupt. To receive, it reads LSR in a loop: bit 0 set means a byte waits in RBR, which it then reads; otherwise it reads LSR again. While it hunts MAGIC the loop has no limit; after MAGIC it counts the polls against RX_TIMEOUT (7.3). To send, it waits for LSR.THRE = 1 and writes THR.

The ROM ignores the line error bits. With 8N1 there is no parity error. A byte received with a framing error is kept as it is and fails a CRC (FHCR, FDCR or FPCR). A byte lost to an overrun leaves the frame one byte short, so the ROM waits for a byte that never comes and sends FTMO. Either way the PC resends the frame.

# 6. Frame format

Figure 6-1. Boot frame; every field little-endian

Table 6-1. Frame fields

| Offset | Field | ROM check |
| --- | --- | --- |
| 0x00 | MAGIC | the bytes ‘Q’, ‘S’, ‘O’, ‘C’ in that order |
| 0x04 | LENGTH | multiple of 4, 132 to 61 440 |
| 0x08 | DEBUG_PROG | not checked: 0x0000_0055, a debug program follows the header; any other value, none |
| 0x0C | HDR_CRC | CRC32 of bytes 0x00–0x0B |
| 0x10 | DEBUG_PAYLOAD | only with DEBUG_PROG = 0x55: 4 096 bytes, the debug program zero-padded by the PC; written to ISRAM from 0x2000_0000 |
| 0x1010 | DBG_CRC | only with DEBUG_PROG = 0x55: CRC32 of DEBUG_PAYLOAD |
| D | PAYLOAD | LENGTH bytes of the application binary, zero-padded by the PC; written to ISRAM from 0x2000_1000. D = 0x10, or 0x1014 after a debug program |
| D + LENGTH | PAY_CRC | CRC32 of PAYLOAD |

The frame carries no address. The debug program always goes to 0x2000_0000, the payload always to 0x2000_1000, and the ROM always jumps to 0x2000_1080, after the application’s 128-byte vector table. LENGTH is at least 132 so that the image reaches the first instruction at 0x2000_1080, and at most 61 440 so that it ends inside ISRAM, at 0x2001_0000. The debug program has no length field: it is always sent in full, so it can neither overlap the payload nor stop short.

DEBUG_PROG is a whole word so that every field stays on a 4-byte boundary. The ROM compares the whole word with 0x0000_0055 and checks nothing else: any other value means no debug program, and the PC loader sends 0x0000_00AA for that. HDR_CRC already shows that the value is the one the PC sent, and the PC sends the debug program on the same test, so the two sides cannot disagree.

CRC32 is the zlib variant: reflected polynomial 0xEDB88320, initial value and final XOR 0xFFFF_FFFF. The ROM computes it bitwise.

# 7. Bootloader behaviour

Figure 7-1. Bootloader flow

Table 7-1 describes each step of Figure 7-1 and how each failure is found. Every check is done by the bootloader in software.

Table 7-1. Steps of Figure 7-1

| Step of Figure 7-1 | What the ROM does | Failure detected, token |
| --- | --- | --- |
| init UART0 | Section 5. | – |
| waiting for each MAGIC byte from PC; last 4 bytes = ‘Q’ ‘S’ ‘O’ ‘C’? | Poll LSR with no time limit and shift each byte through a 4-byte window until it holds MAGIC. Nothing is sent. | none |
| waiting for the next header byte: LENGTH, DEBUG_PROG, HDR_CRC | Poll LSR for the 12 bytes of LENGTH, DEBUG_PROG and HDR_CRC. A CRC32 runs over MAGIC, LENGTH and DEBUG_PROG as they arrive. | – |
| new byte within RX_TIMEOUT polls? | The poll count restarts at every byte (7.3). | no: FTMO |
| 12 header bytes received? | No: wait for the next header byte. | – |
| CRC32 = HDR_CRC? | Checked before the fields: with a wrong CRC their values mean nothing. | no: FHCR |
| LENGTH valid? | The CRC only shows that the bytes arrived as the PC sent them, not that the value makes sense. LENGTH must be a multiple of 4 and 132 to 61 440 (Table 6-1), so the payload reaches 0x2000_1080 and stays inside ISRAM. DEBUG_PROG is not checked. | no: FHDR |
| send ACKH | ISRAM is written only after this token. | – |
| DEBUG_PROG = 0x55? | The whole word is compared. No, any other value: go to the payload. | – |
| waiting for the next debug program or DBG_CRC byte; write it to 0x2000_0000 (ISRAM) | Every 4 bytes are written as one word to ISRAM, the first at 0x2000_0000, and fed to a CRC32. The 4 bytes of DBG_CRC are neither written nor fed. | – |
| new byte within RX_TIMEOUT polls? | As for the header. | no: FTMO |
| 4 096 bytes and DBG_CRC received? | The loop bound only. No: wait for the next byte. | – |
| CRC32 = DBG_CRC? | DBG_CRC is the CRC32 the PC computed over the debug program. | no: FDCR |
| send ACKD | The PC may send the payload. | – |
| waiting for the next payload or PAY_CRC byte; write the payload to 0x2000_1000 (ISRAM) | As for the debug program, from 0x2000_1000, with a CRC32 of its own. | – |
| new byte within RX_TIMEOUT polls? | As for the header. | no: FTMO |
| all payload and PAY_CRC bytes received? | The loop bound only: LENGTH payload bytes, then 4 bytes of PAY_CRC. No: wait for the next byte. | – |
| CRC32 = PAY_CRC? | PAY_CRC is the CRC32 the PC computed over the payload; the ROM compares it with its own. | no: FPCR, no jump |
| send ACKP | 7.5. | – |
| read SYSCSR.DBG_EN = 1?; CPU polling | 7.5. Yes: read it again. | – |
| jump to 0x2000_1080 (ISRAM) | 7.5. | – |
| send failure token, clear RX FIFO, discard bytes until quiet for DRAIN_IDLE polls, send QRDY | The token is sent in full first. Then the RX FIFO is cleared, bytes are discarded until the line has been quiet for DRAIN_IDLE, QRDY is sent, and the ROM waits for MAGIC again (7.2). | – |

## 7.1 Handshake

Figure 7-2. PC loader and boot ROM

Table 7-2. Tokens, 4 ASCII bytes, sent by the ROM

| Token | Sent when |
| --- | --- |
| QRDY | after every failure, once the line is drained; not after reset |
| ACKH | header received, HDR_CRC correct, LENGTH in range |
| ACKD | DBG_CRC correct; only with DEBUG_PROG = 0x55 |
| ACKP | PAY_CRC correct |
| FHCR | HDR_CRC wrong |
| FHDR | LENGTH out of range |
| FDCR | DBG_CRC wrong |
| FPCR | PAY_CRC wrong |
| FTMO | more than RX_TIMEOUT between two bytes after MAGIC |

- FAIL in Figure 7-2 is any failure token; Figure 7-1 shows which step sends which.

- Bytes before MAGIC get no reply, so line noise is harmless. No token is sent after reset: the PC sends its frame without waiting. If the frame starts before UART0 is configured, MAGIC is missed, no ACKH comes back, and the PC resends after its 2 s wait.

- Nothing is written to ISRAM before ACKH.

- No token reports a UART error: a CRC failure or FTMO reports it (5).

- ACKP does not say that the application runs: with the DBG_EN pin at 1 the ROM waits for the host first (7.5).

## 7.2 Failure and retry

After a failure token the ROM clears its RX FIFO, discards bytes until the line has been quiet for DRAIN_IDLE, sends QRDY and waits for a new frame; the PC resends the whole frame, debug program included. Words already in ISRAM are overwritten by the next one. The watchdog is off after reset (aon_timer WDOG_CTRL resets to 0), so a download that never completes waits for the PC or a reset. The PC waits for QRDY before it resends, so no byte of the new frame is lost in the drain. The PC stops sending as soon as it reads a failure token: it sends the debug program and the payload in chunks and checks for a token between them, so the ROM does not drain the rest of a bad frame.

## 7.3 Timeouts

The bootloader uses no timer: a timeout is a count of LSR polls. A poll is at least one APB read, and an APB transfer takes at least 2 cycles, so Table 7-3 gives minimum times only. The exact time per poll depends on the bus path and is not specified.

Table 7-3. Timeouts

| Constant | Polls | Minimum, at 2 cycles per poll | Meaning |
| --- | --- | --- | --- |
| RX_TIMEOUT | 1 000 000 | 100 ms, 192 byte times | between two bytes of a frame, after MAGIC |
| DRAIN_IDLE | 500 000 | 50 ms, 96 byte times | quiet line that ends a drain |
| MAGIC hunt | – | – | no limit |
| Debug wait | – | – | no limit: until the host writes DBG_BOOT.dbg_en_mask = 1 |

A slower poll only lengthens a timeout. The PC waits 2 s for each token, plus the transfer time for ACKD and ACKP, so the counts need no tuning. The margins cover gaps in the PC stream from the operating system and the USB-serial adapter. RX_TIMEOUT must stay below the PC’s 2 s wait, so a poll must take fewer than 40 cycles; the time per poll is measured in simulation (10).

## 7.4 Writing ISRAM

The debug program and the payload are written one 32-bit word per four received bytes, by the same loop.

## 7.5 Debug wait and jump

After ACKP the ROM waits for LSR.TEMT = 1, so the token has left UART0. Then it reads SYSCSR DBG_BOOT (0x8000_400C) until bit 0, dbg_en, is 0:

- Normal boot, DBG_EN pin at 0: the first read returns 0.

- Debug boot, DBG_EN pin at 1: the ROM reads in a loop. The CPU runs and is not held in reset, so the host can raise the Ibex debug request at any time. The host ends the wait by writing DBG_BOOT.dbg_en_mask = 1 through the SYSDBG bus master (AXI_S0, S_BUS, P_BUS); the host can only clear dbg_en, never set it. The bus master needs no help from the CPU, so the mask can be written while the CPU is halted.

- A debug request during the wait suspends the loop, it does not end it. Ibex saves the PC of the interrupted loop instruction in dpc and enters the debug program at its dispatch loop, DmHaltAddr = 0x2000_0800 (9). When the host resumes it, dret returns to dpc, back into the wait loop, which reads DBG_BOOT again. Only dbg_en = 0 ends the wait. To stop on the first instruction of the application, the host halts the CPU, plants a breakpoint at 0x2000_1080, writes dbg_en_mask = 1 and resumes.

- DEBUG_PROG other than 0x55 with the pin at 1 is an operator mistake: there is no debug program, so 0x2000_0800 holds no dispatch loop and the host must not raise the debug request. The ROM waits all the same and sends no token; the host releases it with dbg_en_mask = 1.

Then the ROM executes fence.i and jumps to 0x2000_1080. It does not return.

# 8. Bootloader image

Table 8-1. Limits, enforced by the build

| Limit | Value | Enforced by |
| --- | --- | --- |
| Image | 2048 bytes, vector table included (QNSC_ROM_MAS 6) | link.ld, util/gen_rom.py |
| Code | 1 KiB after the vector table (Day005 review) | make size |
| Sections | .text only: no .data, .bss or .rodata | link.ld |

- crt0.S holds the 32 vectors and _start at 0x80: sp = 0x3000_8000, the top of DSRAM, then boot_main.

- Built with -march=rv32imc_zicsr_zifencei, which fence.i needs.

- V3.4 image: 734 bytes, 606 bytes of code after the vector table.

Table 8-2. Source, design/rom/boot/

| File | Content |
| --- | --- |
| boot.h | Frame, tokens, timeouts, the SYSCSR DBG_BOOT address; addresses from the generated qnsc_map.h |
| crt0.S, boot.c | Vectors and _start; the bootloader |
| link.ld | Layout and the limits above |
| Makefile | make, make size, make test, make test-serial; output rom_image.hex |
| tools/qsoc_image.py | Packs an application binary, and with --debug a debug program, into a frame: pads the payload to a multiple of 4 and the debug program to 4 096 bytes, sets LENGTH and DEBUG_PROG |
| tools/qsoc_loader.py | Sends a frame over a serial port at 19 200 baud and follows the handshake: the debug program and the payload in chunks, stopped at a failure token, then a retry |
| tools/rom_model.py, tools/test_protocol.py | Python model of boot.c, DBG_EN pin and dbg_en_mask included, and the loader run against it for every token (make test) |
| tools/test_serial_link.py | The loader command line through pyserial (socket://) against the model (make test-serial) |

# 9. Application contract

- Link at 0x2000_1000, 132 bytes to 60 KiB.

- Put a 128-byte vector table at 0x2000_1000 and set mtvec to it; put _start at 0x2000_1080.

- Set up its own stack and data in DSRAM.

- Find UART0 configured (19 231 baud 8N1, FIFOs on, interrupts off, transmitter empty), every peripheral clock running except TIMER1, and the watchdog off.

- Read and clear SYSCSR RESET_CAUSE (QNSC_SYSCSR_MAS 7.6).

- A debug program is linked at 0x2000_0000, at most 4 KiB, and follows the debug window layout of QNSC_SYSDBG_DECISIONS 4.11 (Table 9-1), because the Ibex debug addresses are fixed when the CPU is built.

Table 9-1. Debug program layout, fixed by the Ibex debug parameters

| Address | Contents |
| --- | --- |
| 0x2000_0000–0x2000_07FF | Instruction sequence area, written by the host |
| 0x2000_0800 | Dispatch loop, DmHaltAddr: Ibex enters here on every halt |
| 0x2000_0810 | Exception handler, DmExceptionAddr: a fault in debug mode |
| 0x2000_0F00 | Result words |
| 0x2000_0F08 | Command word |
| 0x2000_0F0C | Resume flag: the dispatch loop clears it and executes dret |

Without a valid dispatch loop at 0x2000_0800 the first halt runs whatever is there. Before V3.3 SYSDBG wrote the dispatch loop over JTAG; now it comes in the frame. What the loop and the sequences do is specified with the debug flow, not here.

Download time at 19 231 baud: 60 KiB in about 32 s, 22 KiB in about 11.7 s; a debug program adds about 2.1 s.

# 10. Requirements on others, and open items

Table 10-1. Requirements on other owners

| Item | Owner | What it blocks |
| --- | --- | --- |
| qnsc_map.h generated from the contract, as qnsc_pkg.sv is | lead | boot.h without typed addresses |
| SYSCSR DBG_BOOT at 0x0C: dbg_en RO, dbg_en_mask RW, the DBG_EN pin through two flip-flops | SYSCSR (same owner) | 7.5 |
| The DBG_EN pad wired to SYSCSR i_dbg_en | top (lead) | 7.5 |
| QNSC_SYSDBG_MAS follows this flow: no CPU hold, no boot address mux, no debug window initialiser; the bus master reaches SYSCSR | SYSDBG (lead) | debug boot as in 4 and 7.5 |
| Ibex DmHaltAddr = 0x2000_0800, DmExceptionAddr = 0x2000_0810, as in QNSC_SYSDBG_DECISIONS 4.11 | SYSDBG, CPU | the debug request of 7.5, the layout of 9 |

Open items: design/rom/boot/ is in PR #51 (feat/rom-wrapper), not yet merged. ROM owner. Time per LSR poll, measured in simulation, to replace the minimums of Table 7-3. ROM owner.

dbg_en_mask is reset by power-on only (QNSC_SYSCSR_MAS 7.5): after a WDT bite or SW_RST in a debug session the mask keeps the value the host wrote, so the ROM waits again only after a power-on.

# 11. Verification

- BOOT_001 After each reset source the ROM sends no token and accepts a correct frame: ACKH, ACKP, then execution at 0x2000_1080.

- BOOT_002 Bytes before MAGIC get no reply.

- BOOT_003 A wrong HDR_CRC gives FHCR; a LENGTH out of range gives FHDR; neither writes ISRAM. A DEBUG_PROG other than 0x0000_0055 (0x0, 0x33, 0x155) is taken as no debug program: ACKH, ACKP, nothing written below 0x2000_1000.

- BOOT_004 A wrong PAY_CRC gives FPCR and no jump.

- BOOT_005 A gap longer than RX_TIMEOUT after MAGIC gives FTMO; a byte changed on the line gives FHCR, FDCR or FPCR, a byte lost gives FTMO.

- BOOT_006 After every failure the ROM sends QRDY and accepts a resent frame.

- BOOT_007 After ACKP, ISRAM from 0x2000_1000 equals the payload word for word.

- BOOT_008 The build fails when a limit of Table 8-1 is broken.

- BOOT_009 make test passes against rom_model.py for every token.

- BOOT_010 With DEBUG_PROG = 0x55 the ROM answers ACKH, ACKD, ACKP, and ISRAM from 0x2000_0000 equals the debug program padded to 4 096 bytes; a wrong DBG_CRC gives FDCR and a retry.

- BOOT_011 With the DBG_EN pin at 1 the ROM sends ACKP and does not jump until DBG_BOOT.dbg_en_mask is written 1; then it jumps to 0x2000_1080.

- BOOT_012 With the DBG_EN pin at 1 and no debug program the ROM waits as in BOOT_011 and sends no failure token.

# Appendix A. Acronyms

Table A-1. Acronyms

| Acronym | Description |
| --- | --- |
| CRC | Cyclic Redundancy Check |
| DLAB | Divisor Latch Access Bit, LCR[7] |
| LSR | Line Status Register |
| THRE, TEMT | Transmit holding register empty, transmitter empty |

# Appendix B. First review

Table B-1. First review

| Item | Reviewer | Response |
| --- | --- | --- |
| Handshake mandatory: ACK or failure per step | Quan (mentor), Day006 | 7.1 |
| Header checked separately from payload | Quan (mentor), Day006 | 6, HDR_CRC |
| Why LOAD_ADDR and ENTRY are separate | Quan (mentor) | DECISIONS; both removed in V3.2 |
| First fetch is ROM base + 0x80 | Sinh | QNSC_ROM_MAS 6 |
| UART0 at 0x8002_4000, “APB slave 9” | lead, V3.0 | 0x8002_0000, APB_M8, map generated in 3 |
| Byte time 86.8 µs, 1736 cycles | lead, V3.0 | 88.0 µs, 1760 cycles at the real baud rate |
| Word writes justified by “no byte-strobe path” | lead, V3.0 | ISRAM has byte strobes (QNSC_RAM_MAS 7.3); reason removed |
| Figure 4-1: show the CPU release | Quan (mentor), 29/09 | Figure 4-1, Table 4-2 |
| Leave out of the MAS any parameter or step that is not known | Quan (mentor), 29/09 | 7.3: timeouts as minimums |
| Polling for the PC data not defined; QRDY after reset not needed | Quan (mentor), 29/09 | 5: polling; 7.1: no QRDY after reset, kept after a failure |
| “ACKP sent?” should test the CRC | Quan (mentor), 29/09 | Figure 7-1: CRC32 = PAY_CRC? before ACKP; V3.2 Figure 4-1 no longer repeats it |
| How does the CPU run the image loaded over JTAG if it starts in the ROM? | Quan (mentor), 29/09 | V3.1: hardware boot address mux. V3.3: no longer asked, the ROM loads every image |
| Baud rate down to about 10 000–20 000 | Quan (mentor), 29/09 | 5: 19 200 |
| Figure 7-1: how each failure is detected and reported | Quan (mentor), 29/09 | Table 7-1 |
| Describe every step of Figures 4-1 and 7-1 | Quan (mentor), 29/09 | Tables 4-2, 7-1 |
| Drop ENTRY from the header, to match debug boot | Quan (mentor), V3.1 review | 6: no ENTRY, no LOAD_ADDR; the ROM jumps to 0x2000_1080 |
| The ROM loads the programs, not JTAG | Quan (mentor), 12/10 drawing | 4, 7: both programs over UART0 |
| Header field DEBUG_PROG: 0x55 debug program and user program, 0xAA user program only | Quan (mentor), 12/10 drawing | 6: a 4-byte word; 4 096-byte debug program with DBG_CRC |
| ROM reads SYSCSR.DEBUG_EN: 1, CPU polls; 0, jump to the user program | Quan (mentor), 12/10 drawing | 7.5; named DBG_BOOT.dbg_en by the Naming Rule |
| JTAG clears it by writing DEBUG_EN_MASK = 1 through a mux; the pin feeds the status through two flip-flops | Quan (mentor), 12/10 drawing | 7.5, QNSC_SYSCSR_MAS 7.5: dbg_en_mask |
| JTAG can put the CPU in debug mode at any time with cpu_dbg_en | Quan (mentor), 12/10 drawing | 4, 7.5: the CPU is never held, so the request works during the wait and while the application runs |
| Tokens ACKD and FDCR for the debug program | Quan (mentor), 12/10, confirmed | 7.1 |
| Entry 0x2000_1080, after the application’s vector table, not the load address 0x2000_1000 | Quan (mentor), 12/10, confirmed | 6, 7.5 |
| dbg_en_mask reset by power-on only | Quan (mentor), 12/10, confirmed | 10, QNSC_SYSCSR_MAS 7.5 |
| DEBUG_PROG = 0x55: load the debug program; any other value: skip it, no 0xAA check | Quan (mentor), 14/10 | V3.4: 6, 7.1; FHDR for LENGTH only |