# RAM

MICRO-ARCHITECTURE SPECIFICATION – V1.9

QUY NHON SEMICONDUCTORS – QNSC

# Table of Contents

# Table of Figures

Figure 1-1. Figure 1 – Where the RAM sits in QSOC

Figure 3-1. Figure 2 – The AXI4 SRAM controller and its macro

# Reversion and History

| Version | Date | Author/Owner | Description of Change |
| --- | --- | --- | --- |
| V1.0 | 2026-09-16 | Nghia VT | First issue as a standalone document. Supersedes the RAM half of QNSC_RAM_Debugger_MAS V1.2, which specified the RAM and the Debugger together. Content is unchanged from that issue: the controller is nguyenquanicd/AXI4-SRAM-CONTROLLER, with the WSTRB byte-enable path added for QSOC. |
| V1.1 | 2026-09-17 | Nghia VT | Follows the instructor’s ruling of 2026-09-17. The debug program is no longer a block of its own: it is the first 4 KiB of ISRAM. The RAM is now two instances – ISRAM for the program and DSRAM for data – because QSOC has no flash and the program runs from RAM. ROM reduced to 8 KiB by its owner. Memory map rewritten (Tables 10–14), decode windows and the Ibex reset-vector offset documented, and the WSTRB and WRAP findings traced through pulp-platform/axi. |
| V1.2 | 2026-09-17 | Nghia VT | Adopts the memory map agreed with the CPU and bus owners: ISRAM at 0x2000_0000, DSRAM at 0x3000_0000, the APB window at 0x8000_0000, and DSRAM moving to AXI_M2 with AXI2APB to AXI_M3 (Table 10). Decode windows narrowed to the memory actually present, with DECERR everywhere else (Table 12). The debug program is fixed at the bottom of ISRAM so the CPU’s Dm* parameters do not move when PARA_SRAM_DEPTH changes. ISRAM confirmed at 64 KiB, with the firmware cost estimate that decided it added as Table 14. PARA_ADDR_WD corrected to PARA_SRAM_DEPTH where the text meant capacity. |
| V1.3 | 2026-09-18 | Nghia VT | Aligned with QSOC_HAS, the system architecture specification, after a line-by-line comparison. The memory map needed no change: base addresses, sizes, the 4 KiB debug window at the bottom of ISRAM, the ISRAM/DSRAM split and the 16 APB slaves of 16 KiB all match the HAS exactly. What was wrong was the boot flow. V1.0–V1.2 assumed the application arrived over JTAG and that the ROM therefore “does very little: come out of reset, and wait”, stating explicitly that a UART bootloader “would need more” ROM than QSOC has. Both the instructor’s direction and QSOC_HAS say the opposite: the ROM holds a serial bootloader which receives a single header/payload/CRC32 frame over UART0 at 115200 baud – 2.0 s for a 22 KiB image, 5.4 s for a full 60 KiB – and the debug interface is the second load path, used when the ROM contents themselves are suspect. The HAS budgets that bootloader at 1.1 KiB with a bitwise CRC32 or 2.1 KiB with a table, so 8 KiB of ROM is ample for a real bootloader rather than ample because the ROM is nearly empty. Section 5.1 is rewritten accordingly: the conclusion that ISRAM needs 60 KiB free is unchanged, but the reason is now that the bootloader puts the program there. Two of the HAS’s ROM budget items belong to this document and are cross-referenced: the 128-byte trap vector table is the ROM layout of Table 15, and the payload copy loop is the only master that writes ISRAM before the application exists. |
| V1.4 | 2026-09-18 | Nghia VT | Adds the clock and reset domain, which this document had never stated, and closes an open row in QSOC_HAS that named it. New section 5.2. QSOC_HAS Table 4-2 puts ISRAM and DSRAM in one domain, D13, sharing soft-reset bit 12 and one clock gate, with the ROM separate in D12. Sharing one bit has a consequence now recorded: a soft reset of D13 resets the memory holding the code that issued the write, so firmware running from ISRAM cannot reset the RAM domain and survive – safe only from ROM. The boot-memory ordering row that QSOC_HAS left TBD is answered from the AXI specification rather than by simulation. The HAS records two readings of a fetch issued while the memory is still in reset: a stalled handshake, or undefined read data causing a trap. The second cannot happen – AXI requires every VALID to be deasserted during reset, so a slave in reset cannot assert rvalid and there is no read data to be undefined. The core simply waits, so ordering is not a correctness requirement and the clock-and-reset specification is right that no sequencing is needed. What replaces it is a liveness problem worth more attention: AXI has no timeout, so a core waiting on a memory that never leaves reset waits forever with no exception and nothing in any status register, and the only block that can observe it is SYSDBG, which sits outside the ndmreset fan-out. Also fixes a duplicated section number – Open questions was 5.3 twice – and the cross-reference that pointed at it. |
| V1.5 | 2026-09-18 | Nghia VT | Cross-checked against QSOC_HAS EN v1.1, which surfaced a gap this document had: nothing was said about the SRAM macro’s non-functional pins. Section 4.8 now requires margin, retention and test to be tied to their databook defaults, and explains why a specification has to say so – the behavioural model has no such pins, so no simulation can fail because of them. The memory map, the ISRAM split and the boot flow all still agree line for line. Section 5.2 signs off the shared D13 domain that the HAS puts to this author as an open item, and records the reserved SOFT_RST_CTRL bits [15:14] as the escape hatch that keeps the decision reversible |
| V1.6 | 2026-09-21 | Nghia VT | ROM resized from 8 KiB to 2 KiB and the bootloader budget rewritten, following the Day005 review of 2026-09-18, which fixed ROM at 2 KiB with the program inside it at not more than 1 KiB; the ROM owner’s specification agrees, decoding 11 bits for 512 words. Section 5.1 now shows what is actually left – 128 B of forced vector table and 1920 B of usable space – and draws the consequence the earlier text could not: the 2.1 KiB table-driven CRC32 no longer fits, so the bitwise variant is required. The memory map, the decode window and the FPGA block counts are updated. ISRAM and DSRAM are unchanged at 64 KiB and 32 KiB. |
| V1.7 | 2026-09-21 | Nghia VT | Three open questions resolved. The WSTRB addition is claimed by this author, since it lives in the wrapper rather than inside the IP. And the licensing and WRAP-reporting questions both become one question to the mentor: nguyenquanicd/AXI4-SRAM-CONTROLLER is the mentor’s own repository, the same account supplying APB-CSR-Generator, APB-BUS-Generator, FirstX2P and MRV-CPU to QSOC – so the WRAP finding reaches its author directly and benefits ROM as well. |
| V1.8 | 2026-09-21 | Nghia VT | The open-question list is emptied of everything that did not need someone else. Item 1 closes: the ROM owner’s specification confirms the bootloader is theirs at 2 KiB / <= 1 KiB, and the boot flow specification confirms boot_addr_i with Ibex fetching from 0x0000_0080. Item 3 closes as out of scope for v1, with the real cost named – scrambling changes the read path rather than adding a module. The licensing and WRAP-reporting items are reclassified as action items, since they are things to do rather than information to wait for, and both go to the mentor whose repository the IP is. |
| V1.9 | 2026-09-21 | Nghia VT | Corrects a stale cross-reference: section 5.2 argued that SYSDBG could observe a stuck memory reset because it sat outside the ndmreset fan-out, but QNSC_SYSDBG_MAS removed ndmreset at V1.9 following the Day005 reset-source decision. The observation still holds for a different reason – SYSDBG is in domain D15, released before the CPU – but the conclusion is now stronger: the debugger can see a stuck reset and cannot clear it, so recovery is a power cycle and the liveness guarantee is owed by the clock and reset owner. |

# Table of Tables

| Table | Title |
| --- | --- |
| Table 1 | IP sourcing decision |
| Table 2 | RAM controller sub-modules |
| Table 3 | AXI interface signals present and absent |
| Table 4 | FIFO buffers |
| Table 5 | Where the byte enables are lost |
| Table 6 | Closing the byte-enable gap: three options |
| Table 7 | Burst address generation |
| Table 8 | RAM configuration parameters |
| Table 9 | Verification QSOC must add |
| Table 10 | QSOC memory map |
| Table 11 | What splitting the RAM removes, with one CPU2AXI |
| Table 12 | Decode window against memory actually present |
| Table 13 | Where the debug memory can live |
| Table 14 | Estimated ISRAM occupancy for QSOC’s peripheral set |
| Table 15 | ROM layout forced by the Ibex reset vector |
| Table 16 | Interfaces to agree with the team |
| Table 17 | Acronyms |

# Table of Figures

| Figure | Title |
| --- | --- |
| Figure 1 | Where the RAM sits in QSOC |
| Figure 2 | The AXI4 SRAM controller and its macro |

# 1. Overview

## 1.1 Scope

This document specifies the RAM of QSOC, the MCU built in this training project. In the QSOC block diagram it is two slaves: ISRAM on AXI_M1, holding the program, and DSRAM on AXI_M2, holding data. Both are the same controller with a different macro depth, so one specification covers both – section 5.1 explains why the block diagram was split.

The debugger, SYSDBG, is the author’s second block and is specified separately in QNSC_SYSDBG_MAS. The two are described apart because they are different kinds of work: the RAM is an integration of an existing IP, while SYSDBG is an in-house design. They meet only on S_BUS, where the debugger is one of the masters this block must serve.

QSOC uses AXI for the system bus (S_BUS) and APB for peripherals (P_BUS). The IP selected here is therefore a native AXI4 slave, not an adapted one.

Table 1 – IP sourcing decision

| Item | Source | License | Why this one |
| --- | --- | --- | --- |
| RAM controller | nguyenquanicd/AXI4-SRAM-CONTROLLER, rtl/m_vlsi_axi4_sram.sv and four sub-modules | None declared – see 5.3 | Native AXI4 full slave with burst support, written in SystemVerilog, with a 46-test directed regression already in the repository. It speaks AXI on one side and a plain SRAM interface on the other, which is exactly the shape QSOC needs. |
| RAM macro | Technology-dependent; sim/vcs/env/m_vlsi_sram_sp.sv in the same repository is the behavioural model | n/a | Single-port SRAM with a one-cycle read latency. Replaced by a real macro or an FPGA block RAM at implementation time. |
| Byte-enable path | Designed in house – section 4.4 | n/a – own RTL | The controller has no WSTRB path. A RISC-V core executing sb or sh requires one. |

What changed from the earlier issue. The first version of this specification paired an AXI front-end from verilog-axi with OpenTitan’s scrambling and SECDED primitives – sound, but an assembly of three sources, and it made the RAM a security exercise before it was a working memory. AXI4-SRAM-CONTROLLER is a single coherent IP that does the one thing QSOC needs first: terminate AXI4 and drive an SRAM.

## 1.2 Position in the system

Figure 1-1. Figure 1 – Where the RAM sits in QSOC

Three blocks issue transactions on S_BUS: CPU2AXI on AXI_S1, SYSDBG on AXI_S0, and DMA on AXI_S2. Three memories answer them: ROM on AXI_M0, ISRAM on AXI_M1 and DSRAM on AXI_M2. Peripherals sit behind the AXI2APB bridge on AXI_M3.

Two consequences for this specification:

- The RAM must tolerate three independent masters. Arbitration between them is the bus’s responsibility, not the RAM’s, but the RAM must not assume a single requester – and the round-robin arbiter of section 4.1 arbitrates only between this block’s own read and write paths, which is a different question.

- One of those masters is the debugger, which reads and writes the RAM without the CPU being involved. That is how firmware is loaded, and it means accesses can arrive while the CPU is halted or while it is running.

The same controller is instantiated three times in QSOC – ISRAM and DSRAM here, and the ROM by its own owner. A ROM is never written, so the WSTRB gap of section 4.4 does not apply to it, and the block is used there unmodified.

# 2. Feature

## 2.1 Feature – RAM

The RAM block is an AXI4 SRAM controller plus a memory macro. The controller is taken as an IP; the macro is technology-dependent and is chosen at implementation time.

Provided by the IP:

- Native AXI4 (full) slave, not AXI4-Lite: independent read and write channels, each with its own address FSM.

- Burst transfers, AxLEN from 0 (single) to 255 (256 beats), with INCR and FIXED burst types. See section 4.6 on WRAP.

- AXI ID preserved end to end, AWID to BID and ARID to RID, at a configurable width. Transactions are not reordered.

- Five FIFOs, one per AXI channel, so AXI handshaking is decoupled from SRAM access timing and back-to-back transfers do not stall.

- Round-robin arbitration between the read and write paths into the single SRAM port, so neither can starve the other.

- Configurable data width, address width, ID width and FIFO depth.

Added for QSOC, section 4.4:

- A WSTRB byte-enable path, without which a RISC-V sb or sh would destroy the rest of the word.

Deliberately not in this issue: data scrambling, address scrambling and SECDED integrity, so that a working memory comes first. Section 5.3 records what adding them would cost.

Not implemented by the IP, and out of scope: exclusive access, locked transfers, narrow transfers (AxSIZE is not a port – the transfer size is fixed at the data width), and error responses (BRESP and RRESP are hardwired to OKAY).

# 3. Block Diagram

Figure 3-1. Figure 2 – The AXI4 SRAM controller and its macro

# 4. Micro-architecture Details

## 4.1 Structure

Figure 2 shows the controller. It is five modules, and all five are IP – the only RTL QSOC adds is the byte-enable path of section 4.4.

Table 2 – RAM controller sub-modules

| Module | Role |
| --- | --- |
| m_vlsi_axi4_sram | Top level. Instantiates and wires everything below |
| m_vlsi_axfsm | AXI address FSM. Two instances, one for AW and one for AR. Handles the address handshake, generates one address per burst beat, and pushes address, ID and a last-beat flag into a FIFO |
| m_vlsi_fifo | Parameterised synchronous FIFO. Five instances. Full and empty are detected with an extra pointer MSB |
| m_vlsi_arbiter | Round-robin arbiter between the read and write requests for the single SRAM port |
| m_vlsi_sram_misc | Control and datapath glue: FIFO pop logic, the SRAM address and data mux, and generation of the R and B responses |

Each address FSM has two states. S_IDLE waits for AxVALID and AxREADY; S_ADDR emits one beat per cycle for as long as the downstream FIFO can accept one. AxREADY is combinational, asserted when the FSM is idle and the FIFO is not full, so a handshake never promises a beat the FIFO cannot take.

## 4.2 The AXI interface, and what is absent from it

Reading the port list is the fastest way to understand this block’s scope. The top module has 33 ports, and the absences matter as much as the presences.

Table 3 – AXI interface signals present and absent

| Channel | Present | Absent |
| --- | --- | --- |
| AW | awaddr, awvalid, awready, awburst, awlen, awid | awsize, awprot, awcache, awlock, awqos |
| W | wdata, wvalid, wready, wlast | wstrb |
| B | bid, bresp, bvalid, bready | – |
| AR | araddr, arvalid, arready, arburst, arlen, arid | arsize, and the same qualifiers as AW |
| R | rid, rdata, rresp, rvalid, rlast, rready | – |
| SRAM | sram_addr, sram_wdata, sram_we, sram_oe, sram_rdata | byte write enables |

Three consequences follow, and each is addressed below: there is no byte-enable path (4.4), the transfer size is fixed at the data width because AxSIZE is not read (4.5), and BRESP and RRESP are tied to OKAY so the RAM never reports an error (4.7).

i_wlast is in the port list but is not used internally – the last beat is tracked by the address FSM’s own counter, not by the master’s flag.

## 4.3 FIFOs and flow control

Table 4 – FIFO buffers

| FIFO | Width | Contents |
| --- | --- | --- |
| AWFIFO | ADDR + ID + 1 | write address, ID, last flag |
| WFIFO | DATA | write data |
| ARFIFO | ADDR + ID + 1 | read address, ID, last flag |
| RFIFO | DATA + ID + 2 + 1 | read data, ID, RRESP, last flag |
| BFIFO | ID + 2 | ID, BRESP |

All five share one depth, PARA_FIFO_DEPTH. Every READY on the AXI side is derived from FIFO occupancy – o_wready = ~wfifo_full is the clearest case – so back-pressure propagates naturally and no timer or counter is involved.

A write is executed when the arbiter grants and both AWFIFO and WFIFO are non-empty; the two are popped together. When the popped entry carries the last flag, a BRESP entry is pushed into BFIFO.

A read is issued when ARFIFO is non-empty, RFIFO is not full, and no read is already pending. Only one SRAM read is in flight at a time (reg_rd_pending in m_vlsi_sram_misc): the address is presented in one cycle and the data is captured and pushed into RFIFO in the next. This costs read throughput – a burst read cannot issue a new address every cycle – and is the first thing to revisit if the RAM becomes a bottleneck.

## 4.4 The byte-enable path – what QSOC must add

This is the one functional gap, and it is the RTL this project contributes.

The controller has no WSTRB input and the SRAM interface has no byte write enables, so every write is full width. That is a stated design boundary of the IP, not a defect – its README says so plainly – and for ROM it does not matter at all, since a ROM is never written.

For RAM it matters, because RV32 has sb and sh. Any C code touching a uint8_t, a packed struct field, or a string will emit them. Today such a store would write the whole 32-bit word and destroy the three bytes beside the one the program meant to change – silently, with no error response.

The byte-enable information does reach this block; it is dropped here and nowhere earlier. The chain is intact the whole way, which is worth checking rather than assuming:

Table 5 – Where the byte enables are lost

| Stage | Signal | Present |
| --- | --- | --- |
| Ibex core | data_be_o, 4 bit | yes (ibex_top.sv) |
| CPU2AXI – a wrapper merging instr_* and data_*, over axi_from_mem | mem_be_i, 4 bit, becomes AXI wstrb | yes, provided the merge carries it |
| S_BUS – axi_xbar | wstrb carried | yes |
| This controller | no wstrb port at all | no |

So the strobes arrive at the RAM’s door and are discarded there. Adding the port is not compensating for something missing upstream – it is connecting the last link of a chain that is otherwise complete.

Three ways to close it. The third is what this specification proposes.

Table 6 – Closing the byte-enable gap: three options

| Option | Cost | Verdict |
| --- | --- | --- |
| Constrain firmware to word accesses only | No RTL, but it constrains every future line of C and cannot be enforced | Not viable |
| Read-modify-write inside the controller | No macro change, but every sub-word write becomes read then write, and it must be interlocked against reads to the same address | Viable, but slower and easy to get wrong |
| Carry WSTRB through to byte write enables | i_wstrb added to the port list and to WFIFO, passed to the macro as o_sram_be | Proposed. It is what the AXI protocol intends, and one cycle per write |

The change is small and localised: widen WFIFO by DATA/8 bits, add the port, and require a macro with byte write enables. FPGA block RAM and every standard SRAM compiler offer them.

## 4.5 Burst address generation

Each address FSM registers the address, the burst type, the ID and a beat count on the handshake, then produces one address per beat.

Table 7 – Burst address generation

| Burst type | Next address |
| --- | --- |
| FIXED (00) | unchanged |
| INCR (01) | addr + DATA_WD/8 |
| WRAP (10) | see 4.6 |

The increment is hardwired to the full data width. AxSIZE is not a port, so a master requesting a narrower transfer is not refused – it is simply ignored, and the addresses advance by four bytes regardless. QSOC must therefore treat the transfer size as fixed at 32 bits.

Addresses must be aligned to the data width. The stride is a constant, so an unaligned start address stays unaligned for the whole burst.

Address width at the macro. o_sram_addr is a byte address. A real macro is word-addressed, so the integration connects o_sram_addr[ADDR-1:2]. The behavioural model in the IP’s simulation environment indexes with the byte address directly, which is harmless in simulation but is not how a macro is wired.

## 4.6 WRAP: a finding, and the constraint it implies

WRAP is declared as supported but is not implemented. The calculation is

// m_vlsi_axfsm.sv
assign addr_wrap = (reg_axaddr + PARA_BYTE_COUNT) & ~(PARA_BYTE_COUNT - 1);

With a 32-bit data width PARA_BYTE_COUNT is 4, so the mask clears the low two address bits. That aligns the address; it does not wrap it. A correct WRAP returns to the start of a block of (AxLEN + 1) x size bytes, which requires AxLEN to be retained – and the FSM keeps only a down-counter, not the length. For an aligned start address, WRAP is therefore identical to INCR.

The IP’s own regression does not catch this because its reference model computes the same thing:

// sim/vcs/env/axi_svt_basic_env.sv
default: next_addr = (curr_addr + 4) & 32'hFFFF_FFFC;

The four wrap_* tests pass because the checker agrees with the design under test, not because the addresses are right. This is recorded here because it is the kind of agreement that a passing regression cannot reveal, and because the finding should go back to the IP’s authors.

Consequence for QSOC: none in practice, and there is a ready mitigation if that ever changes.

WRAP exists for cache line fills with critical-word-first ordering. Nothing in QSOC issues one today, and two of the three masters cannot:

- The CPU path issues no bursts at all. axi_from_mem, the module in pulp-platform/axi that converts Ibex’s memory interface to AXI, emits single-beat transactions. A burst of any kind, let alone a wrapping one, never reaches the bus from the CPU.

- SYSDBG issues AXI4-Lite single beats, which have no burst type.

- The DMA is the one master that could in principle issue a WRAP, and that is a question for its owner.

If a wrapping burst ever does appear, the fix needs no change to this IP: axi_burst_unwrap.sv, in the same pulp-platform/axi library already chosen for S_BUS, “splits wrapping AXI4 bursts into incremental bursts”. Placing it in front of this controller converts the one burst type it gets wrong into the one it gets right.

The constraint should nonetheless be written down, and belongs in the integration checklist of section 5.3.

## 4.7 Error handling

BRESP and RRESP are always 2'b00 (OKAY). The value is not tied off at the port – both are read out of their response FIFOs – but the constant 2'b00 is what m_vlsi_sram_misc pushes in, so the RAM can never report a SLVERR or a DECERR. Two consequences:

- An out-of-range address is not refused. Address decoding is the bus’s responsibility; anything reaching this block is assumed to be in range. The S_BUS decoder must therefore return DECERR itself for unmapped addresses, rather than routing them here.

- There is no error detection in the memory. Without SECDED there is nothing to report, so the tie-off is consistent with the current scope rather than a shortcut. If integrity is added later (5.3), RRESP becomes a real signal and RFIFO already carries the two bits for it.

## 4.8 Configuration parameters

Table 8 – RAM configuration parameters

| Parameter | Default | Meaning | ISRAM | DSRAM |
| --- | --- | --- | --- | --- |
| PARA_DATA_WD | 32 | Data width in bits | 32 | 32 |
| PARA_ADDR_WD | 32 | Address width in bits | 32 | 32 |
| PARA_ID_WD | 4 | AXI ID width – set by the S_BUS interconnect, which widens master IDs to keep them unique | same | same |
| PARA_LEN_WD | 8 | AxLEN width, 8 gives 256 beats | 8 | 8 |
| PARA_FIFO_DEPTH | 8 | Depth of all five FIFOs, a power of two | 8 | 8 |
| PARA_SRAM_DEPTH | 1024 | Not a controller parameter – it belongs to the SRAM macro, sim/vcs/env/m_vlsi_sram_sp.sv line 13. It is the only thing that sets how much memory is present | 16384 words, 64 KiB | 8192 words, 32 KiB |

The two instances differ in one thing only: the depth of the macro behind them. Every controller parameter is identical, PARA_ADDR_WD included – it is the width of the AXI address bus, 32 bits for both, not a statement about how much memory is present. The size comes from PARA_SRAM_DEPTH on the macro, so changing either RAM’s capacity is a one-parameter edit with no RTL change; what it does change outside this block is the decode window the interconnect hands to that slave, section 5.1.

PARA_FIFO_DEPTH sets all five FIFOs at once. Deeper FIFOs absorb longer bursts without back-pressure and cost registers; there is no way to size the write path and the read path differently without editing the top level.

The real macro has pins the behavioural model does not, and they must not be left to chance. m_vlsi_sram_sp.sv is a simulation model with a functional port list only. The SMIC 28nm macro that replaces it carries margin, retention and test pins as well, and none of them are driven by the controller – the controller does not know they exist. QSOC_HAS puts the rule to this author and it is accepted here: every pin the design does not use is tied to the value its databook recommends rather than to whatever is convenient – margin at its default level, retention off, test off. The reason to write this down in a specification rather than leave it to physical design is that all three of those pins are harmless in simulation and wrong only in silicon: the behavioural model has no margin pin to mis-drive, so no simulation can fail because of it, and a test pin left floating or asserted is the kind of fault that appears as a memory that reads back its own previous contents. It is recorded in the checklist of section 5.3 for that reason.

## 4.9 Verification available with the IP

The repository ships more verification than the RTL, and reusing it is part of the reason for choosing this IP.

- 46 directed tests driven by the Synopsys AXI VIP, covering bursts of every type and length, back-pressure, arbiter fairness and starvation, ID reuse and wrap-around, reset mid-burst, FIFO near-full, and long random soak runs.

- Self-checking, not just protocol-legal stimulus: the environment keeps a memory model and compares every read beat against it.

- Lint scripts for Synopsys VC Static and for Verilator.

Two qualifications. The testbench needs VCS and a licensed Synopsys AXI VIP; without them the tests cannot be run as they stand, and Verilator lint is the only part that runs on an open toolchain. And section 4.6 is a reminder that a self-checking environment is only as correct as its reference model.

## 4.10 Verification QSOC must add

The regression above covers the IP as it stands. It cannot cover the byte-enable path of section 4.4, because that path does not exist yet – so those tests are owed by this project.

Table 9 – Verification QSOC must add

| What is checked | How |
| --- | --- |
| A byte store leaves the other three bytes of the word untouched | Write a known pattern to a word, then write one byte with wstrb = 4'b0010, then read the word back |
| All sixteen wstrb values, including 0000 and 1111 | Directed sweep at one address |
| A burst write where wstrb differs per beat | The strobes travel in WFIFO beside the data, so a per-beat difference is the case that catches a shared-register mistake |
| wstrb does not disturb reads | Read-after-write at the same address, with the read issued while a strobed write is still in the FIFOs |

The first of these is the acceptance test for the addition. It is also the failure that would otherwise reach firmware as silent corruption, which is the reason it is written down rather than left to a general soak test.

# 5. Integration into QSOC

## 5.1 A memory map to propose

The map below is the one agreed with the CPU and bus owners. Two of its regions belong to the blocks specified here and a third is required by the debugger, so the reasoning behind each line is recorded rather than left implicit.

Table 10 – QSOC memory map

| Region | Base | Last address | Size | Bus port | Owner |
| --- | --- | --- | --- | --- | --- |
| ROM, boot code | 0x0000_0000 | 0x0000_07FF | 2 KiB | AXI_M0 | ROM owner |
| Debug program, first 4 KiB of ISRAM | 0x2000_0000 | 0x2000_0FFF | 4 KiB | AXI_M1 | this author |
| ISRAM, main program | 0x2000_1000 | 0x2000_FFFF | 60 KiB | AXI_M1 | this author |
| DSRAM, data | 0x3000_0000 | 0x3000_7FFF | 32 KiB | AXI_M2 | this author |
| APB peripherals behind AXI2APB, 16 slaves of 16 KiB | 0x8000_0000 | 0x8003_FFFF | 256 KiB | AXI_M3 | peripheral owners |
| SYSDBG registers | 0xF000_0000 | 0xF000_000B | 12 B | none – JTAG only | this author |
| Everything else | – | – | – | axi_err_slv, DECERR | bus owner |

Two lines of that table are easy to misread and are worth spelling out.

SYSDBG’s registers are deliberately not on the bus. 0xF000_0000 is an address in the JTAG command space only; SYSDBG decodes it internally and never puts it on S_BUS. The interconnect must therefore leave that region unmapped, answering DECERR. If the decoder claimed it as well, the register would be shadowed by whatever slave won. QNSC_SYSDBG_MAS section 4.7 specifies the registers themselves.

Nothing at all is mapped between 0x8004_0000 and 0xFFFF_FFFF, nor in the gaps between the regions above. Every one of those addresses answers DECERR.

The RAM is two instances, not one. QSOC has no flash, so the program lives in RAM, and the instructor’s direction is to split it: ISRAM holds instructions – both the debug program and the main program – and DSRAM holds data.

That split is worth more than the tidiness, and it is worth being precise about what it does and does not buy, because CPU2AXI is one master port by instruction – Ibex’s instr_* and data_* interfaces are merged by a wrapper before they reach the bus.

S_BUS is a crossbar, so it serves several masters at once as long as they address different slaves. With one RAM, every master queues for one slave port. With two:

Table 11 – What splitting the RAM removes, with one CPU2AXI

| While the CPU fetches from ISRAM | One RAM | ISRAM + DSRAM |
| --- | --- | --- |
| the DMA moves a block | contend | concurrent |
| SYSDBG reads memory for the host | contend | concurrent |
| the CPU itself does a load or store | serialised at CPU2AXI | serialised at CPU2AXI |

So the split does not remove the CPU’s own fetch-versus-data serialisation – the single wrapper decides that – but it does remove contention between the CPU and the two other masters. The DMA is precisely the master that moves large blocks while the CPU runs, which is the case the split is worth having for.

It also removes the reason this author argued against putting the debug program inside a RAM at all: a stray data pointer writes into DSRAM, and cannot reach ISRAM unless it deliberately addresses that range.

The debug program needs no hardware of its own. It is the first 4 KiB of ISRAM and nothing more – an address convention. SYSDBG loads it there through the AXI master it already has, and the main program’s linker script starts at 0x2000_1000. The four Dm* parameters in QNSC_SYSDBG_MAS Table 11 follow directly from ISRAM’s base.

Why the debug program is at the bottom of ISRAM and not the top. This looks like an arbitrary choice and is not. The four Dm* parameters are compile-time parameters of ibex_top, not registers: changing one means re-elaborating the CPU. Put the 4 KiB at the top of ISRAM and its address is base + size - 0x1000, so the day anyone changes PARA_SRAM_DEPTH – the cheap, expected edit of section 4.8 – all four parameters move with it and the CPU has to be rebuilt. Put it at the bottom and the parameters depend on the base alone, which never moves; ISRAM grows upward into the reserved space above it and nothing outside this block notices. The cost is that the main image starts at 0x2000_1000 rather than 0x2000_0000, which is one constant in a linker script.

The size and the decode window are not the same thing, and the difference is worth stating before anyone writes a decoder. A decoder that selected each region by addr[31:28] alone would hand every slave a whole nibble – 256 MB – while the memory actually behind it is kilobytes. The agreed map does not do that. Each window is narrowed to the memory actually present, and everything else answers DECERR:

Table 12 – Decode window against memory actually present

| Region | Decode window handed to the slave | Memory actually present | Outside it |
| --- | --- | --- | --- |
| ROM | 0x0000_0000 – 0x0000_07FF | 2 KiB | DECERR |
| ISRAM | 0x2000_0000 – 0x2000_FFFF | 64 KiB, first 4 KiB the debug program | DECERR |
| DSRAM | 0x3000_0000 – 0x3000_7FFF | 32 KiB | DECERR |
| AXI2APB | 0x8000_0000 – 0x8003_FFFF | 16 APB slaves of 16 KiB | DECERR |

The alternative – leaving each window a full nibble wide – costs one comparison less and is worse: an access to 0x2001_0000 beyond the macro depth would alias, the bits above the depth silently dropped, and land back inside real memory. A stray pointer would then read the wrong data with no indication that anything had happened.

Narrowing needs no new RTL. axi_err_slv.sv in pulp-platform/axi – the library already chosen for S_BUS – is exactly an error-responding slave, and axi_xbar’s address rules take an explicit start and end per slave rather than a mask. It is also what section 4.7 asks for, since this RAM answers OKAY to every address it is given and cannot refuse one itself.

The reserved space above each RAM is not wasted, it is the growth path. 0x2001_0000 – 0x2FFF_FFFF above ISRAM and 0x3000_8000 – 0x3FFF_FFFF above DSRAM stay DECERR today. Enlarging either RAM means raising PARA_SRAM_DEPTH and widening that slave’s end address; because the debug program sits at the bottom of ISRAM, no CPU parameter moves when ISRAM grows.

Where the debug program lives was put to the instructor, and the answer was to keep it inside a RAM. The reason a decision was needed at all is in QNSC_SYSDBG_MAS section 4.11: when debug_req rises, Ibex jumps to DmHaltAddr and fetches from it unconditionally, so something must answer at that address from the very first bring-up, even while all it holds is a spin loop.

Three options were put forward; the record is kept because the reasoning still applies if the arrangement is ever revisited.

Table 13 – Where the debug memory can live

| Option | Cost | Verdict |
| --- | --- | --- |
| A. A dedicated 4 KiB block on its own slave port | One entry in the address decoder, one BRAM tile | Not taken |
| B. Reserve the first 4 KiB of a RAM | Nothing at all – an address convention | Chosen by the instructor. The objection to it was that firmware could overwrite the debug program; splitting the RAM into ISRAM and DSRAM largely answers that, since data writes land in DSRAM |
| C. A wrapper behind AXI_M1 that splits the port | Higher than A – the W channel carries no address, so the wrapper must remember the AW routing until WLAST and track outstanding transactions, which is a second interconnect; and it adds a cycle to every SRAM read | Not taken |

The slave-port count is the same either way, and that is worth stating plainly so nobody re-opens the question expecting a saving. Option A would have given S_BUS four slaves – ROM, SRAM, AXI2APB, debug. The arrangement actually adopted also gives it four – ROM, ISRAM, DSRAM, AXI2APB. What option B avoids is a 4 KiB block of its own, not a port; what it buys is the separation of instructions from data, which has value independent of debugging.

Every region still has a distinct addr[31:28], even though the decoder compares more than that nibble: ROM is 0x0, ISRAM 0x2, DSRAM 0x3, the APB window 0x8, and the SYSDBG registers – which never appear on the bus at all – 0xF. Keeping the regions one nibble apart costs nothing and buys two things: an address is recognisable at a glance while reading a trace, and the coarse rule remains available to any block that only needs to know which region an address is in. SYSDBG uses exactly that rule internally to separate its own registers from everything it forwards to AXI (QNSC_SYSDBG_MAS, section 4.6).

Why 64 KiB of ISRAM and 32 KiB of DSRAM. 32 KiB of ISRAM was also on the table and was rejected; the reasoning is recorded here because the figure is a PARA_SRAM_DEPTH edit away from being changed again and whoever changes it should know what the number was bought with.

- It has to hold the firmware, not just its data. QSOC has no flash, so the application is loaded into RAM after every reset and executed from RAM. The load path is the serial bootloader in ROM – QSOC_HAS specifies a single frame of header, payload and CRC32 over UART0 at 115200 baud, which is 2.0 s for a 22 KiB image and 5.4 s for a full 60 KiB one. Those two figures are worth a note, because computing them from a nominal 115200 baud gives 5.3 s and makes them look wrong: the 20 MHz clock divides by 11, so the achieved rate is 113 636 baud, 1.4 % low, and 60 KiB at ten bits per byte is 5.4 s at that rate. The debug interface is the second load path, used when the ROM contents themselves are suspect, and it is what makes the edit-load-run loop of QNSC_SYSDBG_MAS section 4.13 possible without re-running synthesis. Either way the program arrives in ISRAM, which is why ISRAM is the one that has to be generous:

  - the block diagram carries both a ROM and an SRAM, which only makes sense if they hold different things – the ROM holds the bootloader, ISRAM holds what the bootloader receives;

  - on an FPGA a ROM is block RAM initialised by the bitstream, so changing its contents means re-running synthesis. That is acceptable for a bootloader that changes almost never, and unacceptable for an application under development, which is precisely why the application lives in RAM;

  - the debug program shares ISRAM with the application, taking its first 4 KiB, so the space the application can use is 60 KiB and not 64.

- With the split, ISRAM carries all code – the debug routine in its first 4 KiB and the main image after it – while DSRAM carries only the stack, the heap and the globals.

- The peripheral list is fixed, so the firmware can be costed rather than guessed at. QSOC carries sixteen APB slaves, but only eleven distinct drivers – GPIO0–GPIO3 share one, as do UART0/UART1 and TIMER0/TIMER1. Costing them gives a figure to compare the two candidate sizes against:

- Table 14 – Estimated ISRAM occupancy for QSOC’s peripheral set

| Item | RV32IMC, -Os |
| --- | --- |
| SCRC, SYSCSR, WDT – register writes and little else | ~1 KiB |
| GPIO – one driver, four instances | 0.5 KiB |
| TIMER – apb_timer_unit, 64-bit mode | 1 KiB |
| UART – ring buffer and interrupt handler | 1.5 KiB |
| SPI – host and device | 2.5 KiB |
| I2C – state machine | 2 KiB |
| PWM – apb_adv_timer | 1.5 KiB |
| DMA – idma descriptor setup | 1.5 KiB |
| INTMAP and the interrupt dispatch | 1 KiB |
| The eleven drivers together | ~12.5 KiB |
| crt0, startup, trap frame | 0.7 KiB |
| A hand-written printf – integer, hex, string | 2.5 KiB |
| A bring-up application exercising all sixteen slaves | 5–8 KiB |
| Total, hand-written printf | ~22 KiB |
| newlib-nano printf with float, in place of the hand-written one | 12–18 KiB |
| Total, newlib printf | ~34 KiB |

- A 32 KiB ISRAM leaves 28 KiB for the main image once the debug program has its 4 KiB. The modest case consumes 79% of that before a line of real test code is written, and the other case does not fit at all – and .rodata, 1–3 KiB of format strings and lookup tables, is still to be placed. 60 KiB accommodates both with room to grow.

- These are estimates and are labelled as such. The way to settle it properly is to measure: riscv32-unknown-elf-size -A on a representative image gives text and rodata for ISRAM, and data + bss – plus the stack and heap – for DSRAM.

- Block RAM is not the constraint on the boards this will run on. At 32 bits wide, 2 KiB of ROM is 2 M9K blocks, 32 KiB is 32 and 64 KiB is 64. A MAX 10 10M50 has 182:

|  | ROM | ISRAM | DSRAM | M9K used | of 182 |
| --- | --- | --- | --- | --- | --- |
| 32 KiB ISRAM | 2 | 32 | 32 | 66 | 36% |
| 64 KiB ISRAM | 2 | 64 | 32 | 98 | 54% |

- On an Artix-7 35T the same two come to 30% and 44% of 1,800 Kbit. Both fit both boards, and on a device of this class a SoC with a three-master crossbar and sixteen APB peripherals exhausts logic elements long before it exhausts block RAM. Block RAM is not the resource that decides this.

- The cost of being wrong is asymmetric, and that is what settles it. Choose 64 KiB and be wrong, and 32 M9K blocks sit unused – 17% of the device, with 43% still free. Nobody is blocked and nothing is rebuilt. Choose 32 KiB and be wrong, and it surfaces during integration, at the end of the project, once the firmware exists: PARA_SRAM_DEPTH, the interconnect’s address rule for that slave, synthesis and the regression all move together. The edit itself is cheap either way; when it is discovered is what costs.

ROM is 2 KiB, and the budget inside it is tight enough to state explicitly. The Day005 review of 2026-09-18 fixed ROM at 2 KiB and the program inside it at not more than 1 KiB; the ROM owner’s own specification matches, decoding 11 address bits for 512 words. This supersedes the 8 KiB that earlier revisions of this document carried from QSOC_HAS, and it supersedes the QSOC_HAS bootloader budget of 1.1 KiB with a bitwise CRC32 or 2.1 KiB with a table-driven one – the 2.1 KiB variant no longer fits at all.

What is left after the fixed cost:

| Item | Size | Note |
| --- | --- | --- |
| Trap vector table | 128 B | 0x000 – 0x07F, forced by Ibex, Table 15 |
| Reset entry onward | 1920 B | 0x080 – 0x7FF, all the code there is |
| Day005 budget for the program | <= 1024 B | leaves 896 B spare |

Two consequences for this document. The CRC32 must be the bitwise variant: a 256-entry table is 1 KiB of rodata on its own and would consume the whole budget before any code. And the payload copy loop writes into ISRAM one word at a time, which is still the only master that writes ISRAM before the application exists – the size change does not affect that path.

This is the ROM owner’s block, not this author’s. The figures are recorded here because the memory map, the vector table and the copy target are shared, and because a reader of this document should not be left with a stale 8 KiB.

This corrects an earlier revision of this document, which had the application arriving over JTAG and the ROM doing “very little: come out of reset, and wait”. That was wrong: the instructor’s direction and QSOC_HAS both put the boot flow in ROM, over UART0, with the debug interface as the second path. The conclusion this section reaches is unchanged – ISRAM still has to hold the whole program, so it still wants 60 KiB free – but the reason is that the bootloader puts the program there, not that a debugger does.

The reset vector does not sit where the map suggests, and this is easy to get wrong. Ibex is given boot_addr_i = 0x0000_0000, but it does not fetch its first instruction from there:

// ibex_if_stage.sv
PC_BOOT: fetch_addr_n = { boot_addr_i[31:8], 8'h80 };
`ASSERT(IbexBootAddrUnaligned, boot_addr_i[7:0] == 8'h00)

// ibex_cs_registers.sv -- mtvec.BASE must be 256-byte aligned
mtvec_d = csr_mtvec_init_i ? {boot_addr_i[31:8], 6'b0, ...}

So boot_addr_i must be 256-byte aligned, it becomes the reset value of mtvec, and execution begins at boot_addr_i + 0x80. The first 128 bytes of the ROM are therefore the trap vector table, not code:

Table 15 – ROM layout forced by the Ibex reset vector

| ROM offset | Contents |
| --- | --- |
| 0x000 – 0x07F | mtvec base – the trap vector table. In vectored mode this is 4 bytes per cause, and 32 causes is exactly 0x80 |
| 0x080 | The reset entry point. The first instruction the CPU ever executes |
| 0x084 onward | The rest of the boot code – the reset entry holds one 32-bit instruction, normally a jump to it |

Placing boot code at offset zero is the failure this note exists to prevent: the core would fetch from 0x080, find whatever is there, and take an illegal instruction trap before anything has been initialised – with no error from the bus to explain it.

## 5.2 Clock and reset domain

This section exists because QSOC_HAS carries an unresolved row that names this document, and because the answer is available from the bus protocol rather than from simulation.

Both RAMs are one domain. QSOC_HAS Table 4-2 assigns ISRAM and DSRAM to domain D13, sharing soft-reset bit 12 and one clock gate between them. Neither RAM has a reset or a gate of its own, and the ROM is a separate domain, D12, with bit 11. There is one clock frequency for the whole chip, so nothing here crosses domains.

Sharing one soft-reset bit has a consequence worth stating. A write that resets domain D13 resets both RAMs, which means it resets the memory holding the code that issued the write. Firmware executing from ISRAM cannot soft-reset the RAM domain and survive; the write is only safe from ROM. That is not a defect – there is no use for resetting the RAM a program is running from – but it is a loaded gun in the register map and the SYSCSR owner should know it points here. QSOC_HAS EN v1.1 puts the same trade-off to this author explicitly and keeps the domain shared; this document agrees, and records the escape hatch that makes the decision reversible: SOFT_RST_CTRL bits [15:14] are reserved, so splitting ISRAM and DSRAM into two domains later costs a bit that already exists rather than a change to the register layout.

The boot-memory ordering question, and the answer. QSOC_HAS records two readings of what happens if the memory domain is still in reset when the core fetches its first instruction, and leaves the row open pending simulation:

| Reading | Consequence |
| --- | --- |
| The controller does not accept the request | The bus handshake stalls, the core waits, and the situation corrects itself when the memory leaves reset |
| The fetch returns undefined data | The core traps before anything is initialised |

The second reading cannot happen, and the reason is in the AXI specification rather than in this controller. AXI requires every VALID signal to be deasserted while reset is asserted. A slave held in reset therefore cannot assert rvalid, and without rvalid there is no read data for the core to receive – undefined or otherwise. The core sees arready low, holds arvalid, and waits. So the ordering is not a correctness requirement, and the clock-and-reset specification is right that no sequencing is needed for it.

What it is instead is a liveness requirement, and that is worth more attention than the ordering was. Because AXI has no timeout, a core waiting on a memory that never leaves reset waits forever, with no exception, no error and nothing in any status register to say why. The only thing that can observe it is SYSDBG, which lives in its own clock and reset domain, D15, released before the CPU (QNSC_SYSDBG_MAS section 4.2), so it is still alive when the memory domain is not. It can halt the core and read arvalid state over JTAG while the rest of the chip is stuck.

But observing is now all it can do. The Day005 review of 2026-09-18 fixed the chip’s reset sources at three – power-on, watchdog and software – and removed debug reset, so SYSDBG can see a stuck reset and report it but cannot clear it. Recovery from this state is a power cycle. That makes the liveness requirement stronger than it was when this paragraph was first written: nothing in the chip rescues a memory domain that never leaves reset, so the clock and reset owner must guarantee it does.

## 5.3 Interfaces to agree with the team

Table 16 – Interfaces to agree with the team

| Item | Owner of the other side | Why it matters |
| --- | --- | --- |
| S_BUS AXI geometry: data width, address width, ID width | bus owner | Feeds PARA_DATA_WD, PARA_ADDR_WD and PARA_ID_WD |
| The memory map of section 5.1, and the ISRAM / DSRAM sizes | memory map owner, project level | Feeds PARA_SRAM_DEPTH on each macro and the interconnect’s address rule for each slave |
| A fourth slave port on S_BUS | bus owner | Section 5.1 – the RAM is now two instances: ISRAM on AXI_M1, DSRAM on AXI_M2, AXI2APB moving to AXI_M3 |
| The CPU2AXI wrapper must carry data_be_o through to wstrb | CPU / bus owner | Table 5 – an instruction fetch is always a full word, so the byte enables exist only on the data side and are easy to drop when the two are merged |
| Ibex boot_addr_i = 0x0000_0000 | CPU owner | The reset vector must land in the ROM |
| 0xF000_0000 left unmapped in the interconnect | bus owner | Table 10 – the SYSDBG registers exist in the JTAG command space only and must not be shadowed by a bus slave |
| Masters constrained to INCR and FIXED | bus owner, CPU owner, DMA owner | Section 4.6 – the WRAP path does not wrap |
| Transfer size fixed at 32 bits | bus owner | AxSIZE is not a port; a narrower request is ignored, not refused |
| DECERR for unmapped addresses returned by the bus decoder | bus owner | Section 4.7 – the RAM answers OKAY to everything |
| SRAM macro with byte write enables | technology / FPGA owner | Section 4.4 – required by the WSTRB path |
| Unused macro pins tied to their databook defaults – margin, retention, test | technology / physical design owner | Section 4.8. Accepted from QSOC_HAS. The controller drives none of them and no simulation can fail because of them, so this is silicon-only risk |
| Whether the same controller instance is reused for the ROM | project level | It is suitable unmodified; only the macro differs |

## 5.4 Open questions

- Closed: both confirmations have arrived. The ROM owner’s own specification confirms the bootloader is theirs and fixes ROM at 2 KiB with the program inside it at not more than 1 KiB, superseding the 1.1–2.1 KiB budget this item used to quote. The boot flow specification confirms boot_addr_i = 0x0000_0000 and that Ibex fetches from 0x0000_0080, citing ibex_if_stage.sv:243. Section 5.1 carries both numbers.

- 64 KiB of ISRAM remains the number to build against, for the reasons in section 5.1. Reducing it later is a change to PARA_SRAM_DEPTH and one address rule in the interconnect, with no RTL edit – and because the debug program sits at the bottom of ISRAM rather than the top, no CPU parameter moves with it.

- Claimed: the WSTRB addition is this author’s. It lives in the wrapper around the controller, not inside the IP, and the RAM wrapper is this block’s deliverable – so no handover is needed and the work can start. Section 4.4. It is the one piece of RAM RTL this project writes, and it should be agreed before it is written rather than discovered in integration.

- Closed: out of scope for v1, and the reason is a read-path change rather than a missing module. prim_ram_1p_scr presents a req/rvalid interface and adds read latency, whereas this controller assumes a fixed one-cycle read (reg_rd_pending). Adding scrambling and integrity would mean rebuilding the read path, not dropping a module in. Recorded as possible work for a later version, with the cost named so nobody mistakes it for an afternoon’s integration.

Two action items rather than open questions. Both are things to do, not information to wait for, and both go to the same person: nguyenquanicd/AXI4-SRAM-CONTROLLER is a repository of the project’s own mentor, the same account supplying APB-CSR-Generator, APB-BUS-Generator, FirstX2P and MRV-CPU to QSOC.

| Action | Why it matters |
| --- | --- |
| Ask the mentor what licence the IP carries. The repository has no LICENSE file, so the terms are undefined rather than permissive | Every block that instantiates it inherits the answer – ROM as well as RAM |
| Report the WRAP finding, with the reference in Appendix C | WRAP is declared and not implemented, section 4.6. The fix reaches its author directly and benefits ROM, which uses the same IP |

# 6. Observations

- Choosing an IP by bus protocol first was right, and reading its port list second was what made the choice safe. AXI4-SRAM-CONTROLLER speaks AXI4 natively, which is why it was selected; but the 33-port list is what revealed that WSTRB and AxSIZE are absent, and those two absences shape the rest of this specification. A feature table would not have shown it – the README’s own compliance table marks byte strobes as unsupported in one row and is easy to read past.

- A self-checking regression is only as correct as its reference model. Section 4.6 is the clearest lesson in this document: 46 tests pass, four of them exercise WRAP, and the address calculation is wrong in the design and wrong in the same way in the checker. Reading the model, not just the pass rate, is what found it.

- The one gap is also the one contribution. Everything this block does, it does already; the byte-enable path is the whole of what QSOC adds, and it is small. That is what a good integration looks like, and it is worth stating plainly rather than inflating.

- This block and SYSDBG are specified from opposite directions, deliberately. The RAM is an integration: a proven controller, where the work is choosing it correctly, parameterising it, and adding the one path it does not have. Both, in the end, came down to the same activity – reading somebody else’s RTL closely enough to know exactly where it stops.

# Appendix A. Acronyms

Table 17 – Acronyms

| Acronym | Description |
| --- | --- |
| APB | Advanced Peripheral Bus |
| AXI | Advanced eXtensible Interface |
| FIFO | First In First Out buffer |
| FSM | Finite State Machine |
| IP | Intellectual Property block |
| QSOC | The MCU built in this training project |
| RTL | Register Transfer Level |
| SECDED | Single Error Correct, Double Error Detect |
| SRAM | Static Random Access Memory |
| VIP | Verification Intellectual Property, a pre-built verification component |
| WSTRB | AXI write strobe, one byte-enable bit per data byte |

# Appendix B. First Review

| Item | Reviewer | Response |
| --- | --- | --- |

# Appendix C. References

- VLSI Technology (nguyenquanicd), AXI4 SRAM Controller, https://github.com/nguyenquanicd/AXI4-SRAM-CONTROLLER

- ARM, AMBA AXI and ACE Protocol Specification, IHI 0022

- lowRISC, OpenTitan, primitives prim_ram_1p_scr, prim_secded_inv_39_32, https://github.com/lowRISC/opentitan

- QSOC block diagram and IP assignment sheet, VLSIT_DeepTraining_20260908

- QSOC_HAS – the system architecture specification. Authority for the boot flow of section 5.1, the memory map, and the mandatory protocol conversions

- QNSC_SYSDBG_MAS – the companion specification for the author’s second block